| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0055-v1 |
| --- | --- |

| Title: | CE9-1.2 and CE9-1.4: BDOF early termination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan Kyohei Unno, Kei Kawamura, Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan Takeshi Chujoh, Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +886-3-5670766 {jenny.lai, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com +81-50-3750-2744 {kenji.kondo, Masaru.Ikeda, teruhiko.s}@sony.com +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com +81-07-1617-3470 {chujoh.takeshi, ikai.tomohiro} @sharp.co.jp |
| Source: | MediaTek Inc., Sony Corporation, KDDI Corp., Sharp Corporation |  |  |

_____________________________

# Abstract

In this contribution, CE9-1.2 and CE9-1.4 tests, which are both built on top of CE9-1.1 in JVET-O0054, are reported. In CE9-1.1, when decoder motion vector refinement (DMVR) is applied, the subblock early termination in bidirectional optical flow (BDOF) reuses the sum of absolute differences (SAD) in DMVR and the 4x4 block early termination in BDOF is removed. In order to unify and simplify the processing flow of BDOF, the subblock early termination and 4x4 block early termination in BDOF are totally removed even when DMVR is not applied. In this way, SAD operations in BDOF are totally removed regardless of applying DMVR or not. The difference between CE9-1.2 and CE9-1.4 is the threshold value of SAD calculated in DMVR for the early termination of BDOF when DMVR is applied. The threshold of CE9-1.2 is same as that in CE9-1.1. The threshold of CE9-1.4 is doubled. Simulation results reportedly show that the luma BD-rate is -0.02% for RA without decoding time increase for CE9-1.2, and the luma BD-rate is 0.01% for RA with 2% decoding time reduction for CE9-1.4 when the anchor is VTM5.0.

# Introduction

In VTM5.0, DMVR is adopted to refine motion vectors (MVs) at the decoder for a coding unit (CU) when the CU is coded with regular merge/skip mode and bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order, the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture, and the bi-prediction with CU weights (BCW) selects equal weights. When DMVR is applied, one luma coding block (CB) is divided into several independently processed subblocks of size min(cbWidth,16)×min(cbHeight,16). DMVR refines MVs of each subblock by minimizing the SAD between 1/2-subsampled 10-bit L0 and L1 prediction samples generated by bilinear interpolation. For each subblock, integer △MV search around the initial MVs (i.e., the MVs of the selected regular merge/skip candidate) is first performed using SAD, and then fractional △MV derivation is performed to obtain the final MVs.

BDOF refines the luma prediction samples for a CU when the CU is coded with bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order, and BCW selects equal weights. The eight-tap interpolation is used for generating the initial L0 and L1 prediction samples according to the input MVs (e.g., final MVs of DMVR in case of enabling DMVR). Next, a two-level early termination process is performed. The first early termination is at subblock level, and the second early termination is at 4×4 block level and is checked when the first early termination does not occur. At each level, the SAD between full-sampled 14-bit L0 and L1 prediction samples in each subblock/4x4 block is calculated first. If the SAD is smaller than one threshold, BDOF is not applied to the subblock/4x4 block. Otherwise, BDOF parameters are derived and used to generate the final luma sample predictors for each 4x4 block. In BDOF, the subblock size is the same as that in DMVR, i.e., min(cbWidth,16)×min(cbHeight,16).

When the CU is coded with regular merge/skip mode, one reference picture is before the current picture and the other reference picture is after the current picture in display order, the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture, and BCW selects equal weights, DMVR and BDOF are both applied. The flow of cascading DMVR and BDOF processes is shown in Fig. 1. This is the critical path of inter prediction in a hardware decoder of VTM5.0.

Figure 1.  The flow of cascading DMVR and BDOF processes in VTM5.0. The DMVR SAD operations and BDOF SAD operations are different and not shared.

# Proposed method

As shown in Fig. 1, the SAD operations are required in DMVR and BDOF processes. In CE9-1.1, in order to reduce the latency and operations in this critical path, when DMVR and BDOF are both applied, it is proposed to reuse the subblock SAD calculated in DMVR for the subblock early termination in BDOF, and it is also proposed to remove the 4x4 block early termination in BDOF. The threshold per sample in the subblock early termination is set to the threshold per sample in 4x4 block early termination divided by 32, where 32 is derived based on the prediction sample bitdepth difference and subsampling ratio difference in SAD operations between DMVR and BDOF.

In this contribution, we further remove the subblock early termination and 4x4 block early termination in BDOF when DMVR is not applied. Therefore, SAD operations in BDOF are totally removed regardless of applying DMVR or not.

The threshold of SAD calculated in DMVR for subblock early termination of BDOF when DMVR is applied is described as follows.

- In CE9-1.2

Threshold = 8 * dx * (dy >> 1)

Here, dx indicates the subblock width, and dy indicates subblock height.

This threshold is the same as that in VTM5.0 and CE9-1.1 as follows.

  - Threshold = ((dx >> 2) * (dy >> 2) * bdofBlkDiffThres) >> 5

  - Here, bdofBlkDiffThres =  1 << ( BitDepthY − 3 + shift ), and shift = Max( 2, 14 − BitDepthY ).

- In CE9-1.4

Threshold = 16 * dx * (dy >> 1)

This threshold is equal to ((dx >> 2) * (dy >> 2) * bdofBlkDiffThres) >> 4.

# Experimental results

The proposed method has been implemented on VTM5.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [2]. Table 1 shows the simulation results of CE9-1.2. Table 2 shows the simulation results of CE9-1.4.

Table 1.  Simulation results of CE9-1.2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.17% | -0.02% | 100% | 100% |
| Class A2 | -0.04% | 0.07% | 0.04% | 100% | 99% |
| Class B | 0.00% | 0.03% | -0.01% | 100% | 100% |
| Class C | -0.02% | 0.05% | -0.08% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.02% | 100% | 100% |
| Class D | -0.04% | -0.10% | -0.02% | 99% | 99% |
| Class F | -0.01% | 0.00% | 0.01% | 100% | 97% |

Table 2.  Simulation results of CE9-1.4

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.26% | 0.01% | 100% | 98% |
| Class A2 | 0.02% | 0.00% | -0.02% | 100% | 98% |
| Class B | 0.02% | 0.00% | -0.02% | 100% | 97% |
| Class C | 0.01% | -0.01% | -0.07% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.05% | -0.03% | 100% | 98% |
| Class D | -0.01% | 0.00% | 0.05% | 100% | 98% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 96% |

# Conclusions

Based on CE9-1.1, this contribution proposes to further remove subblock early termination and 4x4 block early termination in BDOF when the DMVR is not applied. No SAD operations are required in BDOF. Simulation results show almost no impact on BD-rate and 0-2% decoding time reduction.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

KDDI Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).