| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0096-v1 |
| --- | --- |

| Title: | CE9-1.5: Best DMVR cost based early termination for BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Tel: Email: | +91-8066601077 sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

In this contribution, the details of CE9-1.5 test are summarized and the results are reported. Two tests were performed under this core experiment. In both tests, 4x4 level early termination check is disabled for BDOF. In addition, for sub-block in CUs for which DMVR is not applied and only BDOF is applied, the sub-block level early termination procedure is identical to that in VTM5.0. However, in CUs where both DMVR and BDOF are applied, the two tests performed, namely CE9-1.5a and CE9-1.5b differ slightly. In CE9-1.5a, the best integer distance SAD cost computed for a given sub-block during DMVR is used to perform the early termination check for BDOF against a threshold that is identical to that used in VTM5.0. In CE9-1.5b, a derived cost is computed corresponding to the refined MV whenever parametric error surface based refinement to the integer displacement is obtained for the delta motion vector and is used to perform the early termination check. When parametric error surface based refinement is not applicable, then the integer distance SAD cost computed during DMVR for that sub-block is used to perform the early termination check. The luma BD-rate for CE9-1.5a against VTM5.0 anchor is -0.01% for RA, while that of CE9-1.5b against VTM5.0 anchor is -0.02% for RA. Both sub-tests result in negligible change to the average encoding or decoding times.

# Introduction

The currently adopted BDOF employs early termination checks at the sub-block level and 4x4 block level. These early terminations are intended primarily to reduce average encoding or decoding times. Whenever both DMVR and BDOF are applied to a CU, it is possible to use the best DMVR cost to perform early termination for BDOF. The 4x4 level early termination for BDOF poses additional hardware requirements for accumulating the SADs at 4x4 level. In JVET-N0297, the re-use of DMVR cost corresponding to the DMVR refined MV was proposed along with complete elimination of early terminations for the case of BDOF when DMVR is not applied, motivated by the fact that non-DMVR BDOF CUs constituted only a small percentage in CTC conditions. However, during the last JVET meeting, it was concluded that average decoding time reduction for BDOF for software implementations is important and should not depend only on other tools. All proposals related to this idea are currently being studied in CE9-1. In this contribution, CE9-1.5 related aspects are summarized and the results are presented.

# Proposed method

This test uses the lowest SAD cost computed or derived during DMVR at a sub-block level to disable BDOF for that sub-block, whenever both DMVR and BDOF are applied to the CU to which the sub-block belongs. Whenever DMVR is not applied, but BDOF is applied, the sub-block level early termination check present in VTM5 will be used. The test removes the second level early termination for BDOF at the 4x4 level for all cases. The threshold value used shall be the same threshold that is used at the sub-block level in VTM-5.0.

The main purpose of the test is to enable clock-gating for BDOF in hardware using information available from the DMVR stage to achieve a lower average power consumption. In software implementations, it is aimed at preserving a large portion of the average decoding time reduction offered by the early termination checks employed in VTM5.

There are two sub-tests proposed:

- CE9-1.5a: Using only the best SAD cost amongst the integer distance DMVR refinement positions to compare against a threshold, and disabling BDOF if the value is less than or equal to the threshold.

- CE9-1.5b: Using the derived SAD cost based on the parametric error surface function, when parametric function based refinement is present. Otherwise, the best SAD cost amongst the integer distance DMVR refinement positions is used.

The derived SAD cost in CE9-1.5b is computed as described in JVET-N0296 as follows:

DerivedCost=SAD0,0- x0*SAD-1,0-SAD1,04- y0*(SAD0,-1-SAD0,1)4

where, the position (0,0), (-1,0), (1,0), (0,-1) and (0,1) are relative to the best integer distance displacement after DMVR in L0 reference and x0 and y0 are the sub-pixel accurate displacements relative to the integer distance displacement that is computed using the parametric error surface equation.

In both tests, BDOF is skipped when the DMVR-based cost is less than a pre-determined threshold that is computed as a function of the fixed bit-depth (of 10) used by DMVR and the number of samples used in the integer distance cost computation.

The details of the two tests are summarized below in Table-1.

|  | When DMVR is enabled, cost that is used for BDOF early termination at sub-block level | BDOF sub-block level early termination Threshold | when DMVR is disabled, the type of Early Termination used for BDOF |
| --- | --- | --- | --- |
| CE9-1.5a | Integer-distance SAD | 4 per sample used to compute integer distance SAD | Subblock level |
| CE9-1.5b | SAD derived from 5 integer distance SAD values, when parametric error surface is applicable; Integer-distance SAD, otherwise | 4 per sample used to compute integer distance SAD | Subblock level |

Table-1: Summary of the two sub-tests under CE9-1.5

# Experimental results

The proposed method has been implemented on VTM5.0 [1]. The simulations were performed following the JVET common test conditions (CTC) in the random access (RA) configuration. Table 2 and 3 show the simulation results of the proposed sub-tests.

Table 2: Results of CE9-1.5a

Table 3: Results of CE9-1.5b

From the results, it can be seen that use of derived cost in CE9-1.5b offers a slightly better BDRATE and also a slightly reduced encoding and decoding time ratio than the integer distance cost used in CE9-1.5a.

# Conclusions

This contribution proposes to reuse the SAD calculated in DMVR for early termination in BDOF, in order to eliminate the need to compute the sub-block level SAD for BDOF early termination check from the DCTIF based interpolation. Simulation results show that a luma BD-rate gain of -0.02% for RA with negligible change to encoding/decoding time is possible while eliminating the 4x4 level early termination for BDOF completely.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- S. Sethuraman, CE9-related: DMVR costs based early termination for BDOF in CE9-2.6, JVET-N0296, Geneva, 2019.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).