| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 Jul. 2019 | Document: JVET-O0097-v2 |
| --- | --- |

| Title: | CE8-3.1: Enable Transform Skip in CUs using ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Tung Nguyen, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany Tsung-Chuan Ma, Xiao-yu Xiu, Yi-Wen Chen, Xianglin Wang Kwai Inc. 8910 University Center Line, Suite 400, San Diego, CA, 92122 | Tel: Email: | +49 30 31002-511, +1 (310) 567-2937 santiago.de.luxan@hhi.fraunhofer.de tsung-chuanma@kwai.com |
| Source: | Fraunhofer HHI Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution enables the transform skip mode on CUs using ISP at the TU level. The experimental results are the following:

- Normal QP range: BD-Rate of 0.00% (CTC), 0.02% (class F) and -0.02% (TGM) for the AI configuration. The corresponding encoder run-times are 100% (CTC), 101% (class F) and 102% (TGM). In the case of RA, the gains are -0.02% (CTC), -0.01% (class F) and -0.04% (TGM) and the associated encoder run-times are 100% (CTC), 100% (class F) and 100% (TGM).

- Low QP range: BD-Rate of 0.00% (CTC), -0.02% (class F) and -0.08% (TGM) for the AI configuration. The corresponding encoder run-times are 101% (CTC), 101% (class F) and 102% (TGM). In the case of RA, the gains are 0.00% (CTC), -0.07% (class F) and the associated encoder run-times are 100% (CTC) and 100% (class F). Results are not shown for TGM sequences using the RA configuration due to the problem of employing the BD-Rate formula at these low rates.

# Introduction

In the current design of the Intra Sub-Partitions (ISP) coding mode in VTM-5.0 Transform Skip (TS) is disabled in all CUs using ISP. The scope of this contribution consists in simplifying the design by removing this restriction. Specifically, TS is enabled at the TU level. This means that there is a TS flag for each TU with a non-zero CBF within the CU using ISP. If the TS flag is 1, then that particular TU will use TS. Otherwise it will use the normal implicit transform of ISP.

The context used to signal the TS TU level flag is the same as the one used by non-ISP blocks.

# Experimental results

According to the common test conditions [2], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with the VTM-5.0 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler. SCC TGM sequences have also been simulated.

## QP Range (22-37)

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,01% | 0,01% | 99% | 99% |
| Class A2 | 0,00% | -0,03% | -0,02% | 99% | 100% |
| Class B | 0,00% | -0,01% | 0,00% | 100% | 100% |
| Class C | -0,01% | 0,02% | 0,07% | 100% | 101% |
| Class E | 0,01% | 0,03% | 0,05% | 101% | 101% |
| Overall | 0,00% | 0,00% | 0,02% | 100% | 100% |
| Class D | 0,00% | 0,07% | 0,03% | 100% | 101% |
| Class F | 0,02% | 0,05% | -0,05% | 101% | 99% |
| Class TGM | -0,02% | 0,02% | 0,03% | 102% | 102% |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,07% | 0,05% | 100% | 101% |
| Class A2 | -0,01% | 0,05% | -0,02% | 100% | 99% |
| Class B | 0,00% | 0,03% | -0,02% | 100% | 100% |
| Class C | -0,06% | -0,22% | -0,18% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | -0,05% | -0,05% | 100% | 100% |
| Class D | 0,02% | -0,16% | 0,05% | 100% | 100% |
| Class F | -0,01% | 0,12% | 0,10% | 100% | 101% |
| Class TGM | -0,04% | 0,01% | -0,09% | 100% | 100% |

## QP range (2-17)

Table III: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,02% | 0,01% | 100% | 99% |
| Class A2 | 0,00% | 0,00% | 0,00% | 101% | 99% |
| Class B | 0,00% | -0,01% | 0,01% | 101% | 100% |
| Class C | -0,01% | 0,00% | 0,03% | 100% | 103% |
| Class E | 0,00% | 0,01% | -0,02% | 101% | 99% |
| Overall | 0,00% | 0,00% | 0,01% | 101% | 100% |
| Class D | -0,01% | -0,02% | -0,02% | 100% | 99% |
| Class F | -0,02% | -0,04% | -0,06% | 101% | 103% |
| Class TGM | -0.08% | -0.03% | -0.02% | 102% | 103% |

Table IV: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.01% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0,00% | -0,02% | 0,01% | 101% | 101% |
| Class C | -0,01% | -0,03% | 0,01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | 0.01% | 100% | 101% |
| Class D | -0,02% | -0,01% | -0,03% | 100% | 101% |
| Class F | -0,07% | 0,00% | -0,04% | 100% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Note that results are not shown for the TGM class in the RA configuration in the low QP case, given that at very low rates the Rate-PSNR behavior of some sequences can be different from the usual pattern. For example, in the VTM-5.0 anchor with low QPs, the sequence Desktop shows a higher PSNR for QP 7 than for QP 2, as shown in  Table 5. Therefore, the BD-Rate formula will not work appropriately in this case and the results will not be reliable.

Table 5: VTM-5.0 rate and PSNR values for the sequence Desktop using a RA configuration

| Desktop | 2 | 5419.94 | 65.30 | 66.98 | 67.03 | 15803.87 | 5.57 | 4.39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7 | 4346.44 | 68.19 | 62.25 | 62.29 | 14722.66 | 5.74 | 4.09 |
|  | 12 | 3439.70 | 60.67 | 57.80 | 57.92 | 15033.62 | 5.80 | 4.18 |
|  | 17 | 2763.09 | 57.14 | 53.19 | 53.39 | 14533.54 | 6.46 | 4.04 |

# Draft changes

if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) {

if( transform_skip_enabled_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize )

transform_skip_flag[ x0 ][ y0 ]

if( (( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( !transform_skip_flag[ x0 ][ y0 ] ) )

tu_mts_idx[ x0 ][ y0 ]

}

# Conclusion

It is observed that the proposed simplification does not have a significant negative impact in either CTC or SCC sequences.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).