| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0099-v2 |
| --- | --- |

| Title: | CE3-1.3: CU-level low latency intra sub-partitions for 4x8 and 8x4 CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hua Yang, Yuwen He, Rahul Vanam 4 Research Way 3rd Floor, Princeton, NJ 08540 USA | Tel: Email: | +1-609-608-0479 Hua.Yang@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Intra sub-partitions (ISP) predicts successive sub-partitions using reconstructed samples from its previous adjoining sub-partitions. Different proposals on ISP sub-partition control/restrictions are studied in CE3-1. This proposal presents result for CE3-1.3. In this proposal, a CU-level low latency ISP scheme is presented that for 4x8 and 8x4 CUs, intra-prediction is conducted once for the whole CU, and thus, transform coding of the two sub-partitions can be conducted in parallel to reduce decoding pipeline latency as compared to the original ISP approach with sequential coding of the sub-partitions. In addition, 1xN, 2xN sub-partitions are disabled to favor the hardware (HW) implementation preference of at least 4 samples in widths of coding blocks for better memory access throughput efficiency. In CTC testing, compared to VTM-5.0, the proposed CE3-1.3 low latency ISP scheme overall yields 0.08%/0.04%/0.06% luma BD-rate loss for AI/RA/LDB, with similar decoding and encoding time. Compared to the change of disabling 1xN and 2xN sub-partitions alone (i.e. the CE3-1.2.1 test), our additional change of applying CU-level intra-prediction for 4x8, 8x4 CUs yields 0.00%/0.01%/0.04% extra luma BD-rate loss for AI/RA/LDB, with similar decoding and encoding time.

# Introduction

The intra sub-partitions (ISP) is an intra coding tool that was adopted in VVC during the 13th JVET meeting.  ISP splits a luma intra-predicted block vertically or horizontally into two or four sub-partitions depending upon the block size.  Block size of 4×8 and 8×4 are always split into two sub-partitions, while larger blocks are split into four sub-partitions. The residual is added to the prediction samples of the first sub-partition to yield reconstructed samples, which are then used to predict the next sub-partition. The process of prediction and reconstruction is repeated on successive sub-partitions. Figure 1(a) illustrates the prediction of a sub-partition ‘B’ using bottom row samples from reconstructed sub-partition ‘A’. Since a sub-partition cannot be predicted until its previous sub-partition has been reconstructed, it introduces latency for processing successive sub-partitions.  This proposal presents a scheme to reduce this latency in ISP.

In addition, in the original ISP, 1xN, 2xN sub-partitions are used for vertical splits of 4xN (N≥8), 8xN (N≥8), as illustrated in Figure 2. Since it is not preferable to perform less than 4 samples per row operations from the HW throughput efficiency point of view, it is more HW friendly and efficient to disable such 1xN, 2xN sub-partitions in ISP.

- (b)

Figure 1. ISP sub-partition prediction schemes. (a) The original ISP. (b) The proposed low latency ISP.

1

4x8

4xN

(N≥16)

8x4

8xN

(N≥8)

V:

H:

4x4

No ISP

4

4x8N8x44

(N≥16)

N

4x8N8x44

(N≥16)

N/4 = 4

(min)

8

2

4x8N8x44

(N≥16)

4x8N8x44

(N≥16)

8

N/4 = 2

(min)

2

8

N

2

Figure 2. The original ISP sub-partitions in VTM5.0. H: horizontal split, V: vertical split.

# Proposed Low Latency ISP

In this proposal, to reduce decoding pipeline latency, for 4x8 and 8x4 CUs, intra-prediction is conducted once for the entire CU, and thus, the two sub-partition TUs can be coded in parallel rather than sequentially as in the original ISP approach. This is illustrated in Figure 1 (b).

4x8

4xN

(N≥16)

8x4

8xN

(N≥8)

V:

H:

4x4

No ISP

No ISP

No ISP

4

4x8N8x44

(N≥16)

4

(N≥16)

N

4x8N8x44

(N≥16)

8

2

4x8N8x44

(N≥16)

4x8N8x44

(N≥16)

8

PU

PU

N/4 = 4

(min)

N/4 = 2

(min)

Figure 3. The proposed CU-level low latency ISP changes include: (1) 1xN, 2xN sub-partition removal (highlighted in yellow), (2) CU-level intra-prediction for 4x8, 8x4 CUs (highlighted in green).

In addition, 1xN, 2xN sub-partitions are disabled for better HW efficiency. The overall changes of the proposed ISP approach are illustrated in Figure 3.

# Experimental results

Using VTM-5.0 as the anchor, CTC testing results comparing the proposed CE3-1.3 approach of both CU-level intra-prediction for 4x8, 8x4 CUs and 1xN, 2xN sub-partition removal to the original ISP approach as in VTM-5.0 anchor are provided below in Table 1, for AI, RA, and LDB, respectively [2].

Table 1. Simulation results of the proposed low latency ISP approach CE3-1.3 vs original ISP in VTM-5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.02% | 100% | 100% |
| Class A2 | 0.01% | 0.00% | -0.05% | 100% | 99% |
| Class B | 0.04% | 0.00% | 0.07% | 100% | 100% |
| Class C | 0.12% | 0.09% | 0.18% | 100% | 98% |
| Class E | 0.23% | 0.08% | 0.14% | 100% | 99% |
| Overall | 0.08% | 0.03% | 0.07% | 100% | 99% |
| Class D | 0.18% | 0.30% | 0.06% | 100% | 99% |
| Class F | 0.15% | 0.24% | 0.16% | 101% | 99% |
| Class TGM | 0.07% | 0.07% | 0.03% | 99% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.10% | 0.16% | 101% | 100% |
| Class A2 | 0.05% | 0.03% | -0.01% | 100% | 100% |
| Class B | 0.05% | 0.01% | -0.08% | 101% | 100% |
| Class C | 0.03% | 0.05% | -0.18% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.00% | -0.04% | 101% | 100% |
| Class D | 0.09% | -0.07% | -0.02% | 101% | 100% |
| Class F | 0.11% | 0.15% | 0.18% | 100% | 100% |
| Class TGM | -0.05% | -0.09% | -0.06% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.28% | -0.24% | 100% | 99% |
| Class C | 0.01% | -0.14% | 0.11% | 101% | 99% |
| Class E | 0.20% | -1.20% | -0.21% | 100% | 101% |
| Overall | 0.06% | -0.23% | -0.12% | 100% | 100% |
| Class D | 0.03% | -0.44% | 0.61% | 101% | 98% |
| Class F | 0.18% | 0.28% | -0.26% | 100% | 100% |
| Class TGM | -0.04% | -0.11% | -0.07% | 99% | 99% |

For comparison, the simulation results of comparing the proposed CE3-1.3 scheme to only disabling 1xN, 2xN sub-partitions (which is actually the CE3-1.2.1 test in this meeting [1]) are presented below in Table 2. (Herein, we have checked our CE3-1.2.1 result with the official submitted result of CE3-1.2.1 and confirmed that they are the same. Note that official CE3-1 does not require Class TGM and LDB results, which are additionally provided herein for better clarity. Also, for confirmation check with official CE3-1.2.1, we do not compare the encoding and decoding time differences as the two tests were run independently under separate testing environments.)

Table 2. Simulation results of the proposed CE3-1.3 vs 1xN, 2xN sub-partition removal only (i.e. CE3-1.2.1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE3-1.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.00% | 100% | 99% |
| Class A2 | -0.01% | 0.10% | 0.00% | 100% | 100% |
| Class B | 0.00% | -0.01% | -0.04% | 100% | 100% |
| Class C | 0.01% | 0.04% | 0.13% | 100% | 99% |
| Class E | 0.02% | -0.11% | -0.06% | 100% | 101% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.04% | 0.03% | -0.09% | 100% | 99% |
| Class F | 0.02% | 0.15% | 0.04% | 100% | 99% |
| Class TGM | -0.02% | 0.01% | -0.01% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE3-1.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.24% | 0.13% | 100% | 100% |
| Class A2 | 0.00% | 0.03% | -0.06% | 100% | 100% |
| Class B | 0.00% | 0.04% | 0.12% | 100% | 100% |
| Class C | 0.04% | 0.23% | -0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.13% | 0.04% | 100% | 100% |
| Class D | 0.02% | -0.05% | -0.11% | 100% | 99% |
| Class F | 0.04% | 0.12% | 0.04% | 100% | 100% |
| Class TGM | -0.10% | -0.09% | -0.04% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE3-1.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.01% | 0.07% | 100% | 99% |
| Class C | -0.01% | -0.28% | 0.05% | 100% | 99% |
| Class E | 0.16% | -0.86% | -0.45% | 100% | 100% |
| Overall | 0.04% | -0.31% | -0.07% | 100% | 100% |
| Class D | 0.04% | -0.87% | -0.52% | 101% | 98% |
| Class F | 0.23% | 0.23% | 0.49% | 100% | 100% |
| Class TGM | -0.03% | -0.03% | -0.08% | 100% | 99% |

# Conclusion

To reduce decoding pipeline latency, we propose to apply CU-level intra-prediction for 4x8, 8x4 CUs in ISP. In addition, to be more HW implementation friendly and efficient, 1xN, 2xN sub-partitions are removed as well from ISP. Comparing to VTM5.0 in CTC, the proposed approach of CE3-1.3 yields 0.08%/0.04%/0.06% luma BD-rate loss (AI/RA/LDB) with similar encoding and decoding time, while comparing to the change of removing 1xN, 2xN sub-partitions alone (i.e. CE3-1.2.1), our additional change of CU-level intra-prediction for 4x8, 8x4 CUs yields 0.00%/0.01%/0.04% extra luma BD-rate loss for AI/RA/LDB, with similar encoding and decoding time.

# References

- G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-N1023, 14th meeting, Mar. 2019, Geneva, CH.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th meeting, Mar. 2019, Geneva, CH.

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).