| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3-12 July 2019 | Document: JVET-O0100 |
| --- | --- |

| Title: | CE11: Gradual Random Access (GRA) using encoder and normative restrictions (Tests CE11-2.1.a, CE11-2.1.b and CE11-2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | David Gommelet, Jean-Marc Thiesse, Didier Nicholson | Tel: Email: | +33 1 46 73 06 06 david.gommelet@vitec.com jean-marc.thiesse@vitec.com didier.nicholson@vitec.com |
| Source: | VITEC 99 rue Pierre Sémard 92320 Chatillon, France |  |  |

_____________________________

# Abstract

This contribution presents the results of CE11 tests 2.1a, 2.1.b and 2.2 on gradual random access. In CE11-2.1.a, the Gradual Random Access (GRA) method applies motion vector (MV) restriction at the encoder and uses the Constrained Intra Prediction (CIP) tool implemented in the VTM 5.0 as well as virtual boundaries. Test 2.1.b adds CIP restriction to only refreshed regions on top of test 2.1.a. Test 2.2 introduced a straightforward signalization of GRA which enabled tools as in test 2.1.b as well as avoiding parsing of derivable flags. The implementation is available on the CE11 software repository https://vcgit.hhi.fraunhofer.de/jvet-n-ce11/VVCSoftware_VTM/branches

The following simulation results report BD-rate on Y, Cb and Cr components as well as encoding/decoding time over VTM5.0 using test conditions defined in CE11.

- Test 2.1.a: 	9.54%, 11.28%, and 16.61%, with   96% and 114% Enc/Dec Time

- Test 2.1.b: 	7.96%,   8.66%, and 13.02%, with 101% and 115% Enc/Dec Time

- Test 2.2: 	%, %, and %, with % and % Enc/Dec Time

# Introduction

GRA or Intra Refresh (IR, also Progressive IR (PIR)) is a technique used to reduce encoder-decoder end-to-end delay. The key idea is to limit the large bitrate variations between I frames and B frames while maintaining the same random access period. Indeed, intra picture bits sizes are significantly higher than inter ones, and therefore introduce a transmission time overhead.

A first proposal for normative handling of GRA was presented at Ljubljana [1] and lead to the creation of the AHG14 which aims at studying the opportunity of integrating such a normative solution. At Macao, [2] provides a great overview of the issues to tackle to have an efficient support of intra refresh within VVC. An encoder-only implementation of intra refresh has been proposed in [3-4] and an implementation of the proposed intra refresh support has been developed in [6]. The results were updated in VTM4.1 for Geneva meeting [5].

Growing interest in GRA has led to the creation of a Core Experiment at Geneva meeting: CE11 [6]. Specific test conditions have been discussed and agreed on by all proponents. This CE main objective is to compare the various GRA methods listed in 4 categories: encoder-only restriction, virtual boundary based, coded region based, and distributed hidden-picture. This contribution is focused on virtual boundary based methods.

# Proposed Solutions

The implementation is available on the CE11 software repository https://vcgit.hhi.fraunhofer.de/jvet-n-ce11/VVCSoftware_VTM/branches

## CE11-2.1.a: motion vector restriction and virtual boundaries.

The solution studied in this proposal consists in periodically forcing a group of pixels (column, line, etc.) to intra prediction while using only P or B pictures. The position of these groups of pixels moves over during the encoding process while producing a clean/refreshed area in the frame, as illustrated in the following figure.

Figure 1: Intra Refresh principle (column based)

The clean region is encoded such that it can be decoded only from clean regions of the past frame. In this way, a B picture can be random-accessed and progressively reconstructed. After a full refresh cycle, the whole picture is clean or refreshed and can be displayed and a second refresh cycle can begin. In this new refresh cycle the clean region becomes the dirty one and a new clean area is progressively reconstructed.

This solution is achieved by a combination of encoder-only changes exploiting minor normative changes as defined at Geneva meeting. It is activated in the implementation with the macro JVET_CE11_21A which activates several other macros. 
The encoder-only modifications, activated through the macro JVET_IR_ENC_ONLY, include:

- The forcing of Intra CU according to the refreshed pattern. In this test, the refresh pattern is a column, as illustrated in Figure 1. The granularity of the column is aligned on 8x8 CU basis (macro JVET_IR_SIZE_MULTIPLE) using two different column sizes over a cycle allows to suit any image size or refresh cycle. The two width of the intra column w1 and w2 are computed using the width of the video sequence and the refresh cycle period:

w1=ceilwidthperiod*8*8	and		w2=w1-8

Inside the cycle, the number of frames Nw1 using w1 and the number of frames Nw2 using w2 are computed with:

Nw1=width-period*w18 	and 	Nw2=period-Nw1

As an example, for a 416x240 video sequence, a refresh cycle period of 50 would not be achievable with only one column size. Using the proposed solution:  w1=16, w2=8, Nw1=2  and Nw2=48, meaning that the refresh cycle use 2 columns with a width of 16 pixels and 48 columns with a width of 8 pixels.

- Motion vectors constraints for the clean region such that its prediction does not use any pixels coming from the dirty region.

- Virtual boundary for disabling the loop-filters [7] across the clean and dirty region to ensure that dirty pixels are not propagated into the clean region. Another solution consists in keeping the loop-filters and to further constrain the motions vectors within the clean area however this solution has a noticeable impact on RD performances, especially for static videos. 
Note that the virtual boundaries [7] were not present in the tagged version of VTM 5.0. We used the following version of VTM5.0 as agreed on JVET CE11 mailing list:
https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/597d5b7f36b8e3495493d2c29ae82aed7a903ab8

- Removing of former reference pictures when starting a new refresh cycle.

The additional normative change consists in enabling Constrained Intra Prediction (CIP) at slice level and modifying the virtual boundary PPS signalling with an option to use slice level signalling:

- CIP ensure the reconstruction of Intra CU adjacent to the dirty region, since some reference samples uses dirty pixels. CIP is activated through the JVET_IR_ENC_ONLY since it is available in the VTM software while it is not yet included in the VVC draft.

The following signalization was modified.

PPS level:

| pps_constrained_intra_pred_enabled_flag | u(1) |
| --- | --- |

Slice level:

| if ( slice_type != I ) { |  |
| --- | --- |
| if( pps_constrained_intra_pred_enabled_flag ){ |  |
| slice_constrained_intra_pred_override_enabled_flag | u(1) |
| } |  |
| } |  |

- The existing virtual boundaries implementation is signalled at the PPS level since the boundaries positions are assumed static along the video. However, in GRA case, the boundary moves for each frame. This feature is activated through JVET_IR_ENC_ONLY_BOUND and consists in adding the following signalization:
PPS level:

| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| --- | --- |
| pps_virtual_boundaries_slice_signalling | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag && ! pps_virtual_boundaries_slice_signalling) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |

Slice level:

| if( pps_loop_filter_across_virtual_boundaries_disabled_flag && pps_virtual_boundaries_slice_signalling) { |  |
| --- | --- |
| slice_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < slice_num_ver_virtual_boundaries; i++ ) |  |
| slice_virtual_boundaries_pos_x[ i ] | u(v) |
| slice_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < slice_num_hor_virtual_boundaries; i++ ) |  |
| slice_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |

The other macros JVET_FIRST_CYCLE_ISNOT_IR and JVET_CE11_CHECK_DEC_RA _FOR_IR activated through JVET_CE11_21A are defined for the testing conditions defined in CE11 and detailed in section 3.

## CE11-2.1.b: locally constrained intra prediction

On top of the CE11-2.1.a, this second test introduces locally constrained intra prediction instead of the classical CIP applied to the whole frame. 
This local CIP is signalled at a slice level via a window area using 4 values:

| if ( slice_type != I ) { |  |
| --- | --- |
| if( pps_constrained_intra_pred_enabled_flag ){ |  |
| slice_constrained_intra_pred_override_enabled_flag | u(1) |
| if(slice_constrained_intra_pred_override_enabled_flag){ |  |
| slice_local constrained_intra_pred enabled_flag | u(1) |
| if( slice_local constrained_intra_pred enabled_flag ){ |  |
| slice_CIP_window_left_offset | ue(v) |
| slice_CIP_window_top_offset | ue(v) |
| slice_CIP_window_width | ue(v) |
| slice_CIP_window_height | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Inside this CIP window, reference sample pixels can be used for intra prediction only if they are intra-predicted and already inside the CIP window. At the window borders, reference sample pixels intra predicted but outside of the CIP window cannot be used. Outside this CIP window, there is no constraint for intra prediction.

This tool allows restricting the CIP only around the intra refresh column. However, since reference sample pixels can have a width of two times the maximum CU width, the local CIP window defined in this test as a width equals to the maximum CU width. It ensures that a CU inside the clean region but outside the CIP window will not use pixels from the dirty region. Indeed, for these CUs and small intra refresh column, the reference sample pixels can go further than the intra refresh column, thus using dirty pixels.

## CE11-2.2: specific intra refresh signalization

The main drawback of CE11-2.1 solutions is the need to indicate for each picture the new position of refresh area since this is moving. In this test, a straightforward signalling is proposed to gathers efficiently the modifications. The signalization is as follows

- at SPS level:

| sps_intra_refresh_enabled_flag | u(1) |
| --- | --- |

- at PPS level:

| if ( sps_intra_refresh_enabled_flag ){ |  |
| --- | --- |
| pps_intra_refresh mode_flag | u(1) |
| pps_intra_refresh period | ue(v) |
| pps_intra_refresh size1 | ue(v) |
| pps_intra_refresh nb_size1 | ue(v) |
| pps_intra_refresh delta_QP | ue(v) |
| } |  |

- pps_intra_refresh_mode specifies the mode of intra refresh (Column or Line for instance)
- pps_intra_refresh_period specifies the period of the intra refresh cycle. 
- pps_intra_refresh_size1 specifies the first size of the refresh pattern (width of the column, height of the line). This is signaled as a multiple of the CU min size (0 for 8 pixels, 1 for 16 pixels, etc.). The second size intra_refresh_size2 is directly deduced from the first one. 
- pps_intra_refresh_Nb_size1 specifies the number of column using intra_refresh_size1.  The number of column using  intra_refresh_size2 is deduced using intra_refresh_period. 
- pps_intra_refresh_delta_qp specifies a delta QP for the intra refresh blocks. This allows to optimize the encoder quality balancing between intra refresh blocks and others.

- at Slice level:

| if ( slice_type != I && sps_intra_refresh_enabled_flag ){ |  |
| --- | --- |
| slice_intra_refresh_direction_flag | u(1) |
| slice_intra_refresh_position | ue(v) |
| } |  |

- slice_intra_refresh_direction specifies the direction of the refresh (right to left/left to right for Column, top to bottom/bottom to top for Line). This is used to determine motion vectors constraints during the encoding/decoding.

- slice_intra_refresh_position specifies the position of the intra refresh blocks, this is given according to pps_intra_refresh_size1 et pps_intra_refresh_nb_size1.

This dedicated signalization allows enabling the modifications as presented in test CE11-2.1.b while avoiding all previous signalization for virtual boundaries and CIP since it is deduced at the decoding side thanks to the intra-refresh signalization.

Secondly, the local CIP window can be narrowed to the size of the intra refresh column since at decoding side it is possible to deduce which reference pixels are in the dirty or clean region and forbid the use of dirty pixels.

Finally, the following flags are not transmitted for CU derived as intra refresh CU: pred_mode_flag and cu_skip_flag

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) && cu_isIntraRefresh[x0][y0] == 0) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && cu_isIntraRefresh[x0][y0] == 0) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |

# Experimental Results

The simulations are compared to the reference described in [1], using common test conditions (CTC) and low-delay B Main10 configuration as a basis. The following changes were agreed on by all proponents:

- The GOP size is 1 and each picture refers up to 4 immediate preceding pictures in decoding order.

- An IDR picture is inserted every N pictures. N being the frame rate of the tested sequence.

- IntraQPOffset and QPoffset are set equal to 0.

- SEIDecodedPictureHash is set to 1.

- PSNR and bitrates computed without the first I picture + the first cycle (meaning that the first N pictures are not considered in BD-Rate computations), this is activated through the macro JVET_CE11_REMOVE_RD_FOR_FIRST_CYCLE

After discussions on the JVET mailing list with CE11 proponents, it was agreed that the first cycle should be encoded as the first cycle of the reference. Then, the second cycle begins with an intra frame for the reference and with GRA picture for tests. This feature is activated through the macro JVET_FIRST
_CYCLE_ISNOT_IR.

The additional parameters for CE11-2.1.a, CE11-2.1.b, and CE11-2.2 are the same. They are added to the configuration file are as follows:

IntraRefreshType : 1		# Intra Refresh type flag 0: disable, 1: activated

IntraRefreshStart : 0			# Position of the first column of Intra refresh (in pixels)

IntraRefreshDirection : 0	# Direction of Intra Refresh 0: left to right, 1: right to left

IntraRefreshPeriod	 : 30	# Period of the Intra Refresh

LoopFilterAcrossVirtualBoundariesDisabledFlag :1   # Disables in-loop filtering operations across the virtual boundaries.

VirtualBoundariesSliceSignalling :1	  # Modifies the PPS signalling with a slice signaling

The macro JVET_CE11_CHECK_DEC_RA _FOR_IR is present for testing purpose. It consists in providing enabling a new decoding parameter “RandomAccessPos”. For the frame with POC equal to this value, the decoding process discards the buffer of reference frames. After a full refresh cycle, the displayed picture should be totally reconstructed.

## Simulation result of CE11-2.1.a

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 5,92% | 13,22% | 22,80% | 97% | 110% |
| Class C | 8,68% | 7,54% | 9,04% | 95% | 108% |
| Class E | 16,70% | 13,05% | 16,40% | 97% | 127% |
| Overall | 9,54% | 11,28% | 16,61% | 96% | 114% |
| Class D | 11,23% | 16,56% | 16,99% | 99% | 111% |
| Class F | 24,18% | 26,64% | 29,78% | 120% | 133% |

## Simulation result of CE11-2.1.b

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 3,69% | 9,51% | 17,71% | 109% | 112% |
| Class C | 6,68% | 4,38% | 5,27% | 96% | 110% |
| Class E | 16,77% | 12,94% | 15,55% | 97% | 127% |
| Overall | 7,96% | 8,66% | 13,02% | 101% | 115% |
| Class D | 10,13% | 14,20% | 14,46% | 101% | 113% |
| Class F | 22,03% | 23,30% | 26,17% | 120% | 133% |

## Simulation result of CE11-2.2

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C | 5,68% | 2,70% | 3,58% | 97% | 111% |
| Class E | 13,94% | 8,99% | 12,11% | 97% | 127% |
| Overall |  |  |  |  |  |
| Class D | 8,82% | 11,93% | 12,57% | 101% | 113% |
| Class F |  |  |  |  |  |

# Conclusion

The current contribution document presents and reports the result of the CE tests CE11-2.1.a, CE11-2.1.b, CE11-2.2. Three methods are proposed to support GRA using different levels of normative changes. It is recommended to integrate the suggested modifications within the next VTM software version and to consider the proposed solutions for integration in the standard.

# Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

# References

[1] J.-M. Thiesse, D.Nicholson, “Improved Cyclic Intra Refresh”, JVET-K0212, July 2018.

[2] K.Kazui, “AHG14: Study of methods for progressive intra refresh”, JVET-L0079, October 2018.

[3] J.-M. Thiesse, D.Nicholson, D.Gommelet “AHG14: Intra Refresh Test conditions and Anchors generation Proposal”, JVET-L0160, October 2018.

[4] J.-M. Thiesse, D.Nicholson, D.Gommelet “AHG14: Normative Intra Refresh Proposal”, JVET-L0161, October 2018.

[5] J.-M. Thiesse, D.Gommelet, D.Nicholson “AHG14: Updates on Intra Refresh Proposal”, JVET-N0391, March 2019.

[6] K. Kazui, J.M. Thiesse, Hendry, K. Kawamura, “Description of Core Experiment 11 (CE11): Gradual random access”, JVET-N1031, March 2019.

[7] S.Y. Lin, L. Liu, J.L. Lin, Y.C. Chang, C.C. Ju, P. Hanhart, Y. He, “AHG12: Loop filter disabled across virtual boundaries”, JVET-N0438, March 2019.

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).