| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0113-v1 |
| --- | --- |

| Title: | AHG17: On parsing dependency between parameter sets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Futurewei) 10180 Telesis Court San Diego, CA, 92121, USA Jill Boyce (Intel) | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com jill.boyce@intel.com |
| Source: | Futurewei Technologies, Inc., Intel |  |  |

_____________________________

# Abstract

This contribution proposes the following two aspects:

- To allow parsing dependency between different types of parameter sets, mainly to avoid the burden in designing of parameters for making sure that there is no such parsing dependency, and at the same time to avoid some redundant signalling overhead of lower-level parameter sets; and

- To not specify a parameter sets activation process but rather use phrases like "the active xPS" in the VVC specification instead of phrases like "the referred xPS".

The text changes for the above proposals are included in an attachment to this contribution, with change marks relative to JVET-N1001-v7.

# Discussion

In AVC and HEVC, the designs of parameter sets followed a design principle where parsing dependency between different types of parameter sets is disallowed. Consequently, e.g., if some PPS syntax elements have a semantics dependency on some SPS syntax elements, to avoid parsing dependency of PPS syntax on SPS syntax, some information that exists in the SPS is repeatedly signalled in the PPS. For example, the AVC PPS syntax includes the syntax element pic_size_in_map_units_minus1, which is specified to be always equal to PicSizeInMapUnits − 1, while PicSizeInMapUnits is derived by SPS syntax elements pic_width_in_mbs_minus1 and pic_height_in_map_units_minus1.

In our understanding, the justification of the design principle was to allow parsing of parameter sets independently of each other. For example, PPSs can be parsed without parsing the referred SPSs first.

However, with some thinking and discussions, we could not figure out an actual scenario where a PPS would have to be parsed before parsing of the referred SPS. When provided in the bitstream, it can always be required that an SPS is present earlier than any PPS that refers to the PPS. When provided by an external means, e.g., in a Session Description Protocol (SDP) file, or the sample entry of an ISO base media file format track, such order can also be required.

Therefore, we feel that this design principle can actually be given up.

Our above understanding has been strengthened by the fact that, at the 14th JVET meeting in Geneva, two adoptions, on uniform-spaced tile/brick signalling and virtual boundaries, respectively, have both accidentally introduced PSS parsing dependencies on SPS. Resolving of those introduced parsing dependencies would either need to introduce more signalling overhead in the PPS or, in the case of uniform-spaced tile/brick signalling, revoke entirely or partially the adoption.

Here is one more factor we would like to use to advocate giving up independent parameter set parsing. Currently the VVC draft text only has an empty clause on parameter set activation, and compared to HEVC, there are now two more types of parameter sets, namely APS and DPS.

If independent parameter set parsing is given up, we believe then there will no longer be a need to work on the parameter set activation process, and throughout the spec the phrases like "the referred xPS" can be simply replaced with "the active xPS" and alike.

In addition, the newly adopted uniform tile/brick signalling as well as the virtual boundary signalling as they are.

In conclusion, giving up independent parameter set parsing needs much less work to do, does not lose anything really useful, and keep something useful (like the newly adopted uniform tile/brick signalling). So, why not?

# Proposal

Based on the above reasoning, we propose the following:

- For VVC parsing dependency between different types of parameter sets is allowed, mainly to avoid the burden in designing of parameters for making sure that there is no such parsing dependency, and at the same time to avoid some redundant signalling overhead of lower-level parameter sets.

- Do not specify a parameter sets activation process, but rather use phrases like "the active xPS" in the VVC specification instead of phrases like "the referred xPS".

The text changes for the above proposals are included in an attachment to this contribution, with change marks relative to JVET-N1001-v7.

# Patent rights declaration(s)

Futurewei Technologies, Inc. and Intel do not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the contributors).