| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0114-V2 |
| --- | --- |

| Title: | CE8: Unified MVD and Block Vector Difference Coding (CE8-1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar Seethal Paluri Seung-Hwan Kim 10225 Willow Creek Rd, San Diego, CA, U.S.A. 92131 | Tel: Email: | +1-858-635-5226 mehdi.salehifar@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This document reports on the core experiment test CE8-1.2, “Unified MVD and BVD Coding”, which is to address the different characteristics of IBC Block Vector Difference (BVD). In test 1 three additional context coded bin for each MVDx and MVDy component, including gt3, gt7, and gt15 flags are used.  In test 2 one additional context coded bin for each MVDx and MVDy component, including gt3 is used.

Test 1 reportedly provides 0.81%, 0.82 % and 0.88 % luma BD-rates improvement for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions.

Test 1 reportedly provides 0.00%, -0.03 % and 0.01 % luma BD-rates differences for the AI, RA, and LB settings, respectively, for CTC test sequences under the CTC common test conditions.

Test 2 reportedly provides 0.67%, 0.59 % and 0.51 % luma BD-rates improvement for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions.

Test 2 reportedly provides 0.00%, -0.02 % and 0.00 % luma BD-rates differences for the AI, RA, and LB settings, respectively, for CTC test sequences under the CTC common test conditions.

One additional test based on JVET-O0053 encoder modification on top of Test 2 is performed.

Additional test 2 results reportedly provides 0.89%, 0.73 % and 0.73 % luma BD-rates improvement for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions.

# Introduction

In HEVC and VTM5.0, MVD is coded by MVDx and MVDy component independently. Each MVD component is coded by greather_than_zero (GT0), greather_than_one (GT1), and the remaining absolute value is coded by Exponential Golomb code with order 1 (EG1).

# Problem Statement

Since the adoption of intra block copy (IBC), the existing MVD coding is also used to code MVD and BVD. However, the statistics of IBC MVD is very different from that of non-IBC MVD. In addition, the statistics between MVDx and MVDy is also different each other in IBC case as shown in Fig. 1.

Here is the stats for absolute value of MVD of IBC case:

Also Fig 1 is the stats for the bin value up to 50:

Fig 1: Absolute value of MVD x and y distribution for IBC case

In summary in IBC case the data is less skewed , is asymetric between x and y, also it is bounded to a certain value, which is smaller than normal MVD.

# Proposed Method

In this contribution we propose two unfied MVD/BVD coding methods as follows:

- Test 1: Three additional context coded bin for each MVDx and MVDy component, including gt3, gt7, and gt15 flags.

- Test 2: One additional context coded bin for each MVDx and MVDy component, including gt3 flag.

In particular in test 1 first a flag abs_mvd_greater0_flag[] sent to determine if absolute value of MVDi is greater than 0 or not.

If the flag abs_mvd_greater0_flag[] is 1, then another flag abs_mvd_greater1_flag[]  is coded to determine if the absolute value of MVDi is greater than 1 or not.

If the flag abs_mvd_greater1_flag[] is 1, then another flag abs_mvd_greater3_flag[] is coded to determine if the absolute value of MVDi is greater than 3 or not.

If the flag abs_mvd_greater3_flag[] is 0, then a flag coded to determine if the absolute value of MVDi is 2 or 3. If the flag abs_mvd_greater3_flag[] is 1, then another flag abs_mvd_greater7_flag[]  is coded to determine if the absolute value of MVDi is greater than 7 or not.

If the flag abs_mvd_greater7_flag[] is 0, then a FLC used to determine if the absolute value of MVDi is 4 to 7. If the flag abs_mvd_greater7_flag[] is 1, then another flag abs_mvd_greater15_flag[]  is coded to determine if the absolute value of MVDi is greater than 15 or not.

If the flag abs_mvd_greater15_flag[] is 0, then a FLC used to determine if the absolute value of MVDi is 8 to 15. If the flag abs_mvd_greater15_flag[] is 1, then remaining level of absolute value of MVDi  (absolute value of MVDi  minus 16) is coded using Exponential Golomb of order 4.

Finally for all the cases with absolute value of MVDi greater than 0, a sign flag mvd_sign_flag[]  is coded.

This method is actually similar to the current MVD coding in VTM5, however the EG prefixes till value 15 are coded with context coded bin instead of the bypass bin.

So for the simplification another way to represent it is to combine all the abs_mvd_greaterX_flag[] together as abs_mvd_prefix[] and binarized using Truncated Unary and context coded.

| abs_mvd_prefix |  | gt0 | gt1 | gt3 | gt7 | gt15 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 0 |  |  |  |  |
| 1 |  | 1 | 0 |  |  |  |
| 2 |  | 1 | 1 | 0 |  |  |
| 3 |  | 1 | 1 | 1 | 0 |  |
| 4 |  | 1 | 1 | 1 | 1 | 0 |
| 5 |  | 1 | 1 | 1 | 1 | 1 |

For the case of abs_mvd_prefix[] bigger than 1 and smaller than 5 (abs mvd above 1, below 16), FLC code abs_mvd_suffix_below16[] is used to code the rest. For the case of abs_mvd_prefix[] equals 5 (abs mvd greater than or equal 16) Exponential Golomb code of order 4 is used to code the remaining.

| Abs Value Level | abs_mvd_prefix | FLC |
| --- | --- | --- |
| 0 | 0 | N/A |
| 1 | 1 | N/A |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 3 | 00 |
| 5 | 3 | 01 |
| 6 | 3 | 10 |
| 7 | 3 | 11 |
| 8 | 4 | 000 |
| 9 | 4 | 001 |
| 10 | 4 | 010 |
| 11 | 4 | 011 |
| 12 | 4 | 100 |
| 13 | 4 | 101 |
| 14 | 4 | 110 |
| 15 | 4 | 111 |
| > 15 | 5 | No FLC/ Use EG4 for the remaining |

Test 2 is similar to method 1, only using one additional context coded gt3 flag.

# Experimental results

The proposed method has been implemented based on VTM 5.0 reference software according to common test conditions defined in JVET-N1010 [1].

Results for CE8-1.2-A (CTC Condition):

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 90% |
| Class F | -0.31% | -0.37% | -0.30% | 102% | 107% |
| Class SCC | -0.81% | -0.79% | -0.77% | 101% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.03% | 101% | 102% |
| Class A2 | -0.05% | 0.02% | 0.03% | 101% | 102% |
| Class B | -0.02% | -0.05% | -0.11% | 101% | 104% |
| Class C | -0.04% | -0.14% | -0.23% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.05% | -0.10% | 101% | 103% |
| Class D | 0.01% | 0.01% | 0.26% | 100% | 103% |
| Class F | -0.26% | -0.35% | -0.09% | 101% | 104% |
| Class SCC | -0.82% | -0.74% | -0.75% | 101% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.18% | 0.24% | 100% | 103% |
| Class C | 0.02% | 0.00% | -0.05% | 101% | 107% |
| Class E | 0.01% | 0.45% | -0.22% | 101% | 105% |
| Overall | 0.01% | 0.18% | 0.03% | 101% | 105% |
| Class D | 0.00% | -0.03% | 0.74% | 101% | 105% |
| Class F | -0.25% | -0.22% | -0.40% | 101% | 104% |
| Class SCC | -0.88% | -0.87% | -0.81% | 100% | 105% |

Results for CE8-1.2-A (CE8 Condition):

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.02% | 0.00% | 138% | 103% |
| Class A2 | -0.80% | -1.08% | -0.99% | 143% | 103% |
| Class B | -0.79% | -0.72% | -0.87% | 150% | 104% |
| Class C | -0.30% | -0.23% | -0.22% | 152% | 104% |
| Class E | -1.28% | -1.26% | -1.34% | 152% | 101% |
| Overall | -0.64% | -0.64% | -0.68% | 148% | 103% |
| Class D | -0.57% | -0.56% | -0.67% | 144% | 90% |
| Class F | -0.31% | -0.37% | -0.30% | 102% | 106% |
| Class SCC | -0.81% | -0.79% | -0.77% | 101% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.09% | 0.06% | 106% | 103% |
| Class A2 | -0.10% | -0.27% | -0.11% | 106% | 102% |
| Class B | -0.15% | -0.47% | -0.56% | 108% | 104% |
| Class C | 0.14% | -0.31% | -0.13% | 112% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.31% | -0.23% | 108% | 103% |
| Class D | 0.09% | -0.15% | -0.04% | 111% | 103% |
| Class F | -0.26% | -0.35% | -0.09% | 101% | 104% |
| Class SCC | -0.82% | -0.74% | -0.75% | 101% | 105% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.13% | -0.08% | 118% | 105% |
| Class C | 0.09% | 0.01% | -0.30% | 124% | 110% |
| Class E | 0.02% | -0.11% | 2.08% | 113% | 106% |
| Overall | 0.04% | 0.03% | 0.39% | 119% | 107% |
| Class D | 0.07% | 0.09% | 0.54% | 123% | 104% |
| Class F | -0.25% | -0.22% | -0.40% | 100% | 105% |
| Class SCC | -0.88% | -0.87% | -0.81% | 101% | 107% |

Results for CE8-1.2-B (CTC Condition):

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 105% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 90% |
| Class F | -0.24% | -0.27% | -0.21% | 102% | 106% |
| Class SCC | -0.67% | -0.61% | -0.63% | 102% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.22% | 0.06% | 102% | 102% |
| Class A2 | -0.03% | 0.08% | -0.02% | 101% | 102% |
| Class B | -0.01% | -0.10% | -0.12% | 101% | 104% |
| Class C | -0.04% | -0.10% | -0.21% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.09% | -0.09% | 101% | 103% |
| Class D | 0.03% | -0.06% | 0.16% | 101% | 104% |
| Class F | -0.20% | -0.17% | -0.10% | 101% | 105% |
| Class SCC | -0.59% | -0.60% | -0.63% | 102% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.19% | 0.13% | 100% | 103% |
| Class C | -0.02% | -0.01% | -0.33% | 101% | 108% |
| Class E | 0.00% | 0.32% | -0.12% | 101% | 105% |
| Overall | 0.00% | 0.16% | -0.09% | 101% | 105% |
| Class D | -0.05% | 0.10% | 0.49% | 100% | 105% |
| Class F | -0.14% | 0.36% | 0.39% | 101% | 104% |
| Class SCC | -0.51% | -0.50% | -0.56% | 101% | 106% |

Results for CE8-1.2-B (CE8 Condition):

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.01% | 0.04% | 139% | 103% |
| Class A2 | -0.79% | -1.05% | -0.97% | 142% | 104% |
| Class B | -0.79% | -0.71% | -0.86% | 151% | 105% |
| Class C | -0.31% | -0.23% | -0.24% | 152% | 103% |
| Class E | -1.28% | -1.18% | -1.30% | 152% | 101% |
| Overall | -0.64% | -0.62% | -0.67% | 148% | 103% |
| Class D | -0.55% | -0.51% | -0.65% | 144% | 90% |
| Class F | -0.24% | -0.27% | -0.21% | 102% | 107% |
| Class SCC | -0.67% | -0.61% | -0.63% | 102% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.22% | 0.19% | 106% | 103% |
| Class A2 | -0.09% | -0.24% | -0.16% | 106% | 102% |
| Class B | -0.14% | -0.26% | -0.70% | 108% | 104% |
| Class C | 0.16% | -0.18% | -0.21% | 112% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.23% | -0.28% | 108% | 103% |
| Class D | 0.13% | -0.01% | 0.45% | 111% | 102% |
| Class F | -0.20% | -0.17% | -0.10% | 101% | 104% |
| Class SCC | -0.59% | -0.60% | -0.63% | 101% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.21% | -0.39% | 119% | 105% |
| Class C | 0.07% | -0.06% | -0.14% | 124% | 110% |
| Class E | 0.00% | -0.74% | 1.59% | 114% | 107% |
| Overall | 0.03% | -0.12% | 0.19% | 119% | 107% |
| Class D | 0.06% | -0.29% | 0.52% | 123% | 103% |
| Class F | -0.14% | 0.36% | 0.39% | 100% | 105% |
| Class SCC | -0.51% | -0.50% | -0.56% | 101% | 108% |

Results for CE8-1.2-B with Encoder Modification (CE8 Condition):

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | 0.03% | 141% | 104% |
| Class A2 | -0.80% | -0.99% | -1.02% | 145% | 103% |
| Class B | -0.82% | -0.69% | -0.87% | 154% | 105% |
| Class C | -0.31% | -0.24% | -0.25% | 154% | 105% |
| Class E | -1.33% | -1.33% | -1.40% | 155% | 102% |
| Overall | -0.66% | -0.63% | -0.70% | 150% | 104% |
| Class D | -0.58% | -0.68% | -0.70% | 146% | 92% |
| Class F | -0.35% | -0.27% | -0.29% | 103% | 107% |
| Class SCC | -0.89% | -0.80% | -0.79% | 102% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.25% | 0.11% | 107% | 102% |
| Class A2 | -0.14% | -0.34% | -0.14% | 107% | 101% |
| Class B | -0.18% | -0.45% | -0.68% | 108% | 104% |
| Class C | 0.14% | -0.26% | -0.27% | 112% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.34% | -0.30% | 109% | 103% |
| Class D | 0.12% | 0.01% | 0.04% | 111% | 102% |
| Class F | -0.30% | -0.08% | -0.11% | 101% | 104% |
| Class SCC | -0.73% | -0.54% | -0.64% | 102% | 105% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.27% | 0.05% | 120% | 104% |
| Class C | 0.11% | 0.24% | -0.10% | 125% | 108% |
| Class E | 0.08% | -0.68% | 0.20% | 115% | 106% |
| Overall | 0.06% | 0.02% | 0.04% | 120% | 106% |
| Class D | 0.07% | -0.28% | 0.70% | 124% | 104% |
| Class F | -0.24% | -0.67% | -0.14% | 101% | 105% |
| Class SCC | -0.73% | -0.63% | -0.53% | 101% | 107% |

# Conclusion

In the contribution modified MVD/BVD coding proposed, which improves the BD rate performance. It is recommended that the proposed method be adopted in the next VTM.

# Bin to Bit Ratio Stat

Bin to bit ratio stat for AI cases are provided:

Test A:

Test B:

Test B with encoder modification:

VTM 5:

# Reference

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).