Test A:

#### Motion vector difference syntax

| mvd_coding( x0, y0, refList ,cpIdx ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag [0] | ae(v) |
| abs_mvd_greater0_flag [1] | ae(v) |
| if(abs_mvd_greater0_flag [0]) |  |
| abs_mvd_greater1_flag[0] | ae(v) |
| if(abs_mvd_greater0_flag [1]) |  |
| abs_mvd_greater1_flag[1] | ae(v) |
| if(abs_mvd_greater0_flag [0]){ |  |
| if(abs_mvd_greater1_flag [0]){ |  |
| abs_mvd_greater3_flag[0] | ae(v) |
| if(!abs_mvd_greater3_flag [0]) |  |
| abs_mvd_suffix(2) [0] | ae(v) |
| else{ |  |
| abs_mvd_greater7_flag[0] | ae(v) |
| if(!abs_mvd_greater7_flag [0]) |  |
| abs_mvd_suffix(4) [0] | ae(v) |
| else{ |  |
| abs_mvd_greater15_flag[0] | ae(v) |
| if(!abs_mvd_greater15_flag [0]) |  |
| abs_mvd_suffix(8) [0] | ae(v) |
| else{ |  |
| abs_mvd _minus16[0] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| mvd_sign_flag[0] | ae(v) |
| } |  |
| if(abs_mvd_greater0_flag [1]){ |  |
| if(abs_mvd_greater1_flag [1]){ |  |
| abs_mvd_greater3_flag[1] | ae(v) |
| if(!abs_mvd_greater3_flag [1]) |  |
| abs_mvd_suffix(2) [1] | ae(v) |
| else{ |  |
| abs_mvd_greater7_flag[1] | ae(v) |
| if(!abs_mvd_greater7_flag [1]) |  |
| abs_mvd_suffix(4) [1] | ae(v) |
| else{ |  |
| abs_mvd_greater15_flag[1] | ae(v) |
| if(!abs_mvd_greater15_flag [1]) |  |
| abs_mvd_suffix(8) [1] | ae(v) |
| else{ |  |
| abs_mvd _minus16[1] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| mvd_sign_flag[1] | ae(v) |
| } |  |
| } |  |

#### Motion vector difference semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0, where compIdx represent direction x or y.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1.

When abs_mvd_greater1_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_greater3_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 3.

When abs_mvd_greater3_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_greater7_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 7.

When abs_mvd_greater7_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_greater15_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 15.

When abs_mvd_greater15_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_suffix(N) [compIdx ] specifies the suffix of the mvd which could be 0 to N-1.

abs_mvd_minus16[ compIdx ] plus 16 specifies the absolute value of a motion vector component difference.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows:

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater3_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater7_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater15_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_suffix(N) [] | FL | cMax = N-1 |
|  | abs_mvd_minus16[ ] | EG4 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| abs_mvd_greater0_flag[compIdx ] | compIdx==0 ? 0 : 1 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater3_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater7_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater15_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_suffix(N) [] | bypass | bypass | bypass | na | na | na |
| abs_mvd_minus16[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |

Test B:

#### Motion vector difference syntax

| mvd_coding( x0, y0, refList ,cpIdx ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag [0] | ae(v) |
| abs_mvd_greater0_flag [1] | ae(v) |
| if(abs_mvd_greater0_flag [0]) |  |
| abs_mvd_greater1_flag[0] | ae(v) |
| if(abs_mvd_greater0_flag [1]) |  |
| abs_mvd_greater1_flag[1] | ae(v) |
| if(abs_mvd_greater0_flag [0]){ |  |
| if(abs_mvd_greater1_flag [0]){ |  |
| abs_mvd_greater3_flag[0] | ae(v) |
| if(!abs_mvd_greater3_flag [0]) |  |
| abs_mvd_suffix_flag [0] | ae(v) |
| else |  |
| abs_mvd _minus4[0] | ae(v) |
| } |  |
| mvd_sign_flag[0] | ae(v) |
| } |  |
| if(abs_mvd_greater0_flag [1]){ |  |
| if(abs_mvd_greater1_flag [1]){ |  |
| abs_mvd_greater3_flag[1] | ae(v) |
| if(!abs_mvd_greater3_flag [1]) |  |
| abs_mvd_suffix_flag [1] | ae(v) |
| else |  |
| abs_mvd _minus4[1] | ae(v) |
| } |  |
| mvd_sign_flag[1] | ae(v) |
| } |  |
| } |  |

#### Motion vector difference semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0, where compIdx represent direction x or y.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1.

When abs_mvd_greater1_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_greater3_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 3.

When abs_mvd_greater3_flag [ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_suffix_flag [compIdx] specifies the suffix of the mvd which could be 0 or 1.

abs_mvd_minus4[ compIdx ] plus 4 specifies the absolute value of a motion vector component difference.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows:

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater3_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_suffix_flag [] | FL | cMax = 1 |
|  | abs_mvd_minus4[ ] | EG2 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| abs_mvd_greater0_flag[compIdx ] | compIdx==0 ? 0 : 1 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater3_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_suffix_flag [] | bypass | na | na | na | na | na |
| abs_mvd_minus4[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |