| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0115-v1 |
| --- | --- |

| Title: | CE7-2.3: Joint coding of chroma residuals |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Geert Van der Auwera, Adarsh K. Ramasubramonian, Muhammed Coban, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com geertv@qti.qualcomm.com aramasub@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE7-2.3, i.e., the joint coding of chroma residual method previously proposed in JVET-N0347. Compared to JVET-N0054 – the joint coding of chroma residual method adopted in VTM-5 – the proposed method signals two residuals, i.e., JointResCb = (ResCb − ResCr) / 2 and JointResCr = (ResCb + ResCr) / 2, which are independently transformed and quantized. The results for CTC and lowQP test conditions compared to VTM-5.0 are reported below:

Table-1: Average BD rate results for CTC

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | −0.12% | −0.18% | 0.54% | 102% | 99% |
| RA | −0.06% | 0.18% | 0.59% | 100% | 100% |
| LD | −0.02% | 3.44% | 0.83% | 99% | 101% |

Table-2: Average BD rate results for low-QP

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | −0.19% | 0.20% | 0.13% | 104% | 102% |
| RA | −0.10% | 0.06% | 0.02% | 101% | 98% |
| LD | −0.06% | 0.16% | −0.17% | 102% | 102% |

# Introduction

As described in JVET-N0347 [1], the proposed method signals two residuals (instead of one residual as JVET-N0054 [2]): JointRes1 and JointRes2. The residual of Cb and Cr component are derived from the joint residuals are as follows:

resCb = JointResCb + JointResCr

resCr = −JointResCb + JointResCr

The joint chroma coding is applied on the TU level, and a tu_joint_CbCr_residual flag is signalled for each TU having cbf_cb equal to 1 to indicate whether the joint chroma coding (tu_joint_CbCr_residual equal to 1) or the independent chroma coding (tu_joint_CbCr_residual equal to 0) is applied. When a TU uses joint chroma coding, a QP offset of −1 is applied (identical to VTM-5). At the encoder side, the lambda value is decreased to 0.8x for intra blocks (0.6x in VTM-5), and 0.6x for non-intra blocks (idem VTM-5). Only one context is used for the jointCbCr flag.

# Experimental results

The proposed technique has been implemented on top of VTM-5.0. The experimental results under the AI, RA and LDB configurations are reported in Table-3 for Qp values {22, 27, 32, 37} and in Table-4 for low-Qp values {2, 7, 12, 17} (with first 100 frames), as requested for CE7-2 experiments [3][4].

Table-3: BD rate results for CTC

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.85% | 0.35% | 102% | 99% |
| Class A2 | -0.46% | -0.44% | 0.90% | 103% | 103% |
| Class B | -0.03% | -0.15% | 0.71% | 101% | 98% |
| Class C | -0.06% | 0.06% | 0.51% | 102% | 98% |
| Class E | -0.08% | 0.39% | 0.10% | 101% | 97% |
| Overall | -0.12% | -0.18% | 0.54% | 102% | 99% |
| Class D | -0.05% | 0.17% | 0.13% | 101% | 101% |
| Class F | -0.16% | 0.24% | 0.34% | 100% | 95% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.60% | 0.44% | 98% | 100% |
| Class A2 | -0.19% | 0.19% | 0.79% | 101% | 103% |
| Class B | 0.00% | 0.27% | 0.78% | 98% | 97% |
| Class C | -0.06% | 0.65% | 0.31% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.18% | 0.59% | 100% | 100% |
| Class D | -0.03% | 0.94% | 0.32% | 102% | 104% |
| Class F | -0.16% | 0.38% | 0.18% | 99% | 97% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 3.41% | 0.59% | 99% | 98% |
| Class C | -0.02% | 3.34% | 0.35% | 99% | 106% |
| Class E | -0.02% | 3.63% | 1.88% | 99% | 101% |
| Overall | -0.02% | 3.44% | 0.83% | 99% | 101% |
| Class D | 0.03% | 4.99% | 1.01% | 102% | 96% |
| Class F | -0.08% | 1.67% | -0.62% | 95% | 92% |

Table-4: BD rate results for low-QP

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.02% | 102% | 97% |
| Class A2 | -0.31% | 0.20% | 0.17% | 106% | 102% |
| Class B | -0.10% | 0.09% | 0.07% | 104% | 99% |
| Class C | -0.41% | 0.42% | 0.28% | 106% | 110% |
| Class E | -0.14% | 0.36% | 0.08% | 105% | 100% |
| Overall | -0.19% | 0.20% | 0.13% | 104% | 102% |
| Class D | -0.49% | 0.40% | -0.05% | 106% | 102% |
| Class F | -0.84% | -0.08% | -0.30% | 103% | 98% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.18% | 0.06% | 100% | 97% |
| Class A2 | -0.24% | 0.12% | 0.07% | 105% | 100% |
| Class B | -0.04% | 0.02% | -0.04% | 98% | 97% |
| Class C | -0.14% | 0.27% | 0.02% | 104% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | 0.06% | 0.02% | 101% | 98% |
| Class D | -0.15% | 0.40% | -0.27% | 101% | 98% |
| Class F | -0.46% | 0.06% | -0.35% | 100% | 97% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.06% | -0.10% | 103% | 103% |
| Class C | -0.13% | 0.23% | -0.18% | 102% | 101% |
| Class E | -0.01% | 0.22% | -0.27% | 101% | 102% |
| Overall | -0.06% | 0.16% | -0.17% | 102% | 102% |
| Class D | -0.13% | 0.33% | -0.24% | 100% | 98% |
| Class F | -0.41% | 0.15% | -0.32% | 99% | 95% |

HDR results are pending

In CTC condition, for AI configuration, the proposed method provides coding improvement, however for LD configuration, it provides a loss in U component. For low-QP configuration, such loss in LD is not observed. Overall, the proposed method works well for Class-A2 and Class-F sequences.

# Specification text

7.3.7.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ){ |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| … |  |
| } |  |

8.7.2 Scaling and transformation process:

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

- If cIdx is equal to 2 and tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 1, the residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

resSamples[ x ][ y ]  =  −resSamplesCb[ x ][ y ]		(8-937)

where resSamplesCb is the residual sample array resSamples that is generated in the last invokation of this clause for the Cb component block with cIdx euqal to 1 at the transform block location ( xTbY, yTbY ).

- Otherwise (cIdx is less than 2 or tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 0), the following ordered steps apply:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	(8-938)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

  - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

resSamples[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-939)

  - If cIdx is equal to 2 and tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 1, then resSamplesCb[ x ][ y ] and resSamplesCr[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are modified as follows:

resSamplesCb[ x ][ y ] = resSamplesJointCr[ x ][ y ] + resSamplesJointCb[ x ][ y ] 	(8-940)

resSamplesCr[ x ][ y ] = resSamplesJointCr[ x ][ y ] − resSamplesJointCb[ x ][ y ] 	(8-941)

where resSamplesJointCb is the residual sample array resSamples that is generated in the invocation of this clause for the Cb component block with cIdx equal to 1 at the transform block location ( xTbY, yTbY ) and resSamplesJointCr is the residual sample array resSamples that is generated in the invocation of this clause for the Cr component block with cIdx equal to 2 at the transform block location ( xTbY, yTbY ). resSamplesCb is the modified residual sample array resSamples that is generated in the invocation of this clause for the Cb component block with cIdx equal to 1 at the transform block location ( xTbY, yTbY ) and resSamplesCr is the modified residual sample array resSamples that is generated in the invocation of this clause for the Cr component block with cIdx equal to 2 at the transform block location ( xTbY, yTbY ).

# References

- G. Van der Auwera et al., “CE7-related: Joint coding of chrominance residuals,” Joint Video Experts Team, doc. JVET-N0347, Geneva, CH, Mar. 2019.

- J. Lainema, “CE7-related: Joint coding of chrominance residuals,” Joint Video Experts Team, doc. JVET-N0054, Geneva, CH, Mar. 2019.

[3]   F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

[4]   H Schwarz, M. Coban, "Description of Core Experiment 7 (CE7): Quantization and coefficient coding", Document of Joint Video Experts Team, JVET-N1027.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).