| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0118-v1 |
| --- | --- |

| Title: | CE3-2.2: Enabling parallel reconstruction of small intra-coded chroma blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Adarsh K. Ramasubramonian, Vadim Seregin, Han Huang, Ted Hsieh, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Email: | {lphamvan, geertv, aramasub, vseregin, hanhuang, thsieh, martak}@qti.qualcomm.com, {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Qualcomm Inc. and MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 3-2.2 for increasing the worst-case processing throughput of chroma intra prediction by 4x compared to that in VTM5.0. The approach of this test is to remove the intra prediction dependency between chroma blocks in 16-sample areas. The average (Y, U, V) BD-rate change is (0.07%, 0.02%, 0.09%) and (0.05%, 0.13%, -0.16%) for the RA and LDB configurations, respectively. The average encoding time and decoding time is reportedly (101%, 101%) and (101%, 100%) under the RA and LDB configuration, respectively.

# Introduction

In VTM5.0, it is possible to have small chroma blocks of size smaller than 16 samples including 2x2, 2x4 and 4x2. In a typical hardware video codec, the processing throughput is reduced when a picture is coded using many small blocks. This reduction mainly comes from small intra blocks, because intra blocks have data dependency between neighbouring blocks (e.g., the prediction of a block uses top and left boundary reconstructed samples from neighbouring blocks as reference); hence the reconstruction must be performed sequentially.

The worst-case processing throughput of HEVC occurs when 4x4 chroma intra blocks are processed. In VTM5.0, the size of the smallest chroma intra block is 2x2, and the reconstruction process of a chroma intra block becomes very complex after adopting several tools. It is asserted that the worst-case processing throughput of VTM4.0 is lower than that of HEVC by 4 times.

# Proposed Method

Definition of PPR:

This CE test (CE3-2.2) enables parallel generation of multiple chroma intra coding blocks within a parallel-processable region (PPR) in the single tree case. All the data dependencies between the chroma blocks in the same PPR are eliminated, thus they can be reconstructed in parallel.

Figure 1. PPR example with the minimum number of chroma samples of 16

Dependency removal:

The intra-prediction dependency among the blocks in a 16-sample PPR is eliminated by marking all the blocks within the PPR as “unavailable” for predicting all contained blocks in this PPR. For example, in the top-left case of Figure 1 (quadtree split), the left column of the top-right block is marked as unavailable for intra prediction of this block. The above line of the bottom-left block is unavailable for intra prediction of this block. Also, in this case, the bottom-right block has no available samples for intra prediction. It should be noted that, the block vector of a chroma IBC block is inherited from the corresponding luma block. In this CE test, the block vector of luma component that leads to an overlap between the reference block of the chroma and its PPR area is restricted.

It should be noted that, the substitution of reference samples is automatically performed by the reference substitution process in VVC when a neighbour block is unavailable. In addition, this proposed method is simple, because it does not need any change in other parts of intra prediction.

# Experimental Results

The proposed technique has been implemented on top of VTM5.0. The experimental results under the RA and LDB configurations are summarized in Table 1.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.14% | 0.11% | 99% | 96% |
| Class A2 | 0.03% | 0.08% | 0.11% | 99% | 97% |
| Class B | 0.05% | 0.01% | 0.01% | 101% | 99% |
| Class C | 0.14% | 0.10% | 0.17% | 103% | 109% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.02% | 0.09% | 101% | 101% |
| Class D | 0.21% | 0.62% | 0.63% | 103% | 105% |
| Class F | 0.10% | 0.26% | 0.20% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.14% | -0.33% | 99% | 95% |
| Class C | 0.10% | 0.03% | 0.17% | 103% | 104% |
| Class E | 0.04% | 0.24% | -0.33% | 102% | 102% |
| Overall | 0.05% | 0.13% | -0.16% | 101% | 100% |
| Class D | 0.08% | 0.45% | 1.03% | 100% | 99% |
| Class F | 0.13% | 0.84% | 0.01% | 102% | 98% |

It is reported that the proposed method has an average (Y, U, V) BD-rate change of (0.07%, 0.02%, 0.09%) and (0.05%, 0.13%, -0.16%) for the RA and LDB configuration, respectively.

# Conclusions

This document reports the results of Core Experiment (CE) 3-2.2. The proposed method increases the worst-case processing throughput of VVC by 4x with an average (Y, U, V) BD-rate change of (0.07%, 0.02%, 0.09%) and (0.05%, 0.13%, -0.16%) for the RA and LDB configuration, respectively.

# Patent rights declarations

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).