| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0120 |
| --- | --- |

| Title: | CE8-2.4: Line-based CG Palette Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Chao-Hsiung Hung, Wei-Jung Chien, Ted Hsieh, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, a Line CG based approach is proposed to simplify the buffer usage and syntax in CE8-2.1: palette mode in HEVC. Similar to the coefficient group (CG) used in transform coefficient coding, a CU is divided into multiple line based coefficient group, each consists of m samples, where index runs, palette index values, and quantized colors for escape mode are encoded/parsed sequencially for each CG.

Compared to VTM5.0, the results show Class TGM: -4.35% AI, -1.60% RA, -0.43% LB BD-Rate and Class F -0.81% AI, -0.65% RA, -0.33% LB BD-Rate on YUV4:2:0 sequences. For YUV4:4:4 sequences, the results show -5.94% AI, -4.24% RA, -2.39% LB BD-Rate overall for dual tree off, and -2.34% AI, -1.93% RA, -1.91% LB BD-Rate for dual tree on.

Compared to CE8-2.1:palette mode in HEVC, the results show Class TGM: -0.06 AI, 0.10% RA, 0.00% LB BD-Rate and Class F -0.09% AI, -0.10% RA, -0.05% LB BD-Rate on YUV4:2:0 sequences. For YUV4:4:4 sequences, the results show -0.17% AI, -0.08% RA, -0.31% LB BD-Rate overall for dual tree off, and -0.21% AI, -0.21% RA, -0.12% LB BD-Rate for dual tree on.

# Introduction

In the palette mode design in HEVC, the syntax element palette_index_idc and palette_escape_val are CABAC bypass coded. In order to improve CABAC throughput, syntax element palette_index_idc for the whole CU are grouped and parsed before the palette run coding, i.e., copy_above_palette_indices_flag and palette_run, which are CABAC context coded. The syntax element palette_escape_val for the whole CU, on the other hand, are grouped and parsed after palette run coding.  However, the buffer size needed to store the index values, index map, and escape colors can take up to the CU size in the worse case. In order to reduce the buffer size, a line CG based palette mode is proposed, which divided a CU into multiple segments with m samples, where index runs, palette index values, and quantized colors for escape mode are encoded/parsed sequencially for each CG. In addition, it is proposed to move the parsing of CABAC bypass coded syntax element, i.e., palette_index_idc, palette_escape_val, after palette run coding.

# Proposal

In this proposal, each CU of palette mode is divided into multiple segments of m samples (m=16 in this test) based on the traverse scan mode, as shown in Figure 1.

Figure 1: Sub-block-based index map scanning for palette

The encoding order for palette run coding in each segment is as follows: For each pixel, 1 context coded bin run_copy_flag = 0 is signalled indicating if the pixel is of the same mode as the previous pixel, i.e., if the previous scanned pixel and the current pixel are both of run type COPY_ABOVE or if the previous scanned pixel and the current pixel are both of run type INDEX and the same index value. Otherwise, run_copy_flag = 1 is signaled. The context index for coding run_copy_flag is derived based on the run type (PreviousRunType) of the nearest scan position with run_copy_flag = 0, and the distance from the nearest scan position (PreviousRunTypePosition) with run_copy_flag = 0. Let curSanPos to be the current scan position, and binDist = curSanPos – PreviousRunTypePosition – 1. The context index is derived as

| binDist | 0 | 1 | 2 | 3 | > = 4 |
| --- | --- | --- | --- | --- | --- |
| PreviousRunType = = 1 | 5 | 6 | 6 | 7 | 7 |
| PreviousRunType = = 0 | 0 | 1 | 2 | 3 | 4 |

If the pixel and the previous pixel are of different mode, one context coded bin copy_above_palette_indices_flag is signaled indicating the run type, i.e., INDEX or COPY_ABOVE, of the pixel. Same as the HEVC palette, decoder doesn’t have to parse run type if the sample is in the first row (horizontal traverse scan) or in the first column (vertical traverse scan) since the INDEX mode is used by default. Also, decoder doesn’t have to parse run type if the previously parsed run type is COPY_ABOVE.

After palette run coding of pixels in one segment, the index values (for INDEX mode) and quantized escape colors are bypass coded and grouped apart from encoding/parsing of context coded bins to improve throughput within each line CG. Since the index value is now coded/parsed after run coding, instead of processed before palette run coding as in HEVC, encoder doesn’t have to signal the number of index values num_palette_indices_minus1 and the last run type copy_above_indices_for_final_run_flag.

# Simulation Results

## Test sequences 4:2:0

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for Class F and Class TGM. Table 1 shows the results of CE8.2.4 compared with VTM5.0 on test sequences 4:2:0.

Table 1: Results of the CE8.2.4 on test sequences 4:2:0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.02% | 112% | 101% |
| Class A2 | 0.04% | 0.09% | 0.05% | 112% | 101% |
| Class B | 0.05% | 0.03% | 0.06% | 114% | 106% |
| Class C | 0.05% | 0.07% | -0.01% | 110% | 103% |
| Class E | 0.06% | 0.02% | -0.11% | 113% | 102% |
| Overall | 0.04% | 0.03% | 0.01% | 112% | 103% |
| Class D | 0.03% | 0.24% | -0.04% | 108% | 103% |
| Class F | -0.81% | -0.73% | -0.94% | 109% | 107% |
| Class TGM | -4.35% | -4.70% | -4.04% | 104% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.09% | 0.10% | 102% | 101% |
| Class A2 | 0.06% | -0.05% | 0.10% | 100% | 99% |
| Class B | 0.03% | -0.03% | -0.23% | 101% | 100% |
| Class C | 0.02% | -0.10% | -0.18% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.06% | -0.09% | 101% | 101% |
| Class D | 0.04% | -0.18% | 0.11% | 100% | 101% |
| Class F | -0.65% | -1.01% | -0.76% | 111% | 107% |
| Class TGM | -1.60% | -2.46% | -2.04% | 104% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.18% | 0.21% | 101% | 100% |
| Class C | 0.05% | 0.24% | -0.13% | 102% | 107% |
| Class E | 0.09% | -0.46% | 0.79% | 100% | 98% |
| Overall | 0.04% | 0.04% | 0.24% | 101% | 102% |
| Class D | 0.01% | 0.11% | 1.28% | 105% | 112% |
| Class F | -0.33% | -0.92% | -0.57% | 111% | 102% |
| Class TGM | -0.43% | -0.69% | -0.54% | 108% | 102% |

## Test sequences 4:4:4

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, and RDPCM are turned on for all Classed. Table 2 shows the results with DualITree turned off while Table 3 shows the results with DualITree turned on.

Table 2: Results of the CE8.2.4 on test sequences 4:4:4, DualITree = 0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -16.50% | -23.18% | -22.52% | 99% | 85% |
| TGM 720p | -4.93% | -11.07% | -13.46% | 104% | 97% |
| Animation | 0.04% | -0.25% | -0.27% | 104% | 105% |
| Mixed content | -1.16% | -5.55% | -6.52% | 103% | 98% |
| Camera-Captured content | 0.01% | 0.01% | 0.04% | 104% | 99% |
| Overall | -5.94% | -10.28% | -10.93% | 103% | 95% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -10.72% | -18.28% | -17.12% | 92% | 91% |
| TGM 720p | -4.87% | -10.15% | -12.43% | 101% | 99% |
| Animation | 0.07% | -0.21% | -0.19% | 108% | 103% |
| Mixed content | -0.44% | -4.45% | -5.40% | 106% | 100% |
| Camera-Captured content | -0.01% | 0.24% | 0.18% | 102% | 101% |
| Overall | -4.24% | -8.47% | -8.96% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.86% | -11.99% | -11.37% | 103% | 94% |
| TGM 720p | -2.32% | -6.43% | -8.00% | 105% | 97% |
| Animation | 0.00% | -0.19% | -0.12% | 110% | 105% |
| Mixed content | 0.31% | -3.09% | -4.80% | 107% | 98% |
| Camera-Captured content | -0.03% | 0.03% | -0.11% | 100% | 100% |
| Overall | -2.39% | -5.55% | -6.16% | 105% | 98% |

Table 3: Results of the CE8.2.4 on test sequences 4:4:4, DualITree = 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.87% | -14.11% | -13.85% | 110% | 91% |
| TGM 720p | -1.90% | -7.47% | -8.92% | 109% | 98% |
| Animation | 0.06% | -1.40% | -0.91% | 105% | 97% |
| Mixed content | -0.06% | -6.54% | -6.28% | 108% | 98% |
| Camera-Captured content | 0.06% | 0.02% | 0.01% | 109% | 100% |
| Overall | -2.34% | -7.25% | -7.45% | 109% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.54% | -13.62% | -12.99% | 94% | 93% |
| TGM 720p | -1.35% | -7.44% | -8.78% | 103% | 100% |
| Animation | 0.10% | -0.93% | -0.51% | 108% | 103% |
| Mixed content | -0.55% | -7.14% | -6.13% | 106% | 100% |
| Camera-Captured content | 0.05% | 0.05% | -0.07% | 102% | 102% |
| Overall | -1.93% | -7.16% | -7.11% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.46% | -10.96% | -10.50% | 104% | 95% |
| TGM 720p | -1.43% | -5.72% | -7.52% | 104% | 98% |
| Animation | 0.12% | -0.31% | -0.16% | 112% | 99% |
| Mixed content | -0.48% | -6.95% | -6.55% | 107% | 99% |
| Camera-Captured content | 0.05% | -0.08% | -0.01% | 101% | 100% |
| Overall | -1.91% | -5.89% | -6.14% | 105% | 98% |

# Additional results

## Test sequences 4:2:0, Low QP

The proposed method is evaluated for low QP (QP = 2, 7, 12, 17) according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The DualITree is turned on for all Classes. The IBC, HashME, and RDPCM are turned on only for Class F and Class TGM. Table 4 shows the results of CE8.2.4 compared with VTM5.0 on test sequences 4:2:0.

Table 4: Results of the CE8.2.4 on test sequences 4:2:0, Low QP

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0, CTC, LQP |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.00% |  | -0.02% |  | -0.03% |  | 108% |  | 104% |
| Class A2 | 0.01% |  | 0.01% |  | 0.01% |  | 107% |  | 103% |
| Class B | -0.22% |  | -0.19% |  | -0.16% |  | 109% |  | 104% |
| Class C | -0.18% |  | -0.22% |  | -0.19% |  | 108% |  | 102% |
| Class E | -1.11% |  | -1.89% |  | -2.28% |  | 110% |  | 96% |
| Overall | -0.28% |  | -0.42% |  | -0.47% |  | 108% |  | 102% |
| Class D | -0.09% |  | -0.32% |  | -0.38% |  | 107% |  | 102% |
| Class F | -1.06% |  | -1.22% |  | -1.26% |  | 109% |  | 103% |
| Class TGM | -11.20% |  | -10.66% |  | -10.25% |  | 104% |  | 88% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC, LQP |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.03% | 0.03% |  | 0.05% |  | 110% |  | 109% |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class B | -0.03% | -0.04% |  | 0.00% |  | 104% |  | 109% |  |
| Class C | -0.05% | -0.09% |  | -0.06% |  | 99% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |
| Class D | -0.07% | 0.00% |  | -0.08% |  | 96% |  | 98% |  |
| Class F | -0.90% | -1.00% |  | -1.15% |  | 103% |  | 98% |  |
| Class TGM | -6.54% | -5.08% |  | -5.03% |  | 94% |  | 95% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0, CTC, LQP |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.01% |  | 0.00% |  | 101% |  | 106% |  |
| Class C | 0.00% | 0.01% |  | 0.00% |  | 101% |  | 102% |  |
| Class E | -0.04% | -0.13% |  | -0.25% |  | 101% |  | 105% |  |
| Overall | -0.01% | -0.03% |  | -0.06% |  | 101% |  | 104% |  |
| Class D | 0.00% | -0.07% |  | 0.03% |  | 100% |  | 104% |  |
| Class F | -0.53% | -0.56% |  | -0.57% |  | 106% |  | 104% |  |
| Class TGM | -4.02% | -3.22% |  | -3.07% |  | 97% |  | 93% |  |

## Test sequences 4:4:4, Low QP

The proposed method is evaluated for low QP (QP = 2, 7, 12, 17) according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, and RDPCM are turned on for all classes. Table 5 shows the results with DualITree turned off while Table 6 shows the results with DualITree turned on.

Table 5: Results of the CE8.2.4 on test sequences 4:4:4, DualITree = 0, Low QP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -34.76% | -39.73% | -39.08% | 102% | 73% |
| TGM 720p | -16.04% | -17.92% | -18.97% | 109% | 88% |
| Animation | -0.18% | -0.33% | -0.32% | 106% | 110% |
| Mixed content | -4.43% | -6.07% | -6.04% | 108% | 99% |
| Camera-Captured content | 0.01% | 0.00% | 0.00% | 108% | 111% |
| Overall | -14.45% | -16.63% | -16.73% | 107% | 91% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -25.25% | -32.60% | -31.11% | 89% | 88% |
| TGM 720p | -13.63% | -15.84% | -17.32% | 104% | 96% |
| Animation | -0.02% | -0.15% | -0.11% | 110% | 113% |
| Mixed content | -2.81% | -4.01% | -4.68% | 106% | 98% |
| Camera-Captured content | 0.00% | 0.02% | -0.02% | 105% | 108% |
| Overall | -10.93% | -13.73% | -13.87% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, LQP, DT0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -25.69% | -30.12% | -28.93% | 102% | 88% |
| TGM 720p | -10.77% | -12.91% | -13.95% | 102% | 92% |
| Animation | -0.01% | -0.01% | -0.05% | 107% | 112% |
| Mixed content | -2.06% | -3.60% | -3.98% | 105% | 97% |
| Camera-Captured content | 0.00% | 0.00% | -0.03% | 101% | 102% |
| Overall | -10.14% | -12.20% | -12.24% | 103% | 96% |

Table 6: Results of the CE8.2.4 on test sequences 4:4:4, DualITree = 1, Low QP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -24.85% | -28.62% | -28.80% | 120% | 77% |
| TGM 720p | -9.30% | -12.66% | -12.82% | 121% | 96% |
| Animation | -1.02% | -1.33% | -1.37% | 114% | 104% |
| Mixed content | -3.87% | -5.34% | -5.27% | 120% | 100% |
| Camera-Captured content | 0.01% | 0.02% | 0.02% | 108% | 112% |
| Overall | -10.02% | -12.25% | -12.33% | 118% | 94% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -23.40% | -27.21% | -25.98% | 93% | 90% |
| TGM 720p | -9.02% | -12.29% | -12.90% | 105% | 96% |
| Animation | -0.33% | -0.46% | -0.50% | 105% | 100% |
| Mixed content | -1.86% | -3.08% | -3.18% | 103% | 97% |
| Camera-Captured content | -0.01% | -0.01% | 0.00% | 103% | 105% |
| Overall | -9.06% | -11.21% | -11.07% | 101% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, LQP, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -22.05% | -25.98% | -25.01% | 103% | 88% |
| TGM 720p | -7.13% | -9.61% | -10.22% | 104% | 92% |
| Animation | -0.06% | -0.20% | -0.19% | 106% | 102% |
| Mixed content | -1.54% | -2.73% | -2.88% | 106% | 98% |
| Camera-Captured content | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Overall | -8.10% | -10.07% | -9.99% | 104% | 95% |

# Reference

- Y.-H. Chao, T. Hsieh, M. Karczewicz, “CE8-related: Line-based CG Palette Mode”, JVET-N0550, 14th meeting, Geneva, CH, March 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).