| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0121 |
| --- | --- |

| Title: | CE8-4.2: Dual tree disabled for YUV4:4:4 format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Chao-Hsiung Hung, Hongtao Wang, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz | Tel: Email: | yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the results VTM-5.0 for sequences of YUV4:4:4 format, with and without DualITree enabled. The results for dual tree disabled show -6.19% AI, -2.06% RA, and -0.31% LB BD-Rate in TGM 1080p sequences, and -2.55% AI, -1.01% RA, and -0.16% LB BD-Rate overall for luma, with 160%, 108% and 100% in encoding time, and 93%, 100% and 100% in decoding time under AI, RA, and LD configurations, respectively.

# Simulation Results

Table 1 shows the results with DualITree enabled and disabled on test sequences 4:4:4. The results are evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The anchor is DualITree turned on while the test is DualITree turned off.

Table 1: Results of DualITree turned on and off on sequences 4:4:4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.19% | -7.63% | -7.87% | 151% | 91% |
| TGM 720p | -2.88% | -3.39% | -2.86% | 159% | 93% |
| Animation | -0.04% | -1.51% | -2.30% | 167% | 94% |
| Mixed content | -1.70% | -1.53% | -1.77% | 153% | 94% |
| Camera-Captured content | 1.63% | 2.49% | 2.76% | 190% | 97% |
| Overall | -2.55% | -3.12% | -3.15% | 160% | 93% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.06% | -2.83% | -2.85% | 105% | 101% |
| TGM 720p | -1.82% | -3.01% | -2.84% | 112% | 100% |
| Animation | 0.25% | -1.51% | -1.85% | 107% | 100% |
| Mixed content | -1.12% | -1.56% | -1.56% | 106% | 99% |
| Camera-Captured content | 1.59% | 1.61% | 2.05% | 108% | 102% |
| Overall | -1.01% | -1.85% | -1.80% | 108% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0, CTC, DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.31% | -0.73% | -0.66% | 100% | 100% |
| TGM 720p | -0.34% | -0.76% | -0.86% | 99% | 101% |
| Animation | 0.01% | 0.60% | 0.24% | 102% | 97% |
| Mixed content | -0.06% | -0.24% | -0.24% | 101% | 99% |
| Camera-Captured content | 0.17% | 0.24% | 0.24% | 101% | 101% |
| Overall | -0.16% | -0.33% | -0.39% | 100% | 100% |

# Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).