| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O00124-v2 |
| --- | --- |

| Title: | CE11/AHG14: Test 3.1 - an approach for support of GRA/GDR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Jianle Chen Seungwook Hong | Email: | fhendry@futurewei.com ye-kui.wang@futurewei.com jianle.chen@futurewei.com seungwook.hong@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

This contribution describes test results for CE 11 test 3.1. The test is designed to provide implementation for support of gradual random access (GRA) / gradual decoding refresh (GDR) based on using coded region method in which the refreshed region and the unrefreshed region are coded in different slices. It is asserted that other than the need to treat the refreshed region boundaries in reference pictures as picture boundaries in certain cases, the approach implemented in test 3.1 relies only on high-level syntax features.

In v2 of this contribution, more simulation results are reported and the specification text is included as an attachment.

# Introduction

At the 14th JVET meeting in Geneva, it was agreed to normatively support gradual random access (GRA) / gradual decoding refresh (GDR), and a new NAL unit type has been defined, together with signalling of the recovery point in the slice header, while whether to specify any other normative feature for GRA support has been left for study in CE 11.

This contribution describes test results for CE 11 test 3.1 that as originally proposed in JVET-N0115. The proposed method can be summarized as follows (only the parts not yet included in the VVC draft specification are mentioned below):

- Progressive intra refresh for GRA is performed based on intra coding of entire slices.

  - A GRA picture and the pictures that follow the GRA picture in decoding order and before the recovery point are coded with at least two slices: at least one slice for the refreshed region, and at least one slice for the unrefreshed region.

  - A GRA picture contains at least one slice with refreshed_region_flag equal to 1 that is intra coded.

  - Only pictures within the GRA period starting from the recovery point may have only one slice (they can certainly also have multiple slices).

- A flag called refreshed_region_flag to specify whether a slice is part of the refreshed region or not is signalled in the slice header. All slices having refreshed_region_flag equal to 1 shall cover a connected region.

- The boundaries of the refreshed region in a picture are derived based on the values of the refreshed_region_flag in the slices of the picture.

- For encoding/decoding of a slice in the refreshed region (referred to as a refreshed slice) of a GRA period, the following applies:

  - The slice does not refer to reference pictures that belong to a different GRA period (i.e., does not refer to reference picture that precede the GRA picture in decoding order).

  - Coding blocks within the slice can only refer to reference blocks that belong to the refreshed region in the reference picture. For this, the boundaries of the refreshed region in the reference picture are treated as picture boundaries.

- For encoding/decoding of a slice in the unrefreshed region (referred to as an unrefreshed slice) of a GRA period, the following applies:

  - The slice may refer to reference pictures that belong to same GRA period and/or reference pictures that belong to a different GRA period.

  - When a reference picture belongs to a different GRA period, coding blocks within the slice can only refer to reference blocks that belong to the refreshed region in the reference picture. For this, the boundaries of the refreshed region in the reference picture are treated as picture boundaries.

  - When a reference picture belongs to the same GRA period, there is no special treatment needed.

It is asserted that other than the need to treat the refreshed region boundaries as picture boundaries, the approach implemented in test 3.1 relies only on high-level syntax features.

The text changes for the proposed method are provided in the attachment of this contribution, based on JVET-N1001-v8.

# Handling of refreshed and unrefreshed regions

As described above, coding blocks within a refreshed slice can only refer to reference blocks that belong to the refreshed region in a reference picture that belongs to the same GRA period. Coding blocks within an unrefreshed slice can refer to reference blocks that belong to the refreshed region in a reference picture that belongs to a different GRA period or any reference blocks in a reference picture that belongs to the same GRA period. Figure 1 illustrates the constraints for the refreshed and unrefreshed regions for supporting GRA.

Figure 1 – Coding block referencing for refreshed and unrefreshed regions.

To apply the above constraints, there are two options that can be used. The first option is to apply constraints on the encoder such that it performs motion search only within the given region (i.e., the refreshed region) and to ensure that no sample value outside the given region is used for prediction. The second option is to treat the boundaries of the refreshed region in a reference picture as picture boundaries. However, simulation results reported by the previous CE12 showed that the second option provides better coding performance.

In the VTM reference software implementation, treating boundaries of a region (e.g., tiles or slices) as picture boundaries can be implemented by applying padding to the area in the picture that is outside the given region as illustrated in Figure 2. Note that padding surrounding the picture boundaries has been implemented in the current VTM software, not part of the implementation of the proposed method.

Figure 2 – Padding of the unrefreshed region and picture surrounding.

# Simulation results

## BD-rate performance

Simulation for CE11-3.1 is conducted with test condition as agreed in JVET-N1031 with modification that ALF and LMCS are turned off. ALF is turned off for this test because the implementation of loop filter across tile boundary enabling / disabling is not implemented yet for ALF. LMCS is turned off for this test because in the current implementation, LMCS model parameter is updated at I-slice when encoder uses random access configuration and at fixed location when encoder uses low delay configuration. In test 3.1 implementation, starting from GRA picture until recovery point picture, each picture may contain one I-slice which makes LMCS treated the bitstream like in random access configuration while the test is conducated using low delay configuration.

Table 1 tabulates the performance of CE11-3.1 against the modified anchor. For this simulation, CTU size for anchor and test bitstreams is as defined by CTC.

Table 1 Performance of test 3.1 against anchor with ALF and LMCS off and with CTU size 128.

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | 2.17% | 3.90% | 4.79% |
| Class E | 4.18% | 12.38% | 16.79% |
| Overall | #VALUE! | #VALUE! | #VALUE! |
| Class D | 1.97% | 4.95% | 5.76% |
| Class F | #VALUE! | #VALUE! | #VALUE! |

For additional comparison, we also compare test 3.1 result with anchor without turning off LMCS. Table 2 tabulates such comparison

Table 2 Performance of test 3.1 against anchor with ALF off and with CTU size 128.

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | 2.17% | 3.90% | 4.79% |
| Class E | 4.18% | 12.38% | 16.79% |
| Overall | #VALUE! | #VALUE! | #VALUE! |
| Class D | 1.97% | 4.95% | 5.76% |
| Class F | #VALUE! | #VALUE! | #VALUE! |

## Bitrate fluctuation

Performance of GRA / GRD for reducing end-to-end delay is related to the bitrate fluctuation since the end-to-end delay depends on the peak of bitrate of picture unit or decoding unit.

For the approach tested in test 3.1, it is preferred to have tile columns as many as the number of GRA period. For example, if GRA period is 50, then ideally there should be 50 tile columns in each picture so that refresh region can progressively move from left most of the picture to the right most at every picture starting from GRA picture to recovery point picture.

For the agreed simulation, the GRA period is set to be the same as the frame rate of the test sequence. For the test sequences that are agreed to be used for the simulation, setting GRA period to be the same as the frame rate of the test sequence introduced two problems, they are:

- When CTU size is set equal to 128 for encoding, for test sequence that are agreed to be used as input for the simulation, all of them would have numbers of CTU columns along the picture width far below the given GRA period. One way to deal with this situation is that the refresh region may progress at every picture. In other words, instead moving the intra-coded slice (i.e., a tile column) at every picture for increasing the clean area, the intra-coded slice is present at certain interval such as once every x pictures.

- Having only a few number of CTU columns causes the proportion of area covered by intra-coded slice relatively high when compared to the whole area of the picture which easily causes bitrate peak when the picture is encoded.

To deal with the two issues above, we ran additional simulation in which CTU size is reduced for smaller resolutions. The tested CTU sizes for teh class of test sequences are as follow:

- CTU size 128 for class B sequences

- CTU size 64 for class C, E, and F sequences

- CTU size 32 for class D sequences

Table 3 tabulates the performance of CE11-3.1 with the above CTU size setting.

Table 3 Performance of test 3.1 against anchor with ALF and LMCS off and variable CTU size.

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | 3.10% | 6.72% | 8.88% |
| Class E | 8.49% | 18.26% | 25.36% |
| Overall | #VALUE! | #VALUE! | #VALUE! |
| Class D | 4.36% | 8.73% | 10.09% |
| Class F | #VALUE! | #VALUE! | #VALUE! |

Figure 3 and Figure 4 shows bitrate fluctuation for test 3.1 and anchor for Basketball sequence (i.e., class C) at QP 22 (blue line on the chart), 27 (orange line on the chart), 32 (gray line on the chart), and 37 (yellow line on the chart). Bitrate fluctuation charts for other test sequences are available in the attached spreadsheets.

Figure 3 – Test 3.1 birate fluctuation for QP 22, 27, 32, and 37 for BasketballDrill (Class C)

Figure 4 – Anchor birate fluctuation for QP 22, 27, 32, and 37 for BasketballDrill (Class C)

## Simulating random access from GRA picture

To test correctness of the implementation of approach for test 3.1, decoding for simulating random access from GRA picture is tested. For this test, when decoder receives the target GRA picture, the pels values of all reference pictures available in the DPB are reset to value 0. It is asserted that mismatch-free decoding was achieved for random access from any GRA picture at the recovery point picture associated with the GRA picture.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).