| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0127-v2 |
| --- | --- |

| Title: | CE8-related: IBC modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This proposal is a further improvement over JVET-N0201/N0316 that reduces the complexity of IBC block vector validation check for decoder. While using same amount of additional buffer as VTM5.0, worst-case complexity of IBC vector validation check is greatly reduced.

Simulation results of 2 tests are reported, the 1st test uses proposed method for reconstruction process of IBC block. The 2nd test applies proposed method to block vector search in encoding process on top of first test. Average results of luma BD rate difference for CTC, classF, and TGM compared with VTM5.0 (IBC enabled) are as below:

Test1 (proposed method used for recon process): 		All Intra:  0.00%, 0.00%, 0.00%

RA:	   0.00%, 0.00%, 0.00%

LDB:     0.00%, 0.00%, 0,00%

Test2 (test 1+ proposed method applied to block vector search): 	All Intra:  -0.01%, -0.03%, -0.18%

RA:	   0.00%, -0.02%, -0.18%

LDB:     0.01%, 0.01%, -0.02%

# Introduction

In VTM5.0, conformance check is performed that requires IBC block vector to be within a valid reference area as illustrated in Figure 1 & 2 (JVET-M0407).

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

Curr

X

Figure 1: current CTU processing order and its available reference samples in current and left CTU (when quad-tree split is enforced at 128x128 level)

X

X

Curr

X

X

X

Curr

Figure 2: current CTU processing order and its available reference samples in current and left CTU when first level vertical binary-tree split is applied.

JVET-N0201/N0316 introduces the same validation checks to decoder side to avoid undefined behavior of invalid referencing. If a block vector is determined as invalid, a fallback mode is triggered that set prediction samples to a default value. However, concerns were raised that those checks are computational expensive for hardware; In JVET-N0201/N0306, an IBC block vector is determined as invalid if any of the following conditions are met.

- If any reference sample exceed picture boundary

- If any reference sample’s horizontal and vertical position is greater than or equal to top left sample position of current block area

- If any reference sample is in above CTU row

- If any reference sample is in below CTU row

- If any reference sample exceeds current CTU boundary or left N’s (N =1 for 128x128 sized CTU) CTU boundary

- If any reference sample exceeds 3 VPDU reference area

- If any reference sample is not available when reference area is in current VPDU

It is desirable to reduce complexity based on above analysis, therefore following method is proposed:

A dedicated 128x128 sized buffer is used for decoding current VPDU and storing N (N=3 for CTU size >=64x64) previously decoded VPDU. And IBC reference samples are predicted from this buffer. The reconstructed samples before in-loop filtering are stored in the buffer as prediction samples for IBC referencing.

Size of IBC reference buffer is determined as below:

Luma:

IBCBufferWidth[0] = 128*128/maxCUHeight;							  (1)

IBCBufferHeight[0] = maxCUHeight; 							 		  (2)

Chroma (Cb/Cr):

IBCBufferWidth[1] = IBCBufferWidth/2; 								  (3)

IBCBufferHeight[1] = IBCBufferHeight/2; 								  (4)

Examples of buffer updating process are given in Figure 3, 4, 5.

Figure 3: Example of buffer updating process when quad-tree split is enforced at 128x128 level

Figure 4: Example of IBC reference buffer updating process when first level vertical binary-tree split is applied (starting from VPDU 4)

Figure 5: Example of buffer updating process for CTU size = 64x64

Position of the reference sample (refPos_x, refPos_y) in IBC buffer for a sample located in (currPos_x, currPos_y) with block vector (bv_x, bv_y) is derived in equation (5) (6) :

refPos_x = (currPos_x + bv_x) & (IBCBufferWidth - 1)						    (5)

refPos_y = (currPos_y + bv_y) & (IBCBufferHeight -1)						    (6)

wherein IBCBufferWidth = IBCBufferWidth[isChroma] and IBCBufferHeight = IBCBufferHeight[isChroma]

In the proposed method, an IBC block vector is determined as invalid if any of the following conditions are met.

- If the refPosTL_y + IBC block height > IBCBufferHeight.

- If any reference sample is not decoded in reference block.

The first condition judges if the reference area exceeds bottom boundary of IBC buffer wherein refPosTL_y stands for vertical position of top left pixel of reference area, if yes, the block vector is determined as invalid. However, if the reference area exceeds right boundary, the block vector is still valid and the reference area is wraparound, i.e., copy from two locations in IBC buffer. It is noted that reference area will never exceed left and top boundary due to the way position of reference area is determined.

The second condition judges if all reference samples in the IBC buffer have been decoded or not, if yes, the block vector is judged as valid, otherwise the block vector is judged as invalid. To determine if any reference sample is available or not, a buffer ‘isDecoded’ is used that stores the decoding status of each 4x4 (2x2 for chroma) sub-block in IBC reference buffer. Size of the buffer is 32*32*2 = 2048bits (256 bytes). Example of ‘isDecoded’ buffer status for is given in Figure 6.

Figure 6: Example of ‘isDecoded’ Buffer status (luma).

If an IBC block vector is determined as invalid, the same process as JVET-N0201/N0316 is applied. i.e., set prediction sample to a default value. Otherwise normal IBC copy is performed using prediction samples from IBC reference buffer.

Overall process of proposed method is summarized in pseudo code as below:

- Set values in ‘isDecoded’ buffer to 0 at beginning of each CTU row

- Once a VPDU located in (x, y) starts decoding process, assign buffer from IBC reference buffer to the VPDU

- While Decoding a VPDU

  - When start decoding a VPDU, set corresponding ‘isDecode’ buffer to 0

  - If current block is in IBC mode, go to step 3, otherwise go to step 5

  - Use equation (5) and (6) to calculate vertical position of bottom right pixel in IBC buffer as refPosBR_y, if refPosBR_y >= buffer height, set prediction sample value to a default value and go to step 5, otherwise, go to step 4

  - For each NxN sublock (N=4 for luma block, 2 for chroma block for 420 input sequences) in the reference block, determine 4 corner positions of the subblock as (subRefPosTL_x, subRefPosTL_y), (subRefPosTR_x, subRefPosTR_y), (subRefPosBL_x, subRefPosBL_y) and (subRefPosBR_x, subRefPosBR_y)

If any of the values of isdecode(subRefPosTL_x>>shift, subRefPosTL_y>>shift), isdecode(subRefPosTR_x, subRefPosTR_y), isdecode(subRefPosBL_x, subRefPosBL_y) and isdecode(subRefPosBR_x>>shift, subRefPosBR_y>>shift) is 0, set reference sample value to a default value, otherwise, use normal IBC copy. (shift = 2 for luma block and 1 for chroma block), go to step 5

  - Perform reconstruction using prediction samples and stored the reconstructed samples before in-loop filtering in IBC reference buffer

A comparison of worst case complexity of JVET-N0201/N0316 is given in table 1. Detailed analysis of complexity in terms of number of operations is given in section 6.

Table 1: Comparison of complexity of JVET-N0201/N0316 and proposed method for a 4x4 luma block

| JVET-N0201/N0316 | Proposed method |
| --- | --- |
| 17 comparison 24 addition 3 multiplication | 5 comparison 11 addition 6 bitwise AND |

# Experimental results

There are 2 tests in total. The CTC (common test conditions) defined in [1] was followed.

The first test uses proposed method for reconstruction process of IBC block. Block vector search of the encoding process is not changed. Since the proposed method utilizes ‘less strict’ constraints compared with JVET-N0201/N0316 and VTM5.0 conformance check, the simulation result is same as VTM5.0 anchor.

On top of first test, the second test applies proposed method to block vector search in encoding process, there is slight coding gain (-0.18% gain in AI and RA for TGM sequences) compared with VTM5.0 anchor.

Table 2: Results for Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 95% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 105% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 108% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 100% |
| TGM | 0.00% | 0.00% | 0.00% | 99% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 105% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 98% |
| TGM | 0.00% | 0.00% | 0.00% | 97% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 104% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 103% |
| TGM | 0.00% | 0.00% | 0.00% | 104% | 103% |

Table 3: Results for Test2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.01% | 100% | 102% |
| Class A2 | 0.01% | -0.04% | 0.02% | 107% | 103% |
| Class B | 0.00% | 0.03% | -0.01% | 104% | 110% |
| Class C | -0.01% | -0.01% | 0.02% | 101% | 104% |
| Class E | -0.01% | -0.01% | 0.06% | 104% | 100% |
| Overall | -0.01% | 0.00% | 0.01% | 103% | 104% |
| Class D | -0.02% | -0.07% | 0.03% | 96% | 100% |
| Class F | -0.03% | 0.07% | 0.03% | 101% | 100% |
| TGM | -0.18% | -0.11% | -0.12% | 101% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.07% | 98% | 100% |
| Class A2 | 0.00% | 0.26% | 0.02% | 100% | 101% |
| Class B | -0.03% | 0.10% | 0.00% | 102% | 102% |
| Class C | 0.02% | -0.01% | -0.08% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.09% | 0.00% | 101% | 102% |
| Class D | 0.02% | -0.16% | -0.17% | 102% | 105% |
| Class F | -0.02% | -0.09% | 0.16% | 98% | 102% |
| TGM | -0.18% | -0.13% | -0.10% | 98% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.11% | 0.11% | 101% | 100% |
| Class C | 0.01% | 0.29% | 0.17% | 104% | 100% |
| Class E | 0.08% | 1.22% | 0.26% | 98% | 103% |
| Overall | 0.01% | 0.36% | 0.17% | 101% | 101% |
| Class D | 0.05% | -0.02% | 0.12% | 103% | 100% |
| Class F | 0.01% | -0.18% | 0.46% | 94% | 91% |
| TGM | -0.02% | -0.12% | -0.09% | 98% | 101% |

# Discussions

This contribution further reduces complexity of JVET-N0201/N0316 by introducing dedicated IBC reference buffer. Complexity is greatly reduced. No coding performance difference is found in test 1 and encoder and decoding time impact is not reliable.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Complexity analysis

Complexity analysis of N0201/N0316 for a 4x4 Luma block:

- If any reference sample exceed picture boundary

This step consists of following operations:

Calculating horizontal and vertical position of bottom right pixel of reference block:

int refRightX = xPos + xBv + width - 1;

int refBottomY = yPos + yBv + height - 1;

judging if top left pixel of reference block exceed left picture boundary:

if ((xPos + xBv) < 0)

{

return false;

}

judging if top left pixel of reference block exceed right picture boundary:

if (refRightX >= picWidth)

{

return false;

}

judging if bottom right pixel of reference block exceed top picture boundary:

if ((yPos + yBv) < 0)

{

return false;

}

judging if bottom right pixel of reference block exceed bottom picture boundary:

if (refBottomY >= picHeight)

{

return false;

}

There are in total 4 comparison, 6 addition operations

- If any reference sample’s horizontal and vertical position is greater than or equal to top left sample position of current block area

This step consists of following operations:

Calculating horizontal and vertical position of bottom right pixel of reference block:

if ((xBv + width) > 0 && (yBv + height) > 0)

{

return false;

}

There are in total 2 comparison, 2 addition operations

- If any reference sample is in above CTU row

This step consists of following operations:

if (refTopY >> ctuSizeLog2 < yPos >> ctuSizeLog2)

return false;

There are in total 1 comparison operation

- If any reference sample is in below CTU row

This step consists of following operations:

if (refBottomY >> ctuSizeLog2 > yPos >> ctuSizeLog2)

{

return false;

}

There are in total 1 comparison operation

- If any reference sample exceeds current CTU boundary or left N’s (N =1 for 128x128 sized CTU) CTU boundary

This step consists of following operations:

int numLeftCTUs = (1 << ((7 - ctuSizeLog2) << 1)) - ((ctuSizeLog2 < 7) ? 1 : 0);

if ((refRightX >> ctuSizeLog2 <= xPos >> ctuSizeLog2) && (refLeftX >> ctuSizeLog2 >= (xPos >> ctuSizeLog2) - numLeftCTUs))

There are in total 3 comparison, 3 addition operations

- If any reference sample exceeds 3 VPDU reference area

This step consists of following operations:

if (((refLeftX >> ctuSizeLog2) == ((xPos >> ctuSizeLog2) - 1)) && (ctuSizeLog2 == 7))

{

// ref block's collocated block in current CTU

const Position refPosCol = pu.Y().topLeft().offset(xBv + ctuSize, yBv);

int offset64x = (refPosCol.x >> (ctuSizeLog2 - 1)) << (ctuSizeLog2 - 1);

int offset64y = (refPosCol.y >> (ctuSizeLog2 - 1)) << (ctuSizeLog2 - 1);

const Position refPosCol64x64 = {offset64x, offset64y};

if (pu.cs->isDecomp(refPosCol64x64, toChannelType(COMPONENT_Y)))

return false;

#if JVET_N0383_N0251_IBC_COL_VPDU_REMOVE

if (refPosCol64x64 == pu.Y().topLeft())

return false;

#endif

}

There are in total 4 comparison, 6 + 1 (for calculating address of the position in isDecomp)+ 2(offset function) addition operation, 1 multiplication (for calculating address of the position in isDecomp)

- If any reference sample is not available when reference area is in current VPDU

This step consists of following operations:

const Position refPosLT = pu.Y().topLeft().offset(xBv, yBv);

const Position refPosBR = pu.Y().bottomRight().offset(xBv, yBv);

const ChannelType      chType = toChannelType(COMPONENT_Y);

if (!pu.cs->isDecomp(refPosBR, chType))

return false;

if (!pu.cs->isDecomp(refPosLT, chType))

return false;

There are in total 2 comparison, 4 addition (offset function) and 2 multiplications(for calculating address of the position in isDecomp) operations

Complexity analysis of proposed method for a 4x4 Luma block:

- If the refPosTL_y + IBC block height > buffer height.

This step consists of following operations:

Use equation (6) to calculate vertical position of top left pixel of reference area:

refPos_y = (currPos_y + bv_y)& (IBCBufferHeight -1)

judging if condition 1 is satisfied requires following check:

refPos_y + IBC block height > buffer height

There are in total 1 comparison, 3 addition and 1 bitwise AND operations

- If any reference sample is not decoded in reference block.

This step consists of following operations:

Use equation (5) to calculate horizontal position of top left pixel of reference area:

IBCBufferWidthMinus1 = IBCBufferWidth – 1

refPos_x = (currPos_x + bv_x)& IBCBufferWidthMinus1

judging if condition 2 is satisfied requires calculating position of four corner pixels of a 4x4 sub-block:

Positions of 4 corners are calculated as:

x_right = x + 3

y_bottom = y + 3

if(  isdecoded(  (( x & IBCBufferWidthMinus1) >> 2), ( y >> 2) )

&& isdecoded(  (( x & IBCBufferWidthMinus1) >> 2), (y_bottom >> 2) )

&& isdecoded(  ((x_right & IBCBufferWidthMinus1) >> 2), ( y >> 2) )

&& isdecoded(  ((x_right & IBCBufferWidthMinus1) >> 2), (y_bottom >> 2) ))

There are in total 4 comparison, 4+ 4(for calculating address of the position in isDecoded) addition and 5 bitwise AND operations