| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0128-v1 |
| --- | --- |

| Title: | Non-CE4: Simplification of triangle mode MV context storage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a response to AHG16 comment on complexity of MV context storage of triangle mode. Two methods are proposed. The first method is code optimization that moves CU level MV index mapping to slice level by using mapping list table. The second method is further simplification of MV index mapping process. The first method has no impact on coding efficiency. The second method has coding impact of 0.01% luma BD-rate in RA and 0.00% luma BD-rate in LDB over VTM-5.0 anchor.

# Introduction

AHG16 reported following observation on triangle mode context storage:

The MV context storage of triangle merge mode (in which the triangle MVs are populated to create 4x4 block vectors that are stored as TMVPs and used as spatial MV neighbors of merge/AMVP modes) seems to be unnecessarily complicated, it should be simplified.

For triangle mode CU MV context storage process of VTM5.0, one MV candidate is stored for 4x4 sized sub-blocks not in weighted area and both MV candidates are stored for 4x4 sized sub-blocks in weighted area. For MV storage for sub-blocks in weighted area, if two MV candidates refer to different reference list, a Bi-MV consisting of the two MV candidates is storage in the sub-block. However, if two MV candidates refer to same reference list, a mapping process is needed to generate a Bi-MV from the two MV candidates. Details of the mapping process are as below:

Given the first MV candidate refers to reference list LX (X=0/1),

Search for a reference index i in reference list L(1-X) that refers the same reference picture as the second MV candidate, note that the second MV candidate also refers to reference list LX.

If the reference index i is found, generate a Bi-MV consisting of the first MV candidate and a MV with reference index = i, reference list = L(1-X) while the x and y components equal to that of the second MV candidate. The generated Bi-MV is stored in the sub-block.

Otherwise search for a reference index j in reference list L(1-X) that points to the same reference picture as the first MV candidate.

If the reference index j is found, generate a Bi-MV consisting of the second MV candidate and a MV with reference index = j, reference list = L(1-X) while the x and y components equal to that of the first MV candidate. The generated Bi-MV is stored in the sub-block.

Otherwise store only the first MV candidate in the sub-block.

The mapping process is the most complex part in MV context storage process of triangle mode CU. A straightforward solution is to store only one MV in the weighted area (e.g., store only the first MV candidate). However, this approach causes coding loss as shown in Table 1 (Enc/Dec time is not reliable):

Table 1: simulation results by storing only one MV in weighted area

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.04% | 100% | 99% |
| Class A2 | 0.07% | 0.05% | 0.14% | 97% | 97% |
| Class B | 0.06% | 0.11% | -0.02% | 98% | 100% |
| Class C | 0.04% | -0.05% | -0.14% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.02% | -0.01% | 99% | 98% |
| Class D | 0.07% | 0.21% | 0.23% | 96% | 95% |
| Class F | 0.02% | 0.02% | -0.01% | 101% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | 0.34% | 0.26% | 105% | 104% |
| Class C | 0.21% | 0.18% | 0.36% | 103% | 103% |
| Class E | 0.34% | 0.23% | 0.44% | 103% | 102% |
| Overall | 0.22% | 0.26% | 0.34% | 104% | 103% |
| Class D | 0.20% | -0.29% | 0.52% | 102% | 102% |
| Class F | 0.10% | -0.20% | 0.06% | 101% | 101% |

This contribution proposes two solutions to address the complexity issue:

The first method uses two mapping lists that are constructed in slice level to replace searching process of reference index;

The 1st mapping list consists of N entries (N = number of reference index of reference list L0). A value j is assigned to ith entry (i = 0 … N-1) wherein j is the smallest index number in reference L1 so that reference index i in L0 and reference index j in L1 refer to the same reference picture. If there is no such reference index in L1, j is set to -1.

Similarly, the 2nd mapping list consists of M entries (M = number of reference index of reference list L1). A value j is assigned to ith entry (i = 0 … M-1) wherein j is the smallest index number in reference L0 so that reference index i in L1 and reference index j in L0 refer to the same reference picture. If there is no such reference index in L0, j is set to -1.

An example of mapping lists is given in Table 2&3 as below:

Table 2: Example POC number of reference picture for reference list L0/1

| Ref index Ref list | Index 0 | Index 1 | Index 2 |
| --- | --- | --- | --- |
| L0 | 0 | 16 | 8 |
| L1 | 16 | 0 | 4 |

Table 3: Example of mapping lists given reference lists in Table 2

| list entry mapping list idx | Entry 0 | Entry 1 | Entry 2 |
| --- | --- | --- | --- |
| 0 | 1 | 0 | -1 |
| 1 | 1 | 0 | -1 |

When 2 MV candidates refers to the same reference list, MV storage for weighted area could use mapping list instead of searching process of reference index; therefore the mapping process is modified as:

Given the first MV candidate refers to reference index i of reference list LX (X=0/1) and the second MV candidate refers to reference index j of reference list LX (X=0/1)

Getting value k in entry i of mapping list X.

If k != -1, generate a Bi-MV consisting of the first MV candidate and a MV with reference index = k, reference list = L(1-X) while the x and y components equal to that of the second MV candidate. The generated Bi-MV is stored in the sub-block.

Otherwise getting value m in entry j of mapping list X.

If m != -1, generate a Bi-MV consisting of the second MV candidate and a MV with reference index = m, reference list = L(1-X) while the x and y components equal to that of the first MV candidate. The generated Bi-MV is stored in the sub-block.

Otherwise store only the first MV candidate in the sub-block.

The construction process of mapping list is implementation dependent. An example process is given in Table 4 & 5. In this example, 2 lists of reference index records are first initialized to -1 for reference list L0/1. Once a reference index is inserted for reference list LX, the list is updated with the index if the value recorded equals -1. The final recorded lists are used to construct mapping list table. Worst-case complexity of this example is 32 comparison operations.

Table 4: initialization of reference index record

| POC number Ref list | 0 | 4 | 8 | 16 |
| --- | --- | --- | --- | --- |
| L0 | -1 | -1 | -1 | -1 |
| L1 | -1 | -1 | -1 | -1 |

Table 5: reference index record after constructing reference list

| POC number Ref list | 0 | 4 | 8 | 16 |
| --- | --- | --- | --- | --- |
| L0 | 0 | -1 | 2 | 1 |
| L1 | 1 | 2 | -1 | 0 |

As the first solution can be considered as coding optimization of VTM5.0, there is no coding performance impact.

The second method is a further simplification of reference index mapping process. A flag isRefListIdentical that indicates if reference list L0 and L1 are identical is used. The mapping process is modified as below:

Given the second MV candidate refers to reference index i of reference list LX (X=0/1)

If isRefListIdentical is true, generate a Bi-MV consisting of the first MV candidate and a MV with reference index = i, reference list = L(1-X) while the x and y components equal to that of the second MV candidate. The generated Bi-MV is stored in the sub-block.

Otherwise store only the first MV candidate in the sub-block.

The flag isRefListIdentical is determined slice level, this flag is judged as true only if all conditions below are met:

- Number of reference index in reference list L0 and L1 are the same

- For each reference index i, reference picture referred by index i in reference list L0 and L1 are the same

Worst-case complexity of determining isRefListIdentical is 16 comparison operations.

Coding performance is given in section 2.

# Experimental results

Coding performance of the second solution is reported as below. The CTC (common test conditions) defined in [1] was followed. Note that enc/dec time is not reliable.

CTC Coding impact for both RA and LDB are negligible.

Table 6: Simulation Results of second solution

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.11% | -0.09% | 102% | 96% |
| Class A2 | 0.01% | 0.02% | 0.01% | 101% | 97% |
| Class B | 0.02% | 0.02% | 0.00% | 101% | 98% |
| Class C | -0.02% | -0.01% | -0.07% | 101% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.03% | 101% | 97% |
| Class D | -0.01% | -0.02% | 0.05% | 97% | 93% |
| Class F | 0.00% | 0.05% | 0.01% | 104% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 106% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 103% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 95% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 98% |

# Discussions

This contribution is a response to complexity issue of triangle MV context storage process raised in AHG16. It provides 2 solutions. The first solution can be considered as code optimization therefore there is no coding impact; the second solution is a further simplification with negligible coding impact. Encoding and decoding time impact is not reliable.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text

#### Reference picture mapping process for triangle merge mode

Input to this process are:

- a variable X representing a reference list being equal to 0 or 1,

- a reference index refIdxN.

Output of this process is:

- a reference index refIdxTemp.

refIdxTemp = isRefListIdentical ? refIdxN : -1

the variable isRefListIdentical is derived as follows:

The variable isRefListIdentical is set equal to TRUE:

If NumRefIdxActive[0] != NumRefIdxActive[1], isRefListIdentical is set to FALSE; Otherwise:

For the variable refIdxm with m = 0…NumRefIdxActive[0]-1, the following applied until isRefListIdentical is set to FALSE:

isRefListIdentical = RefPicList[0][m] == RefPicList[1][m] ? TRUE : FALSE

-

-

-

-