| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0132-v2 |
| --- | --- |

| Title: | CE10-2.10/CE10-2.11: Evaluation results of CNN-based filtering with off-line learning model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution reports the results by the convolutional neural network-based filtering (CNNF) with off-line learning model which was proposed in JVET-N0710. Simulation results reportedly show the overall BD-rates for luma by -1.00%, -1.16%, -1.10% and -0.32% in CE10-2.10a, CE10-2.10b, CE10-2.11a and CE10-2.11b, respectively. These differences are caused by bit-depth of CNNF parameter and the difference between in-loop and post processing filter.

Introduction

A convolutional neural network-based filtering (CNNF) with off-line learning model was proposed in JVET-N0710 [1]. This contribution reports the experimental results followed by the test conditions of Core Experiment 10 (CE10) description [2]. One of the CE10 purpose is to study the impact of CNNF parameter. Another purpose is to study the difference between in-loop and post processing filter. It is noted that experiments are performed only the first intra period (without second intra picture) to reduce the experimental burden for the bit-depth investigation.

Overview of the proposed CNNF

The filter structure is shown in Figure 1. This filter has four layers with 3x3 taps. The inputs of “sum” block are the residual signal from the left side of “sum” block, and the reconstructed signal just after deblocking filter from the bottom side of the “sum” block. The output of “sum” block is filtered pixels. Actual output is sum of the reconstructed pixels, and the weighted residual pixels. The weight is derived from the distance from block boundary and the adjacent blocks’ type like intra or inter coding as same as JVET-N0712 [3].

Figure1. The structure of proposed CNNF

Filter coefficients are pre-defined in both the encoder and the decoder so that additional side information is not required. A representation of filter coefficients is fixed point value (integer value). The filter is applied for both intra and inter blocks in intra and inter pictures.

Experimental Results

Test Condition

The proposed methods have been implemented on top of VTM-5.0 [4]. The common test conditions are used for evaluation [5] but the simulation results for CE10-2.10a/b are conducted in only Random Access 10 condition by the short frame which is defined as first intra period in JVET-N1030 [2].

The test conditions consist total of 4 items as described in Table 1. CE10-2.10a/b is the evaluation for the impact of CNNF bit-depth. CE10-2.11a/b is the evaluation for the difference between in-loop and post processing filter. It is noted that the CNNF is adaptively switching on/off in picture/CTU/32x32 block level in CE10-2.10a/b and CE10-2.11b (in-loop filtering), though CNNF is applied always in CE10-2.11b (post filtering). It is also noted that the bit-depth in CNNF parameter of CE10-2.11a/b is same as CE10-2.10b.

Table 1 Test conditions according to CE13 description

| CE10-2.10a | bit-depth | 8-bit (int) weights |
| --- | --- | --- |
| CE10-2.10b |  | 16-bit (int) weights |
| CE10-2.11a | in-loop / post filtering | CNNF as in loop filter |
| CE10-2.11b |  | CNNF as post filter |

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Simulation results

It is noted that encoding runtime does not include the learning time in all simulation results.

    - CE10-2.10a (8bit, in-loop filtering, 1rap)

Table 2 shows the simulation results of CE10-2.10a.

Table 2 The simulation results of CE10-2.10a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.62% | -2.43% | -2.41% | 137% | 78050% |
| Class A2 | -1.24% | -2.60% | -3.04% | 133% | 72488% |
| Class B | -0.79% | -2.51% | -2.40% | 132% | 56114% |
| Class C | -1.38% | -4.27% | -5.90% | 115% | 40342% |
| Class E |  |  |  |  |  |
| Overall | -1.00% | -2.98% | -3.46% | 128% | 57777% |
| Class D | -2.19% | -3.63% | -6.34% | 111% | 28146% |
| Class F | -0.34% | -1.45% | -1.74% | 143% | 29566% |

    - CE10-2.10b (16bit, in-loop filtering, 1rap)

Table 3 shows the simulation results of CE10-2.10b.

Table 3 The simulation results of CE10-2.10b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.84% | -3.61% | -2.59% | 137% | 78532% |
| Class A2 | -1.35% | -2.52% | -2.76% | 133% | 72366% |
| Class B | -0.92% | -2.41% | -2.64% | 133% | 57705% |
| Class C | -1.55% | -4.43% | -6.16% | 115% | 42192% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | -3.21% | -3.59% | 129% | 59072% |
| Class D | -2.36% | -4.07% | -6.87% | 112% | 30628% |
| Class F | -0.28% | -2.24% | -3.40% | 143% | 29769% |

    - CE10-2.11a (16bit, in-loop filtering, full-frame)

Table 4 shows the simulation results of CE10-2.11a.

Table 4 The simulation results of CE10-2.11a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -2.90% | -2.81% | 134% | 74843% |
| Class A2 | -1.41% | -2.39% | -2.88% | 132% | 71277% |
| Class B | -0.88% | -1.95% | -2.12% | 130% | 55776% |
| Class C | -1.51% | -4.63% | -5.63% | 115% | 39500% |
| Class E |  |  |  |  |  |
| Overall | -1.10% | -2.94% | -3.35% | 127% | 56667% |
| Class D | -1.95% | -3.78% | -5.93% | 110% | 22768% |
| Class F | -0.34% | -2.83% | -3.19% | 150% | 37332% |

    - CE10-2.11b (16bit, post-filtering, full-frame)

Table 5 shows the simulation results of CE10-2.11b.

Table 5 The simulation results of CE10-2.11b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.82% | -0.83% | 100% | 78382% |
| Class A2 | -0.28% | -0.52% | -0.58% | 100% | 72840% |
| Class B | -0.22% | -0.47% | -0.65% | 100% | 71033% |
| Class C | -0.59% | -0.88% | -1.16% | 100% | 45185% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.66% | -0.81% | 100% | 64536% |
| Class D | -0.80% | -0.84% | -1.36% | 100% | 34363% |
| Class F | 0.02% | -0.64% | -0.82% | 100% | 83713% |

Subjective Evaluation

From Figure 2 to Figure 4 show the subjective evaluations for original image, VTM5, CE10-2.11a (in-loop) and CE-10.2.11b (post) of Campfire (POC=58), RitualDance (POC=414) and RitualDance (POC=248), respectively. The images of VTM5, CE10-2.11a and CE10-2.11b are under RA10 and QP37 condition. As shown these figures, the basis like a checkerboard appears in original image, VTM5 and CE10-2.11b (post-filtering) whereas it does not appear in CE10-2.11a (inloop-filtering). When the basis size is large, it still remains even if CE10-2.11a if the basis size and is large. When the base size is large, even if it is CE10-2.11a, it seems that the basis still remains.

Figure 2 Subjective evaluations of Campfire (POC=58) in original vs. VTM5 vs. CE10-2.11a vs. CE10-2.11b

Figure 3 Subjective evaluations of RitualDance (POC=414) in original vs. VTM5 vs. CE10-2.11a vs. CE10-2.11b

Figure 4 Subjective evaluations of RitualDance (POC=248) in original vs. VTM5 vs. CE10-2.11a vs. CE10-2.11b

Conclusion

This contribution reported the experimental results followed by the test conditions of Core Experiment 10 (CE10) description [2]. Simulation results reportedly show the overall BD-rates for luma by -1.00%, -1.16%, -1.10% and -0.32% in CE10-2.10a, CE10-2.10b, CE10-2.11a and CE10-2.11b, respectively. The difference is caused by bit-depth of CNN-based filter parameter and the difference between in-loop and post processing filter. In addition, subjective evaluation shows some improvement of subjective quality for noise-cancelling in CE10-1.11a (inloop-filtering) results.

References

- K. Kawamura, Y. Kidani, S. Naito, “CE13-2.6/CE13-2.7: Evaluation results of CNN based in-loop filtering”, JVET-M0710, Geneva, CH, March 2019.

- Y. Li, S. Liu, K. Kawamura, “Description of Core Experiment 10 (CE10): Neural Network based Filter for Video Coding”, JVET-N1030, Geneva, CH, March 2019.

- K. Kawamura, Y. Kidani, S. Naito, “CE13-related: Adaptive CNN based in-loop filtering with boundary weights”, JVET-M0712, Geneva, CH, March 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, JVET-N1002, Geneva, CH, March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).