| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0135-v1 |
| --- | --- |

| Title: | AHG8: Scalability for VVC - general |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry Jianle Chen 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com +1-858-375-8065 fhendry@futurewei.com +1-858-999-4243 jianle.chen@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

The following aspects are proposed in this contribution:

- Support spatial and SNR scalabilities in the 1st edition of VVC, using the same reference picture resampling (RPR) filtering process as for the support of the RPR feature.

- Not to specify separate profile(s) for the scalability support, and use an SEI message to indicate the bitstream being multi-layered with inter-layer prediction as well as other needed scalability information.

- Adopt a starting point of the syntax and semantics of the SEI message, named scalability information SEI message.

Note that handling of POC, RPL, and reference picture marking for VVC scalability support is discussed and proposed in JVET-O0136.

# Introduction

Scalable video coding does provide technical benefits and optimization spaces for various video applications, e.g., as shown in the publicly available 3GPP TR 26.904 for SVC and 3GPP TR 26.948 for SHVC. However, it is also known that so far there has not been much actual deployment of scalable coding by the industry. In our understating, this was mainly due to two reasons: 1) the benefits brought by scalable coding were often considered not sufficient to justify the costs compared to (simulcast of) single-layer coding; 2) the supports of SVC and SHVC were specified in separate profiles from the "main" single-layer profiles. Given that reference picture resampling (RPR) is very likely to be supported in the "main" profile of VVC, which will make the cost of additionally supporting scalable coding very low, we think that support of scalable coding in the "main" profile of VVC 1st edition will make VVC a better codec. We therefore, along with other proposals, e.g., in JVET-O0045, propose to support (spatial and SNR) scalability in the 1st edition of VVC.

With the existence of RPR, the main question of scalability support is high-level syntax (HLS). A key issue herein is the trade-off between how optimal the HLS design is and how much of the HLS stuff needs to be implemented by decoders designed only for decoding of single-layer bitstreams.

There are at least the following possible options going forward:

- Specify scalability support now, and DO NOT specify separate profile(s) just for scalability.

- Specify scalability support now, and specify separate profile(s) just for scalability.

- Not to specify scalability support now, and specify separate profile(s) just for scalability later on, similarly as SVC and SHVC, when needed/agreed.

Among the above, we think option 1 makes the most sense, given that no essential new work on low-level coding tools is really needed once RPR kicks in.

Focusing on option 1 and thinking about the trade-off of the key issue mentioned above, first of all, to largely mitigate the need of implementing scalability-specific HLS by decoders designed only for decoding of single-layer bitstreams, the only way we could think of is to treat scalability support like the support of frame-packing based stereo video and the support of omnidirectional/3600 video, i.e., using SEI messages to indicate the bitstream being frame-packed (for frame packing), ERP/CMP-projected (for omnidirectional/3600 video), and multi-layered with inter-layer prediction (for scalability), as well as the details needed. This way, decoders that are not designed for handling of bitstreams that are frame-packed, omnidirectional/3600 video, or multi-layered can simply always ignore the respective corresponding SEI messages.

If such an approach is taken, once the 1st edition of VVC is available, implementers interested in scalability can take advantage of it, and implementers not interested in scalability at all can just ignore the scalability information SEI message (or whatever other name of the SEI message that we’d like to call it). At the same time, this would still leave a possibility for specifying something like SHVC or SVC in the future when needed/agreed.

For such an approach to work, besides specifying the SEI message itself, there are a bit more HLS work needed, on POC, RPL, and reference picture marking (RPM). Approaches for handling of POC, RPL, and RPM for VVC scalability support are discussed and proposed in JVET-O0136.

# Proposal

## Summary

This contribution proposes to support (spatial and SNR) scalability in the 1st edition of VVC, provided that reference picture resampling (RPR) is supported, using the same RPR filtering process.

To largely mitigate the need of implementing scalability-specific high-level syntax by decoders designed only for decoding of single-layer bitstreams, instead of defining separate profile(s) for the scalability support, it is proposed to treat scalability support like the support of frame-packing based stereo video and the support of omnidirectional/3600 video, by using an SEI message to indicate the bitstream being multi-layered with inter-layer prediction as well as other needed scalability information.

A starting point of the syntax and semantics of the SEI message, named scalability information SEI message, is provided in the next subsection.

## The proposed scalability information SEI message

### Scalability information SEI message syntax

| scalability_info( payloadSize ) { | Descriptor |
| --- | --- |
| scalability_type | u(3) |
| num_layers_minus1 | ue(v) |
| for( i = 0; i <= num_layers_minus1; i++ ) |  |
| layer_id_delta[ i ] | ue(v) |
| for( i = 1; i <= num_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| num_output_layer_sets_minus1 | ue(v) |
| for( i = 1; i <= num_output_layer_sets_minus1; i++ ) { |  |
| num_layers_in_ols_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_layers_in_ols_minus1[ i ]; j++ ) { |  |
| layer_idx_delta_ols_minus1[ i ][ j ] | ue(v) |
| if( scalability_type != 0 ) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |

### Scalability information SEI message semantics

The scalability information SEI message provides scalability information for the applicable access unit set (AAUS) of the scalability information SEI message.

A scalability information SEI message may and may only be present in a CVSS access unit. The set of access units to which a scalability information SEI message applies consists of the access unit containing the scalability information SEI message and all succeeding access units in decoding order until, but excluding, the next access unit that contains a scalability information SEI message (if present) or the end of the bitstream (otherwise). This set of access units is referred to as the AAUS of the scalability information SEI message.

scalability_type indicates the type of scalabilities supported by the layers in the AAUS according to according to the following table:

Table – Interpretation of scalability_type

| scalability_type | Scalability type |
| --- | --- |
| 0 | Spatial and/or SNR scalabilities, where only the highest layer is output |
| 1 | Multiview, where each layer represents a distinct view, and the list of output layers is explicitliy signalled |

Values of scalabiltiy_type that are not listed in the table are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore scalability SEI messages that contain reserved values of scalabiltiy_type.

num_layers_minus1 plus 1 indicates the number of layers in the AAUS. The value of num_layers_minus1 shall be in the range of 0 to 126, inclusive.

layer_id_delta[ i ] is used to derive the layer ID of i-th layer in the AAUS. The value of layer_id_delta[ i ] shall be in the range of 0 to 126, inclusive.

The variable aausLayerId[ i ], indicating the layer ID of the i-th layer in the AAUS, is derived as follows:

if( i  = =  0 )
	aausLayerId[ i ] = layer_id_delta[ i ]
else
	aausLayerId[ i ] = aausLayerId[ i − 1 ] + layer_id_delta[ i ] + 1

direct_dependency_flag[ i ][ j ] equal to 0 indicates that the j-th layer is not a direct reference layer for the i-th layer. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the j-th layer is a direct reference layer for the i-th layer. A layer layerB is a direct reference layer of another layer layerA if and only if at least one picture of layer B is used in inter-layer prediction of a picture of layer A.

When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to num_layers_minus1, it is inferred to be equal to 0.

num_output_layer_sets_minus1 plus 1 indicates the number of OLSs in the AAUS. An OLS is a set of layers that does not depend on any other layers for decoding and for which one or more layers are indicated to be output layers. The value of num_output_layer_sets_minus1 shall be in the range of 0 to 1023, inclusive.

NOTE 1 – The 0-th OLS, or OLS 0, is inferred to contain only the lowest layer, and the lowest layer is inferred to be an output layer.

num_layers_in_ols_minus1[ i ] plus 1 indicates the number of layers in the i-th OLS in the AAUS. The value of num_layers_in_ols_minus1 shall be in the range of 0 to 126, inclusive. The value of num_layers_in_ols_minus1[ 0 ] is inferred to be equal to 0.

layer_idx_delta_ols[ i ][ j ] is used to derive the index of the j-th layer in the i-th OLS in the AAUS. The value of layer_idx_delta_ols[ i ] shall be in the range of 0 to 126, inclusive.

The variable olsLayerIdx[ i ][ j ], indicating the layer index of the j-th layer in the i-th OLS in the AAUS, is derived as follows:

if( i  = =  0 )
	olsLayerIdx[ 0 ][ 0 ] = 0
else {
	if( j  = =  0 )
		olsLayerIdx[ i ][ j ] = layer_idx_delta_ols[ i ][ j ]
	else
		olsLayerIdx[ i ][ j ] = olsLayerIdx[ i ][ j − 1 ] + layer_idx_delta_ols[ i ][ j ] + 1
}

output_layer_flag[ i ][ j ] equal to 1 indicates that the j-th layer of the i-th OLS in the AAUS is an output layer. output_layer_flag[ i ][ j ] equal to 0 indicates that the j-th layer of the i-th OLS in the AAUS is not an output layer. When an OLS is decoded, decoded pictures that are output come only from output layers, while no decoded pictures would be output from any layer that is not an output layer.

When not present, the value of output_layer_flag[ i ][ j ] is derived as follows:

- If scalability_type is not equal to 0, the value of output_layer_flag[ i ][ j ] is inferred to be equal to 0.

- Otherwise (scalability_type is equal to 0), the following applies:

  - If j is equal to num_layers_in_ols_minus1[ i ], the value of output_layer_flag[ i ][ j ] is inferred to be equal to 1.

  - Otherwise (j is less than num_layers_in_ols_minus1[ i ]), the value of output_layer_flag[ i ][ j ] is inferred to be equal to 0.

NOTE 2 – When scalabiltiy_type is equal to 0, the highest layer is inferred to be an output layer and all other layers in an OLS are inferred to be non-output layers.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).