| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0141-v1 |
| --- | --- |

| Title: | AHG12: Sub-picture based motion-constrained independent regions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry Jianle Chen 10180 Telesis Court San Diego, CA, 92121, USA Robert Skupin Yago Sanchez Karsten Suehring Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com +1-858-375-8065 fhendry@futurewei.com +1-858-999-4243 jianle.chen@futurewei.com {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Futurewei Technologies, Inc., Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a design of sub-picture-based coding for VVC. The proposal is a follow-up of JVET-N0826, but now based on the flexible tiling approach based on tiles and bricks that was adopted at the 14th JVET meeting.

The proposal is summarized as follows:

- Pictures may be divided into sub-pictures.

- The indication of existence of sub-pictures is indicated in the SPS, along with other sequence-level information of sub-pictures.

- Whether a sub-picture is treated as a picture in the decoding process (excluding in-loop filtering operations) can be controlled by the bitstream.

- Whether in-loop filtering across sub-picture boundaries is disabled can be controlled by the bitstream for each sub-picture. The DBF, SAO, and ALF processes are updated for controlling of in-loop filtering operations across sub-picture boundaries.

- For simplicity, as a starting point, the sub-picture width, height, horizontal offset, and vertical offset are signalled in units of luma samples in SPS. Sub-picture boundaries are constrained to be slice boundaries.

- Treating a sub-picture as a picture in the decoding process (excluding in-loop filtering operations) is specified by slightly updating the coding_tree_unit( ) syntax, and updates to the following decoding processes:

  - The derivation process for (advanced) temporal luma motion vector prediction

  - The luma sample bilinear interpolation process

  - The luma sample 8-tap interpolation filtering process

  - The chroma sample interpolation process

- Sub-picture IDs are explicitly specified in the SPS and included in the tile group headers to enable extraction of sub-picture sequences without the need of changing VCL NAL units.

- Output sub-picture sets (OSPS) are proposed to specify normative extraction and conformance points for sub-pictures and sets thereof.

Detailed specification text changes are provided in an attachment.

# Introduction

## Motivations for sub-picture-based coding

JVET-M0261 includes an approach for sub-picture-based coding, which can be summarized as follows:

- JVET-M0261 Option 3: Sub-picture-based partitioning and decoding. Each sub-picture may refer to its own PPS and may hence have its own tile partitioning. Sub-pictures are treated like pictures in the decoding process.

As can be seen from JVET-M0261, sub-picture-based coding enables the following optimizations:

- Since sub-pictures of the same picture may refer to different PPSs and hence different tile configurations, flexible tiling (i.e., tiles are not necessarily in tile rows and tile columns) is enabled, which is needed in some viewport-dependent 360° streaming schemes and is more efficient than MCTSs in terms of rate-distortion performance.

- Sub-picture-based decoding would be beneficial in responding to viewing orientation changes in some mixed-resolution viewport-dependent 360° streaming schemes, as described in JVET-M0259.

We believe that sub-picture-based coding with output sub-picture sets are a way that is "more normatively" than using MCTS related SEI messages as in HEVC which would further provide the following benefit:

- When "more normatively" specified than using SEI messages, conformances of both the entire bitstream and individual sub-picture bitstreams are expected to be specified. With this, systems and applications would be able to depend on these clearly specified conforming decoding behaviors and would worry less about these issues (like the conforming decoding behaviors of HEVC MCTSs) and spend less effort to define special systems tools to address these issues (like HEVC file format features such as tile tracks, each of which carries a non-conforming bitstream but the bitstream can be expected to be decoded and used applications, and such as extractor tracks, which may add lots more tracks and/or DASH representations that need to be managed by applications).

Provided that VVC now supports flexible tiling through tiles and bricks, there is no longer a need for different sub-pictures within a picture to refer to different PPSs, and consequently, the flag for controlling of in-loop filtering operations across sub-picture boundaries being disabled or not needs to moved out of the PPS (compared to the design in JVET-N0826). Otherwise, the design in this proposal is basically the same as in the design in JVET-N0826.

# Proposal

The proposed sub-picture-based coding is summarized as follows:

- Pictures may be divided into sub-pictures.

- The indication of existence of sub-pictures is indicated in the SPS, along with other sequence-level information of sub-pictures.

- Whether a sub-picture is treated like a picture in the decoding process excluding in-loop filtering operations can be controlled by the bitstream.

- Whether in-loop filtering across sub-picture boundaries is disabled can be controlled by the bitstream for each sub-picture. Decoding process changes for turning off the three types of in-loop filtering operations at sub-picture boundaries are provided.

- The sub-picture width and height are specified in units of luma samples in SPS. Sub-picture boundaries are constrained to be slice boundaries.

- Treating a sub-picture as a picture in the decoding process (excluding in-loop filtering operations) is specified by slightly updating the coding_tree_unit( ) syntax, and updates to the following decoding processes:

  - The derivation process for (advanced) temporal luma motion vector prediction

  - The luma sample bilinear interpolation process

  - The luma sample 8-tap interpolation filtering process

  - The chroma sample interpolation process

- Sub-picture IDs are explicitly specified in the SPS and included in the tile group headers to enable extraction of sub-picture sequences without the need of changing VCL NAL units.

- Output sub-picture sets (OSPS) are proposed to specify normative extraction and conformance points for sub-pictures and sets thereof.

Detailed specification text changes are provided in an attachment.

# Patent rights declaration(s)

Futurewei Technologies, Inc., Fraunhofer HHI, and Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).