| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0143-v2 |
| --- | --- |

| Title: | AHG12: Signalling of addresses for rectangular slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian Jacob Ström (Ericsson) Hendry Ye-Kui Wang (Futurewei) 10180 Telesis Court San Diego, CA, 92121, USA Sachin Deshpande (Sharp) | Tel: Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com jacob.strom@ericsson.com +1-858-375-8065 fhendry@futurewei.com +1-908-803-3888 ye-kui.wang@futurewei.com sdeshpande@sharplabs.com +1-360-817-8486 |
| Source: | Ericsson, Futurewei Technologies, Inc, Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In the current VVC draft specification, for each rectangular slice, in the PPS, the top-left brick index and a delta value to derive the bottom-right brick index from the top-left brick index are signalled. This contribution proposes to modify the signalling as follows:

- The top-left brick index syntax element is removed. Instead, top-left brick index values are derived.

- The value of the bottom-right brick index is directly signalled without using delta coding in option 1 and with delta coding in option 2.

- Signalling the value of bottom-right brick index for the last rectangular slice in the picture is removed since this can be inferred to be the same as the last brick index in the picture.

The proponents claim that the specification text becomes more compact and clean while at the same time the signalling efficiency improves. A bit saving analysis by the proponents using one example from OMAF and one CE12 example from JVET-M0870 shows PPS bit savings of 89 and 146 bits, respecively for option 1, compared with the current VVC draft. Further a bit saving analysis by the proponents using one example from OMAF and one CE12 example from JVET-M0870 shows PPS bit savings of 157 and 187 bits, respectively for option 2, compared with the current VVC draft.

# Introduction

At the 14th JVET meeting in Geneva, JVET-N0496 suggested a method to improve the signalling efficiency of addresses for rectangular slices by only signalling the bottom-right brick index, instead of signalling both the top-left and bottom-right brick indices. Concerns were expressed during the discussion of the contribution regarding the complexity of the derivation process for the top-left brick index.

After further studying the proposed idea in JVET-N0496 and its interaction with tiles and bricks, it is asserted that the idea to signal only the bottom-right brick index, instead of the top-left brick index and the delta between the bottom-right brick index and the top-left brick index, would work without causing the existing derivation process to be more complicated. In fact, by incorporating the idea, the specification text becomes more compact and clean while at the same time the signalling efficiency improves.

Two options are described. In option 1 bottom right brick index is signalled without delta coding. In option 2 bottom-right brick index is signalled with delta coding.

We therefore recommend the group to consider adopting the idea, and the required text changes for the idea based on the latest VVC WD are provided below for option 1 and option 2.

# Proposal text

## Proposed Text for option 1:

## PPS syntax and semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| ... |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) |  |
| bottom_right_brick_idx[ i ] | u(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

...

top_left_brick_idx[ i ] specifies the brick index of the brick located at the top-left corner of the i-th slice. The value of top_left_brick_idx[ i ] shall not be equal to the value of top_left_brick_idx[ j ] for any i not equal to j. When not present, the value of top_left_brick_idx[ i ] is inferred to be equal to i. The length of the top_left_brick_idx[ i ] syntax element is Ceil( Log2( NumBricksInPic ) bits.

bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ]. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits.

bottom_right_brick_idx[ i ] specifies the brick index of the brick located at the bottom-right corner of the i-th slice. The length of the bottom_right_brick_idx[ i ] syntax element is Ceil( Log2( NumBricksInPic ) ) bits. The value of bottom_right_brick_idx[ num_slices_in_pic_minus1 ] is inferred to be equal to NumBricksInPic − 1. When single_brick_per_slice_flag is equal to 1, bottom_right_brick_idx[ i ] is inferred to be equal to i.

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

NumBricksInSlice[ i ] = 0
botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ top_left_brick_idx[ i ] ]  &&
					BrickColBd[ j ]  <=  BrickColBd[ botRightBkIdx ]  &&
					BrickRowBd[ j ]  >=  BrickRowBd[ top_left_brick_idx[ i ] ]  &&	(7-35)
					BrickRowBd[ j ] <= BrickColBd[ botRightBkIdx ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

The variable TopLeftBrickIdx[ i ], NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the brick index of the brick located at the top left corner of the i-th slice, the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

for( j = 0; i  = =  0  &&  j < NumBricksInPic; j++ )
		BricksToSliceMap[ j ] = −1
NumBricksInSlice[ i ] = 0
for( j = bottom_right_brick_idx[ i ]; j  >=  0; j− − ) {
		if( BrickColBd[ j ]  <=  BrickColBd[ bottom_right_brick_idx[ i ] ]  &&	(7-35)
			BrickRowBd[ j ]  <=  BrickRowBd[ bottom_right_brick_idx[ i ] ]  &&
					BricksToSliceMap[ j ]  = =  −1 ) {
			TopLeftBrickIdx[ i ] = j
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

## Slice header semantics

...

The variable NumBricksInCurrSlice, which specifies the number of bricks in the current slice, and SliceBrickIdx[ i ], which specifies the brick index of the i-th brick in the current slice, are derived as follows:

if( rect_slice_flag ) {
	sliceIdx = 0
	while( slice_address  !=  rect_slice_id[ sliceIdx ] )
		sliceIdx++
	NumBricksInCurrSlice = NumBricksInSlice[ sliceIdx ]
	brickIdx = top_left_brick_idxTopLeftBrickIdx[ sliceIdx ]
	botBrickIdx = brickIdx + bottom_right_brick_idx_delta[ brickIdx ]
	for( bIdx = 0; brickIdx  <=  botBrickIdxbottom_right_brick_idx[ sliceIdx ]; brickIdx++ )	(7-39)
		if( BricksToSliceMap[ brickIdx ]  = =  sliceIdx )
			SliceBrickIdx[ bIdx++ ] = brickIdx
} else {
	NumBricksInCurrSlice = num_bricks_in_slice_minus1 + 1
	SliceBrickIdx[ 0 ] = slice_address
	for( i = 1; i < NumBricksInCurrSlice; i++ )
		SliceBrickIdx[ i ] = SliceBrickIdx[ i − 1 ] + 1
}

## Proposed Text for option 2:

## PPS syntax and semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| ... |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| bottom_right_brick_idx_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| sign_bottom_right_brick_idx_delta[ i ] | u(1) |
| } |  |
| } |  |
| ... |  |
| } |  |
| ... |  |
| } |  |

...

bottom_right_brick_idx_length_minus1 plus 1 specifies the number of bits used to represent the syntax element bottom_right_brick_idx_delta[ i ]. The value of bottom_right_brick_idx_length_minus1 shall be in the range of 0 to Ceil( Log2( NumBricksInPic ) ) − 1, inclusive.

top_left_brick_idx[ i ] specifies the brick index of the brick located at the top-left corner of the i-th slice. The value of top_left_brick_idx[ i ] shall not be equal to the value of top_left_brick_idx[ j ] for any i not equal to j. When not present, the value of top_left_brick_idx[ i ] is inferred to be equal to i. The length of the top_left_brick_idx[ i ] syntax element is Ceil( Log2( NumBricksInPic ) bits.

bottom_right_brick_idx_delta[ i ] when i is greater than 0 specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx [ i ] and the brick index of the bottom-right corner of the (i-1)’th slice. bottom_right_brick_idx_delta[ 0 ] specifies the brick index of the bottom right corner of the 0-th slice. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 1. The value of the BottomRightBrickIdx[ num_slices_in_pic_minus1 ] is inferred to be equal to NumBricksInPic – 1. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) (bottom_right_brick_idx_length_minus1+1) bits.

sign_bottom_right_brick_idx_delta[ i ]  equal to 1 indicates a positive sign for bottom_right_brick_idx_delta[ i ]. sign_bottom_right_brick_idx_delta[ i ]  equal to 0 indicates a negative sign for bottom_right_brick_idx_delta[ i ].

It is a requirement of bitstream conformance that a slice shall include either a number of complete tiles or only a consecutive sequence of complete bricks of one tile.

The variable NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

NumBricksInSlice[ i ] = 0
botRightBkIdx = top_left_brick_idx[ i ] + bottom_right_brick_idx_delta[ i ]
for( j = 0; j < NumBricksInPic; j++) {
		if( BrickColBd[ j ]  >=  BrickColBd[ top_left_brick_idx[ i ] ]  &&
					BrickColBd[ j ]  <=  BrickColBd[ botRightBkIdx ]  &&
					BrickRowBd[ j ]  >=  BrickRowBd[ top_left_brick_idx[ i ] ]  &&	(7-35)
					BrickRowBd[ j ] <= BrickColBd[ botRightBkIdx ] ) {
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

The variable TopLeftBrickIdx[ i ], NumBricksInSlice[ i ] and BricksToSliceMap[ j ], which specify the brick index of the brick located at the top left corner of the i-th slice, the number of bricks in the i-th slice and the mapping of bricks to slices, are derived as follows:

for( j = 0; i  = =  0  &&  j < NumBricksInPic; j++ )
		BricksToSliceMap[ j ] = −1
NumBricksInSlice[ i ] = 0
for( j = ( BottomRightBrickIdx[ i ] = bottom_right_brick_idx_delta[ i ] ] + ( i > 0 ?
			(sign_bottom_right_brick_idx_delta[ i ] ? BottomRightBrickIdx[ i − 1 ] :
		 −BottomRightBrickIdx[ i−1 ] ) : 0 ); j  >=  0; j− − ) {
		if( BrickColBd[ j ]  <=  BrickColBd[ BottomRightBrickIdx[ i ] ]  &&	(7-35)
			BrickRowBd[ j ]  <=  BrickRowBd[ BottomRightBrickIdx[ i ] ]  &&					BricksToSliceMap[ j ]  = =  −1 ) {
			TopLeftBrickIdx[ i ] = j
			NumBricksInSlice[ i ]++
			BricksToSliceMap[ j ] = i
		}
}

# Bit count examples

In order to measure the impact on bit cost we analyzed two example tile and slice layouts. The analysis is summarized below and the details are attached to this contribution as an excel file.

## JVET-M0870 example

The first example is based on JVET-M0870 “AHG12: Proposed JVET common test conditions and evaluation procedures for MCTS and sub-pictures with boundary padding”. This document was written for MCTS and sub-picture testing for the CE12 at the time and not to showcase this proposal. JVET-M0870 specifies two use-cases with 6x4 and 12x8 tiles respectively. The example we investigated was to use 12x8 tiles and 6x4 slices per picture with each slice consisting of 2x2 tiles. Our analysis show that the 307 bits needed in the current VVC draft is reduced by 146 bits or 47.6% by option 1 in this proposal.

Our analysis show that the 307 bits needed in the current VVC draft is reduced by 187 bits or 61.0% by option 2 in this proposal.

## OMAF example

The second example is based on an OMAF example as shown in the figure below. Here, each picture consist of 24 bricks, each in its own slice. For this example, our analysis show that the 204 bits needed in the current VVC draft is reduced by 89 bits or 43.6% by option 1 in this proposal. For this example, our analysis show that the 204 bits needed in the current VVC draft is reduced by 157 bits or 77.0% by option 2 in this proposal. Note that the savings are the same for any brick layout with 24 bricks each in its own slice.

# Patent rights declaration(s)

Ericsson, Futurewei Technologies, Inc. and Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).