| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0163-v2 |
| --- | --- |

| Title: | Non-CE4: Simplification on SbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang | Tel: Email: | chenhuanbang@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In version 2, the spec text for aspect 1 only was provided.

This contribution proposes to simplify the derivation of SbTMVP candidate in sub-block merge mode in two aspects. Firstly, the adaptive L0/L1 order for checking spatial motion vectors when deriving the temporal motion vector is removed. It is proposed to always check L0 motion vector firstly. Secondly, the availability check of SbTMVP candidate and the derivation of base motion data is removed. If the collocated sub-block of a sub-block is not inter coded, the motion of that sub-block is set to zero.

For aspect 1, simulation results reportedly show on average 0.00%/0.01% luma BD-rate in RA/LB configurations, compared to VTM-5.0. For aspect 2, simulation results reportedly show on average 0.02%/0.01% luma BD-rate in RA/LB configurations. Simulation results for the combination of two aspects reportedly show on average 0.02%/0.05% luma BD-rate in RA/LB cases.

Introduction

In current VVC, the availability of SbTMVP is decided by checking the prediction mode of the collocated block of current CU. If the collocated block is inter coded, SbTMVP is available. Then the motion data of the collocated block is fetched and scaled to be the base motion data, which is used for filling the sub-block motion data when collocated sub-block is not inter coded.

According to the working draft, the SbTMVP candidate is derived as follow:

- The temporal motion vector is derived by searching a motion vector pointing to collocated picture from the motion information of bottom-left spatial neighboring block (A1):

  - If collocated_from_l0_flag is equal to 0 and current slice is low delay case, MvL1A1 is checked firstly and then MvL0A1 is checked;

  - Otherwise (collocated_from_l0_flag is equal to 1 or current slice is not low delay case), only MvL0A1 is checked.

- The availability of SbTMVP candidate and base motion data is derived as follow:

  - The location of collocated block is derived based on the location of current CU and the temporal motion vector;

  - If the collocated block of current CU is inter coded, SbTMVP is available and then the base motion data is derived; otherwise, SbTMVP is not available;

- If SbTMVP is available, the following applies for each sub-block:

  - The location of the collocated sub-block is derived based on the location of current sub-block and temporal motion vector;

  - If the collocated sub-block is inter coded, the collocated motion vector is derived and assigned to the motion vector of current sub-block; otherwise, the motion vector for current sub-block is set to the base motion vector.

Proposed method

Aspect 1: Derivation of temporal motion vector

In the working draft, if collocated_from_l0_flag is equal to 0 and current slice is low delay case, MvL1A1 is checked firstly and then MvL0A1 is checked. Otherwise, only MvL0A1 is checked.

While in VTM5.0, if collocated_from_l0_flag is equal to 0 and current slice is low delay case, MvL1A1 is checked firstly and then MvL0A1 is checked. Otherwise, MvL0A1 is checked firstly and then MvL1A1 is checked.

In this proposal, the adaptive L0/L1 checking order is removed. MvL0A1 is always checked firstly, followed by the checking of MvL1A1.

Aspect 2: Removal of availability check and derivation of base motion data

In this proposal, the derivation of SbTMVP is simplified by removing the availability check of SbTMVP candidate and the derivation of base motion data. If the collocated sub-block of a sub-block is not inter coded, the motion of that sub-block is set to zero. With this proposal method, the SbTMVP candidate will be always available and then in decoder side, when merge_subblcok_index is not equal to zero, all SbTMVP merge candidate derivation can be skipped.

The derivation of SbTMVP candidate is simplified as follow:

- The temporal motion vector is derived by searching a motion vector pointing to collocated picture from the motion information of bottom-left spatial neighboring block (A1);

- The availability of SbTMVP candidate and base motion data is derived as follow:

  - The location of collocated block is derived based on the location of current CU and the temporal motion vector;

  - If the collocated block of current CU is inter coded, SbTMVP is available and then the base motion data is derived; otherwise, SbTMVP is not available;

- If SbTMVP is available, tThe following applies for each sub-block:

  - The location of the collocated sub-block is derived based on the location of current sub-block and temporal motion vector;

  - If the collocated sub-block is inter coded, the collocated motion vector is derived and assigned to the motion vector of current sub-block; otherwise, the motion vector for current sub-block is set to the base motion vectorzero.

Test specification

The tests for this contribution are descripted in Table 1.

Table 1 test specification for this contribution

| Test # | Test description |
| --- | --- |
| Test 1 | Simplification in derivation of temporal motion vector |
| Test 2 | Removal of availability check and derivation of base motion data |
| Test 3 | Test 1 + Test 2 |

Experiment results

The proposed methods has been implemented on top of VTM5.0. The results the test are shown below against VTM5.0 [1] under CTC [2].

Table 2 Coding performance of test 1 over VTM-5.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.04% | 0.01% | 100% | 100% |
| Class B | 0.02% | 0.07% | 0.01% | 100% | 100% |
| Class C | -0.01% | 0.01% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.01% | 0.08% | 100% | 101% |
| Class F (optional) | 0.00% | 0.01% | -0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.06% | 0.05% | 100% | 101% |
| Class C | 0.05% | -0.28% | 0.04% | 100% | 101% |
| Class E | -0.01% | 0.06% | -0.81% | 100% | 101% |
| Overall | 0.01% | -0.05% | -0.17% | 100% | 101% |
| Class D | -0.04% | 0.11% | 0.62% | 100% | 101% |
| Class F (optional) | -0.09% | -0.23% | 0.00% | 100% | 100% |

Table 3 Coding performance of test 2 over VTM-5.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.08% | 0.07% | 100% | 100% |
| Class A2 | 0.00% | 0.09% | 0.02% | 100% | 100% |
| Class B | 0.03% | 0.03% | -0.04% | 100% | 100% |
| Class C | 0.01% | 0.06% | -0.08% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | -0.02% | 100% | 100% |
| Class D | 0.04% | 0.07% | 0.19% | 101% | 101% |
| Class F | 0.04% | 0.11% | 0.04% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.07% | -0.05% | 100% | 101% |
| Class C | 0.05% | 0.05% | 0.18% | 101% | 101% |
| Class E | -0.01% | 0.19% | -0.26% | 101% | 101% |
| Overall | 0.01% | 0.09% | -0.03% | 100% | 101% |
| Class D | 0.02% | 0.35% | 0.86% | 101% | 101% |
| Class F | 0.01% | 0.19% | 0.04% | 100% | 100% |

Table 4 Coding performance of test 3 over VTM-5.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.08% | 0.05% | 100% | 100% |
| Class A2 | 0.01% | 0.08% | -0.02% | 100% | 100% |
| Class B | 0.02% | 0.05% | -0.01% | 100% | 101% |
| Class C | 0.01% | 0.04% | -0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | -0.02% | 100% | 101% |
| Class D | 0.06% | 0.10% | 0.24% | 100% | 101% |
| Class F | 0.03% | 0.10% | 0.03% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.23% | 0.11% | 100% | 103% |
| Class C | 0.00% | -0.03% | -0.33% | 100% | 101% |
| Class E | 0.15% | 0.72% | -0.13% | 100% | 101% |
| Overall | 0.05% | 0.26% | -0.10% | 100% | 102% |
| Class D | -0.02% | 0.16% | 0.08% | 100% | 101% |
| Class F | -0.10% | 0.93% | 0.33% | 99% | 100% |

Conclusions

This contribution proposes to simplify the derivation of SbTMVP candidate in sub-block merge mode by two aspects. Firstly, the adaptive L0/L1 order for checking spatial motion vectors when deriving the temporal motion vector is removed. It is proposed to always check L0 motion vector firstly. Secondly, the availability check of SbTMVP candidate and the derivation of base motion data is removed. It is recommended to adopt these simplifications into VVC standard.

Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Working draft for aspect 1

The text to be added is mark as green and the text to be removed is marked as red with strikethrough

- 8.5.5.4 Derivation process for subblock-based temporal merging base motion data

…

When availableFlagA1 is equal to TRUE, the following applies:

- If all of the following conditions are true, tempMv is set equal to mvL1A1mvL0A1:

  - predFlagL1A1predFlagL0A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 10 ][refIdxL1A1refIdxL0A1]) is equal to 0,

  - DiffPicOrderCnt(aPic, currPic) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice,

  - slice_type is equal to B,

  - collocated_from_l0_flag is equal to 0.

- Otherwise if all of the following conditions are true, tempMv is set equal to mvL0A1mvL1A1:

  - slice_type is equal to B,

  - predFlagL0A1predFlagL1A1 is equal to 1,

  - DiffPicOrderCnt(ColPic, RefPicList[ 01 ][refIdxL0A1refIdxL1A1]) is equal to 0.

The location ( xColCb, yColCb ) of the collocated block inside ColPic is derived as follows.

…