| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0164-v1 |
| --- | --- |

| Title: | Non-CE4: Simplification on AMVP list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang | Tel: Email: | chenhuanbang@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In current VVC, a two pass approach is conducted for searching the spatial motion vectors in AMVP list construction. This contribution proposes to remove the second pass search of spatial motion vectors. It is claimed that with this modification, spatial motion vector scaling is totally removed from VTM-5.0. Simulation results reportedly show 0.01%/-0.03% luma BR-rate for RA/LB configurations, compared to VTM-5.0.

Introduction

In current VVC, the MVP list for inter mode is constructed as follow:

- Insert a non-scaled spatial predictor from left positions (A0, A1) if found;

- If non-scaled left spatial predictor is not found, insert a scaled left spatial predictor if found;

- Insert a non-scaled spatial predictor from above positions (B0, B1, B2) if found;

- If left spatial predictor is not found, insert a second above spatial predictor (non-scaled or scaled) if found;

- Pruning between the first two predictors;

- If not enough predictors in the list, insert temporal predictor;

- If not enough predictors in the list, insert non-scaled HMVP predictors;

- Fill list with zero motion vector(s).

In this contribution, the second pass search of spatial motion vector is removed, and the construction of MVP list is simplified as follow:

- Insert a non-scaled spatial predictor from left positions (A0, A1) if found;

- Insert a non-scaled spatial predictor from above positions (B0, B1, B2) if found;

- Pruning between the first two predictors;

- If not enough predictors in the list, insert temporal predictor;

- If not enough predictors in the list, insert non-scaled HMVP predictors;

- Fill list with zero motion vector(s).

With this modification, spatial motion vector scaling is totally removed from VTM-5.0.

Experiment results

The proposed methods has been implemented on top of VTM5.0. The results the test are shown below against VTM5.0 [1] under CTC [2].

Table 2 Coding performance of the proposal over VTM-5.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.09% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.04% | 100% | 100% |
| Class B | 0.01% | 0.07% | -0.02% | 101% | 100% |
| Class C | 0.02% | -0.01% | 0.02% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 100% | 100% |
| Class D | 0.02% | 0.18% | 0.15% | 101% | 100% |
| Class F | 0.04% | 0.05% | 0.08% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.15% | 0.12% | 100% | 101% |
| Class C | -0.04% | 0.02% | -0.17% | 100% | 100% |
| Class E | -0.03% | -0.08% | -0.40% | 100% | 101% |
| Overall | -0.03% | 0.05% | -0.11% | 100% | 101% |
| Class D | 0.02% | 0.65% | 0.77% | 100% | 101% |
| Class F | 0.09% | 0.35% | -0.52% | 100% | 100% |

Conclusions

This contribution proposes to remove the second pass search of spatial motion vectors in AMVP list construction. It is claimed that with this modification, spatial motion vector scaling is totally removed from VTM-5.0. It is recommended to adopt this simplification into VVC standard.

Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Working draft

The text to be removed is marked as red with strikethrough

#### 8.5.2.10 Derivation process for spatial motion vector predictor candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the reference index of the current coding unit partition refIdxLX, with X being 0 or 1.

Outputs of this process are (with N being replaced by A or B):

- the motion vectors mvLXN in 1/16 fractional-sample accuracy of the neighbouring coding units,

- the availability flags availableFlagLXN of the neighbouring coding units.

Figure 8-2 provides an overview of spatial motion vector neighbours.

Figure 8-2 – Spatial motion vector neighbours (informative)

The variable currCb specifies the current luma coding block at luma location ( xCb, yCb ) and the variable currPic specifies the current picture.

The variable isScaledFlagLX, with X being 0 or 1, is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- The sample location ( xNbA0, yNbA0 ) is set equal to ( xCb − 1, yCb + cbHeight ) and the sample location ( xNbA1, yNbA1 ) is set equal to ( xNbA0, yNbA0 − 1 ).

- The availability flag availableFlagLXA is set equal to 0 and both components of mvLXA are set equal to 0.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1.

- When availableA0 or availableA1 is equal to TRUE, the variable isScaledFlagLX is set equal to 1.

- The following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ):

- When availableAk is equal to TRUE and availableFlagLXA is equal to 0, the following applies:

- If PredFlagLX[ xNbAk ][ yNbAk ] is equal to 1 and DiffPicOrderCnt( RefPicList[ X ][ RefIdxLX[ xNbAk ][ yNbAk ] ], RefPicList[ X ][ refIdxLX ] ) is equal to 0, availableFlagLXA is set equal to 1 and the following applies:

mvLXA = MvLX[ xNbAk ][ yNbAk ]	(8-384)

- Otherwise, when PredFlagLY[ xNbAk ][ yNbAk ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicList[ Y ][ RefIdxLY[ xNbAk ][ yNbAk ] ], RefPicList[ X ][ refIdxLX ] ) is equal to 0, availableFlagLXA is set equal to 1 and the following applies:

mvLXA = MvLY[ xNbAk ][ yNbAk ]	(8-385)

- When availableFlagLXA is equal to 0, the following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ) or until availableFlagLXA is equal to 1:

- When availableAk is equal to TRUE and availableFlagLXA is equal to 0, the following applies:

- If PredFlagLX[ xNbAk ][ yNbAk ] is equal to 1 , availableFlagLXA is set equal to 1 and the following assignments are made:

mvLXA = MvLX[ xNbAk ][ yNbAk ]	(8-386)

refIdxA = RefIdxLX[ xNbAk ][ yNbAk ]	(8-387)

refPicListA = RefPicList[ X ]	(8-388)

- Otherwise, if PredFlagLY[ xNbAk ][ yNbAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made:

mvLXA = MvLY[ xNbAk ][ yNbAk ]	(8-389)

refIdxA = RefIdxLY[ xNbAk ][ yNbAk ]	(8-390)

refPicListA = RefPicList[ Y ]	(8-391)

- When availableFlagLXA is equal to 1, DiffPicOrderCnt( refPicListA[ refIdxA ], RefPicList[ X ][ refIdxLX ] ) is not equal to 0, mvLXA is derived as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-392)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-393)

mvLXA = Clip3( −131072, 131071, ( distScaleFactor * mvLXA + 
			128 − ( distScaleFactor * mvLXA >= 0 ) )  >>  8 )	(8-394)

where td and tb are derived as follows:

td = Clip3( −128, 127, DiffPicOrderCnt( currPic, refPicListA[ refIdxA ] ) )	(8-395)

tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList[ X ][ refIdxLX ] ) )	(8-396)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- The sample locations ( xNbB0, yNbB0 ), ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) are set equal to ( xCb + cbWidth, yCb − 1 ), ( xCb + cbWidth − 1, yCb − 1 ) and ( xCb − 1, yCb − 1 ), respectively.

- The availability flag availableFlagLXB is set equal to 0 and the both components of mvLXB are set equal to 0.

- The following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk.

- When availableBk is equal to TRUE and availableFlagLXB is equal to 0, the following applies:

- If PredFlagLX[ xNbBk ][ yNbBk ] is equal to 1, and DiffPicOrderCnt( RefPicList[ X ][ RefIdxLX[ xNbBk ][ yNbBk ] ], RefPicList[ X ][ refIdxLX ] ) is equal to 0, availableFlagLXB is set equal to 1 and the following assignments are made:

mvLXB = MvLX[ xNbBk ][ yNbBk ]	(8-397)

refIdxB = RefIdxLX[ xNbBk ][ yNbBk ]	(8-398)

- Otherwise, when PredFlagLY[ xNbBk ][ yNbBk ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicList[ Y ][ RefIdxLY[ xNbBk ][ yNbBk ] ], RefPicList[ X ][ refIdxLX ] ) is equal to 0, availableFlagLXB is set equal to 1 and the following assignments are made:

mvLXB = MvLY[ xNbBk ][ yNbBk ]	(8-399)

refIdxB = RefIdxLY[ xNbBk ][ yNbBk ]	(8-400)

- When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1, availableFlagLXA is set equal to 1 and the following applies:

mvLXA = mvLXB	(8-401)

- When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and the following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ) or until availableFlagLXB is equal to 1:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk.

- When availableBk is equal to TRUE and availableFlagLXB is equal to 0, the following applies:

- If PredFlagLX[ xNbBk ][ yNbBk ] is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made:

mvLXB = MvLX[ xNbBk ][ yNbBk ]	(8-402)

refIdxB = RefIdxLX[ xNbBk ][ yNbBk ]	(8-403)

refPicListB = RefPicList[ X ]	(8-404)

- Otherwise, when PredFlagLY[ xNbBk ][ yNbBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made:

mvLXB = MvLY[ xNbBk ][ yNbBk ]	(8-405)

refIdxB = RefIdxLY[ xNbBk ][ yNbBk ]	(8-406)

refPicListB = RefPicList[ Y ]	(8-407)

- When availableFlagLXB is equal to 1, DiffPicOrderCnt( refPicListB[ refIdxB ], RefPicList[ X ][ refIdxLX ] ) is not equal to 0, mvLXB is derived as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-408)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-409)

mvLXB = Clip3( −131072, 131071,  (distScaleFactor * mvLXB + 
				128 − ( distScaleFactor * mvLXB >= 0 ) )  >>  8 )	(8-410)

where td and tb are derived as follows:

td = Clip3( −128, 127, DiffPicOrderCnt( currPic, refPicListB[ refIdxB ] ) )	(8-411)

tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList[ X ][ refIdxLX ] ) )	(8-412)