| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0168-v1 |
| --- | --- |

| Title: | Non-CE3: MIP low resolution clipping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thibaud Biatek, Geert Van Der Auwera, Marta Karczewicz | Email: | tbiatek@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to reduce the number of required clipping operations for MIP in VVC and for the proposed 8-bit MIP in JVET-O0084. Instead of clipping the upsampled output prediction, the clipping is achieved on the low-resolution prediction prior to the upsampling to reduce MIP complexity.

# Introduction

In the current VVC specification the prediction of a given block using MIP follows three steps:

- Reduced block boundary is computed via Haar-downsampling. The output vector is used as input for the prediction.

- Matrix-vector multiplication with addition of an offset is achieved with reduced-boundary reference vector as input. The output is a low-resolution prediction.

- Linear interpolation is achieved on both row and column if needed to generate full-resolution prediction signal followed by a clipping operation.

It is proposed to reduce the number of clipping operations required by MIP and it tested on both the MIP in VVC and the proposed 8-bit MIP in JVET-O0084.

# Low-resolution clipping

The step 2 (matrix-vector plus offset computation) may lead to out-of-range values that need to be clipped. In the current VVC specification the clipping operation is performed after the linear-interpolation (step 3), on the full-resolution output prediction. An inconvenience of this approach is that it leads to clip a large amount of samples in some cases even though prediction is achieved at a lower resolution.

In this contribution, we propose to shift the clipping operation prior to the linear interpolation to reduce the complexity of the MIP method. Potential gains are provided in Table 1 where we can notice that the proposed method limits the number of clipping operations to 64 (worst case) while it reaches 1024 using the current specification.

Table 1: Number of clipping operations in MIP

| Current specification |  |  |  |  |  | Proposed method |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W=4 | W=8 | W=16 | W=32 |  |  | W=4 | W=8 | W=16 | W=32 |
| H=4 | 16 | 32 |  |  |  | H=4 | 16 | 16 |  |  |
| H=8 | 32 | 64 | 128 |  |  | H=8 | 16 | 16 | 64 |  |
| H=16 |  | 128 | 256 | 512 |  | H=16 |  | 64 | 64 | 64 |
| H=32 |  |  | 512 | 1024 |  | H=32 |  |  | 64 | 64 |

# Proposed specification changes

The proposed method has been tested on top of both VTM-5.0 [2] and 8bit MIP proposal [3] which involves several variations of the specification, described in the following subsections.

## Low-resolution clipping, on top of VTM-5.0

“

For the intra sample prediction process according to predModeIntra, the following ordered steps apply:

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

- The variable modeId is derived as follows:

modeId = predModeIntra − ( isTransposed  ?  numModes / 2  :  0 )	(8-64)

- The weight matrix mWeight[ x ][ y ] with x = 0..2 * boundarySize − 1, y = 0..predC * predC − 1is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-XX [Ed. (BB): add weight matrices once a non-10-bit weight solution is adopted].

- The bias vector vBias[ y ] with y = 0..predC * predC − 1 is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-XX [Ed. (BB): add bias vectors once a non-10-bit weight solution is adopted].

- The variable sW is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-8.

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

oW = 1 << ( sW − 1 )		(8-65)

sB = BitDepthY − 1		(8-66)

incW = ( predC > mipW )  ?  2  :  1		(8-67)

incH = ( predC > mipH )  ?  2  :  1		(8-68)

predMip[ x ][ y ] = ( (i = 02 * boundarySize - 1mWeight i  y*incH*predC+x*incW *p i ) + 
	( vBias[ y * incH * predC + x * incW ]  << sB ) + oW ) >> sW	(8-69)

- The matrix-based intra prediction samples predMip[ x ][ y ] are clipped as follows:

predMip[ x ][ y ] = Clip1Y( predMip[ x ][ y ] )					   (8-70)

- When isTransposed is equal to TRUE, the predH x predW array predMip[ x ][ y ] with x = 0..predH − 1, y = 0..predW − 1 is transposed as follows:

predTemp[ y ][ x ] = predMip[ x ][ y ]	 	(8-71)

predMip =predTemp			(8-72)

- The predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If needUpsBdryVer is equal to TRUE or needUpsBdryHor is equal to TRUE, the MIP prediction upsampling process as specified in clause 8.4.5.2.4 is invoked with the input block width predW, the input block height predH, matrix-based intra prediction samples predMip[ x ][ y ] with x = 0..predW − 1, y = 0..predH − 1, the transform block width nTbW, the transform block height nTbH, the upsampling boundary width upsBdryW, the upsampling boundary height upsBdryH, the top upsampling boundary samples upsBdryT, and the left upsampling boundary samples upsBdryL as inputs, and the output is the predicted sample array predSamples.

- Otherwise, predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 is set equal to predMip[ x ][ y ].

- The predicted samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are clipped as follows:

predSamples[ x ][ y ] = Clip1Y( predSamples[ x ][ y ] )

“

## Low-resolution clipping, on top of 8bit MIP

“

For the intra sample prediction process according to predModeIntra, the following ordered steps apply:

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

- The variable modeId is derived as follows:

modeId = predModeIntra − ( isTransposed  ?  numModes / 2  :  0 )	(8-64)

- The weight matrix mWeight[ x ][ y ] with x = 0..2 * boundarySize − 1, y = 0..predC * predC − 1is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-XX [Ed. (BB): add weight matrices once a non-10-bit weight solution is adopted, Note (PM): mWeight has only 7 columns for MipSizeId == 2].

- The variable sW is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-8.

- The variable fO is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-X.

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

oW = ( 1 << ( sW − 1 ) ) – fO * ( i = 0inSize - 1p i  )	(8-65)

incW = ( predC > mipW )  ?  2  :  1		(8-67)

incH = ( predC > mipH )  ?  2  :  1		(8-68)

predMip[ x ][ y ] = ( ( (i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i ) 
	 + oW ) >> sW )  + dcVal	(8-69)

- The matrix-based intra prediction samples predMip[ x ][ y ] are clipped as follows:

predMip[ x ][ y ] = Clip1Y( predMip[ x ][ y ] )					   	  (8-70)

- When isTransposed is equal to TRUE, the predH x predW array predMip[ x ][ y ] with x = 0..predH − 1, y = 0..predW − 1 is transposed as follows:

predTemp[ y ][ x ] = predMip[ x ][ y ]	 	(8-71)

predMip = predTemp			(8-72)

- The predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If nTbW is greater than predW ornTbH is greater than predH, the MIP prediction upsampling process as specified in clause 8.4.5.2.4 is invoked with the input block width predW, the input block height predH, matrix-based intra prediction samples predMip[ x ][ y ] with x = 0..predW − 1, y = 0..predH − 1, the transform block width nTbW, the transform block height nTbH, the top reference samples refT[ x ] with x = 0..nTbW − 1 , and the left reference samples refL[ y ] with y = 0..nTbH – 1 as inputs, and the output is the predicted sample array predSamples.

- Otherwise, predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 is set equal to predMip[ x ][ y ].

- The predicted samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are clipped as follows:

predSamples[ x ][ y ] = Clip1Y( predSamples[ x ][ y ] )	(8-72)

“

# Experimental Results

The proposed method was integrated into both VTM5.0 and 8bit-MIP and tested under the CTC [1] against VTM-5.0. Results are reported in the following subsections.

## Low-resolution clipping on top of VTM-5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.02% | 100% | 98% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.02% | 0.03% | 102% | 104% |
| Class C | 0.00% | 0.02% | -0.04% | 100% | 99% |
| Class E | 0.00% | 0.01% | 0.04% | 100% | 100% |
| Overall | 0.00% | 0.01% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.02% | -0.01% | 101% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | -0.02% | 100% | 97% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | -0.04% | 0.05% | 101% | 97% |
| Class C | -0.02% | -0.09% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.05% | 0.05% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | 0.04% | -0.11% | 102% | 104% |
| Class E | -0.08% | -0.56% | -0.15% | 101% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.05% | 0.05% | 0.27% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.02% | 0.03% | 0.16% | 102% | 101% |
| Class E | 0.00% | 0.01% | 0.28% | 101% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.03% | 0.10% | -0.18% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Low-resolution clipping on top of 8bit MIP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.05% | -0.12% | -0.10% | 101% | 104% |
| Class C | -0.02% | -0.05% | 0.00% | 100% | 95% |
| Class E | -0.06% | -0.10% | -0.06% | 99% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.08% | -0.04% | 106% | 124% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.02% | -0.03% | -0.29% | 102% | 98% |
| Class C | -0.03% | -0.14% | -0.05% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.06% | 0.11% | 107% | 114% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | -0.01% | 0.44% | 0.69% | 102% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.06% | -0.07% | 1.03% | 105% | 110% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | -0.04% | 1.10% | 1.10% | 101% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.52% | 0.10% | 103% | 107% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution proposes to perform the range clipping operation before interpolation. Experimental results show that the method has no significant impact on coding performance whether it is tested on top of the VTM-5.0 or 8bit MIP proposal.

# Acknowledgement

We would like to thank XXX for crosschecking our contribution.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sürhing, “JVET Common Test Condition and Software Reference Configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

- J. Chen, Y. Ye and S. Kim, “Algorithm Description for Versatile Video Coding and Test Model 5 (VTM5)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1002, Geneva, CH, 19–27 Mar. 2019.

- J. Pfaff, T. Hinz, P. Helle, P. Mekle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand, “8-bit implementation and simplification of MIP”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O0084, Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).