| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0170-v1 |
| --- | --- |

| Title: | Non-CE3: Simplifications of Intra Mode Coding for Matrix-based Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | B. Wang, A. M. Kotra, S. Esenlik H. Gao, E. Alshina (Huawei) | Tel: Email: | biao.wang@huawei.com, |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents two simplifications on intra mode coding for blocks applied with Matrix-based Intra Prediction (MIP). The first simplification uses a default mode list as the MPM list used in MIP intra mode coding. The second simplification removes the mapping tables between normal intra modes and intra modes of MIP. Compared to VTM5.0 with all intra configuration, the two simplifications brings coding performance difference of 0.00% and 0.01%, respectively. When the two simplifications are combined, a coding performance difference of 0.01% is observed. The first simplification makes the MPM list construction not necessary, as a simple default mode list is used. The second simplification reduces access to rom, as mapping table stored in rom is not required.

# Problems

Matrix-based Intra Prediction (MIP) [1] use trained matrix coefficients to perform intra prediction. According to the block size, the intra prediction modes of MIP are classified into three categories:

- 35 modes for 4x4 blocks, i.e. block size type I.

- 19 modes for 8x4, 4x8 and 8x8 blocks, i.e. block size type II.

- 11 modes for other cases where block width and height are smaller than or equal to 64 samples, i.e. block size type III.

A 3-MPM list is used for intra mode coding in MIP. The MPM index is coded using truncated unary codeword. The non-MPM modes, regardless of their associated block size type, are always power of 2 (32 non-MPM modes for block size type I, 16 for II, and 8 for III) and are coded with fixed length codeword. The 3-MPM list is constructed based on the left and above blocks of the current block in an HEVC-like fashion.

The introduction of MIP on top of normal intra prediction raises two potential problems:

- The 3-MPM list construction in MIP intra mode coding introduces another MPM list construction logic on top of the 6-MPM list construction for normal intra mode coding. A simpler MPM list construction process or even a process without construction is desired.

- If a current block is applied with normal intra mode, and its neighboring blocks is/are applied with MIP, a mapping table is used in normal 6-MPM list construction to convert neighboring blocks’ MIP intra mode to normal intra mode. Vice versa, if a current block is applied with MIP, and its neighboring blocks is/are applied with normal intra mode, a mapping table is used in MIP 3-MPM list construction to convert neighboring blocks’ normal intra mode to MIP intra mode. As mapping table is stored in rom and MPM list construction is required for every block applied with normal or MIP intra prediction, frequent access to rom is required.

# Proposed Solutions

In this contribution, two corresponding simplifications are proposed.

The first simplification uses a default MIP intra mode list as the 3-MPM list for each categories. They are:

- {17, 5, 34} for block size type I.

- {0, 8, 17} for block size type II.

- {1, 6, 8} for block size type III.

With this simplification, the 3-MPM list construction is not necessary (with X checks removed) and dependences to neighboring blocks is also removed. To achieve a fair comparison, the number of candidates in MIP MPM returned for the encoder rate-distortion check is not changed.

The second simplification removes the usage of mapping tables between normal intra mode and MIP intra mode. In MPM list construction, when a current block is applied with normal intra mode and a neighboring blocks is applied with MIP, the neighboring block is considered as not available; Vice versa, if a current block is applied with MIP and a neighboring block is applied with normal intra mode, the neighboring block is considered as not available. MIP is applied with luma component only. When chroma intra prediction mode is derived from luma intra prediction mode, and the luma intra prediction mode is applied with MIP, it is considered the luma intra prediction is the planar mode in normal intra prediction modes.

# Experimental Results

The simulations are performed according to common test conditions (CTC) as described in [2]. AI and RA results are reported.

## Simulation result for the 1st simplification

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | -0.01% | 99% | 98% |
| Class A2 | 0.00% | -0.10% | -0.05% | 100% | 101% |
| Class B | 0.00% | -0.03% | 0.02% | 100% | 99% |
| Class C | -0.01% | 0.00% | 0.07% | 100% | 98% |
| Class E | 0.00% | -0.02% | 0.00% | 101% | 104% |
| Overall | 0.00% | -0.04% | 0.01% | 100% | 100% |
| Class D | -0.01% | 0.10% | -0.01% | 100% | 101% |
| Class F | -0.01% | -0.05% | -0.04% | 100% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.16% | 0.06% | 100% | 99% |
| Class A2 | 0.03% | 0.03% | 0.00% | 100% | 100% |
| Class B | 0.00% | -0.14% | 0.00% | 100% | 100% |
| Class C | -0.03% | -0.18% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.12% | 0.01% | 100% | 100% |
| Class D | 0.01% | 0.16% | 0.02% | 100% | 99% |
| Class F | 0.00% | -0.08% | 0.22% | 100% | 100% |

## Simulation result for the 2nd simplification

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.10% | 98% | 99% |
| Class A2 | 0.03% | -0.01% | -0.02% | 100% | 101% |
| Class B | 0.01% | 0.01% | 0.05% | 99% | 100% |
| Class C | 0.00% | -0.01% | 0.07% | 99% | 99% |
| Class E | 0.02% | -0.07% | -0.08% | 100% | 101% |
| Overall | 0.01% | 0.00% | 0.03% | 99% | 100% |
| Class D | -0.01% | 0.04% | 0.02% | 100% | 98% |
| Class F | 0.00% | 0.00% | 0.04% | 99% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.14% | 99% | 100% |
| Class A2 | 0.05% | 0.19% | 0.12% | 99% | 100% |
| Class B | 0.03% | 0.03% | 0.08% | 99% | 100% |
| Class C | -0.05% | 0.03% | -0.07% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.06% | 99% | 99% |
| Class D | 0.03% | 0.09% | 0.10% | 99% | 98% |
| Class F | -0.01% | -0.09% | 0.20% | 99% | 99% |

## Simulation result for the combined simplifications

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.11% | 0.03% | 98% | 98% |
| Class A2 | 0.02% | -0.03% | -0.01% | 100% | 100% |
| Class B | 0.01% | -0.01% | 0.05% | 99% | 100% |
| Class C | 0.01% | -0.06% | 0.06% | 99% | 97% |
| Class E | 0.01% | -0.03% | 0.06% | 100% | 99% |
| Overall | 0.01% | -0.01% | 0.04% | 99% | 99% |
| Class D | 0.01% | 0.19% | -0.02% | 99% | 98% |
| Class F | -0.01% | -0.01% | 0.04% | 99% | 98% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.14% | 0.20% | 99% | 100% |
| Class A2 | 0.03% | 0.14% | 0.08% | 99% | 100% |
| Class B | 0.01% | 0.08% | -0.06% | 99% | 101% |
| Class C | -0.01% | -0.05% | -0.05% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 99% | 100% |
| Class D | -0.01% | -0.10% | 0.34% | 99% | 99% |
| Class F | 0.00% | 0.07% | 0.13% | 99% | 99% |

# Conclusion

The current contribution presents two simplifications for the introduction of MIP blocks. The first simplification removes the MPM list construction for MIP blocks and the second makes the mapping table between MIP and normal intra modes. With these two modifications and their combination, overall merely up to 0.01% coding performance difference is observed compared to VTM5.0, which justifies their benefits.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0207, J. Pfaff et al., 14th JVET meeting, Geneva, CH, 19–27 March 2019.

- “JVET common test conditions and software reference configurations”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, 14th JVET meeting, Geneva, CH, 19–27 March 2019.