| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0177 |
| --- | --- |

| Title: | On HRD Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed to HRD parameters signalling.

Following is proposed:

- Proposal 1: It is proposed to allow signaling of HRD parameters for temporal sub-layers optionally controlled by a flag.

- Proposal 2: It is proposed to conditionally signal syntax elements num_units_in_tick and time_scale based on hrd_parameters_present_flag.

- Proposal 3: It is proposed to signal the syntax elements num_units_in_tick and time_scale with a minus1 coding.

# Introduction

VVC WD [1] includes signalling of HRD parameters and specification of HRD operation.

Following is proposed:

- Proposal 1: It is proposed to allow signaling of HRD parameters for temporal sub-layers optionally controlled by a flag.

- Proposal 2: It is proposed to conditionally signal syntax elements num_units_in_tick and time_scale based on hrd_parameters_present_flag.

- Proposal 3: It is proposed to signal the syntax elements num_units_in_tick and time_scale with a minus1 coding.

# Proposals

## Proposal 1

It is proposed to allow signaling of HRD parameters for temporal sub-layers optionally controlled by a flag.

It is asserted that in current VVC WD where as it is optional to signal HRD CPB parameters (hrd_parameters()), when they are signaled it is required to signal them for all temporal sub-layers. It is proposed to allow signaling HRD CPB parameters such that they can be signalled optionally (as in current VVC WD) but also when they are signaled it is not mandatory to signal them for all the temporal-sub layers. It is proposed that whether HRD CPB parameters are signaled for each temporal sub-layer or only for all the temporal sub-layers is controlled by a flag. In the case the HRD CPB parameters are signalled for all the temporal sub-layers, they are inferred for each temporal sub-layer.

The proposed changes are shown highlighted compared with VVC WD.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| … |  |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| hrd_parameters_present_flag | u(1) |
| if( hrd_parameters_present_flag ) { |  |
| sub_layer_cpb_parameters_present_flag | u(1) |
| sub_layer_cpb_parameters_present_flag? hrd_parameters( 0, sps_max_sub_layers_minus1 ) : hrd_parameters( sps_max_sub_layers_minus1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

hrd_parameters_present_flag equal to 1 specifies that the syntax structure hrd_parameters( ) is present in the SPS RBSP syntax structure. hrd_parameters_present_flag equal to 0 specifies that the syntax structure hrd_parameters( ) is not present in the SPS RBSP syntax structure.

sub_layer_cpb_parameters_present_flag equal to 1 specifies that syntax structure hrd_parameters() includes parameters for temporal sub-layers in the range of 0 to sps_max_sub_layers_minus1, inclusive. sub_layer_cpb_parameters_present_flag equal to 0 specifies that syntax structure hrd_parameters() includes parameters only for sub_layer_hrd_parameters(sps_max_sub_layers_minus) and these parameters apply to all the sub-layers.

    - HRD parameters syntax

| hrd_parameters( startNumSubLayers, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| vui_nal_hrd_parameters_present_flag | u(1) |
| vui_vcl_hrd_parameters_present_flag | u(1) |
| if( vui_nal_hrd_parameters_present_flag ¦ ¦ vui_vcl_hrd_parameters_present_flag ){ |  |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| } |  |
| for( i = 0startNumSubLayers; i <= maxNumSubLayersMinus1; i++ ) { |  |
| fixed_pic_rate_general_flag[ i ] | u(1) |
| if( !fixed_pic_rate_general_flag[ i ] ) |  |
| fixed_pic_rate_within_cvs_flag[ i ] | u(1) |
| if( fixed_pic_rate_within_cvs_flag[ i ] ) |  |
| elemental_duration_in_tc_minus1[ i ] | ue(v) |
| else |  |
| low_delay_hrd_flag[ i ] | u(1) |
| if( !low_delay_hrd_flag[ i ] ) |  |
| vui_cpb_cnt_minus1[ i ] | ue(v) |
| if( vui_nal_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| if( vui_vcl_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| } |  |
| } |  |

    - Sub-layer HRD parameters syntax

| sub_layer_hrd_parameters( subLayerId ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= CpbCnt; i++ ) { |  |
| bit_rate_value_minus1[ i ] | ue(v) |
| cpb_size_value_minus1[ i ] | ue(v) |
| cbr_flag[ i ] | u(1) |
| } |  |
| } |  |

    - Sub-layer HRD parameters semantics

The variable CpbCnt is set equal to vui_cpb_cnt_minus1[ subLayerId ].

bit_rate_value_minus1[ i ] (together with bit_rate_scale) specifies the maximum input bit rate for the i-th CPB when the CPB operates at the access unit level. bit_rate_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. For any i > 0, bit_rate_value_minus1[ i ] shall be greater than bit_rate_value_minus1[ i − 1 ].

The bit rate in bits per second is given by:

BitRate[ i ] = ( bit_rate_value_minus1[ i ] + 1 ) * 2( 6 + bit_rate_scale )	(E-66)

When the bit_rate_value_minus1[ i ] syntax element is not present,

- If bit_rate_value_minus1[ sps_max_sub_layers_minus1 ] is present the value of BitRate[ i ] is inferred to be equal to BitRate[ sps_max_sub_layers_minus1 ].

- Otherwise, the value of BitRate[ i ] is inferred to be equal to CpbBrVclFactor * MaxBR for VCL HRD parameters and to be equal to CpbBrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, CpbBrVclFactor and CpbBrNalFactor are specified in subclause A.4.]

cpb_size_value_minus1[ i ] is used together with cpb_size_scale to specify the i-th CPB size when the CPB operates at the access unit level. cpb_size_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. For any i greater than 0, cpb_size_value_minus1[ i ] shall be less than or equal to cpb_size_value_minus1[ i − 1 ].

The CPB size in bits is given by:

CpbSize[ i ] = ( cpb_size_value_minus1[ i ] + 1 ) * 2( 4 + cpb_size_scale )	(E-67)

When the cpb_size_value_minus1[ i ] syntax element is not present,

- If cpb_size_value_minus1 [ sps_max_sub_layers_minus1 ] is present the value of CpbSize[ i ] is inferred to be equal to CpbSize[ sps_max_sub_layers_minus1 ].

- Otherwise, the value of CpbSize[ i ] is inferred to be equal to CpbBrVclFactor * MaxCPB for VCL HRD parameters and to be equal to CpbBrNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, CpbBrVclFactor and CpbBrNalFactor are specified in subclause A.4.]

cbr_flag[ i ] equal to 0 specifies that to decode this bitstream by the HRD using the i-th CPB specification, the hypothetical stream scheduler (HSS) operates in an intermittent bit rate mode. cbr_flag[ i ] equal to 1 specifies that the HSS operates in a constant bit rate (CBR) mode. When not present,

- If cbr_flag[ sps_max_sub_layers_minus1 ] is present the value of cbr_flag[ i ] is inferred to be equal to cbr_flag[ sps_max_sub_layers_minus1 ].

- Otherwise, the value of cbr_flag[ i ] is inferred to be equal to 0.

## Proposal 2

It is proposed to conditionally signal syntax elements num_units_in_tick and time_scale based on hrd_parameters_present_flag.

It is asserted that in VVC WD the syntax elements num_units_in_tick and time_scale are used to derive a variable ClockTick which is used for HRD operation. As such it is not meaningful to signal these two syntax elements but not signal HRD parameters (hrd_parameters) thus a simplification to the syntax is proposed to conditionally signal these syntax elements based on a single flag.

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| hrd_parameters_present_flag | u(1) |
| if(hrd_parameters_present_flag) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| hrd_parameters_present_flag | u(1) |
| if( hrd_parameters_present_flag ) |  |
| hrd_parameters(sps_max_sub_layers_minus1 ) |  |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

timing_info_present_flag equal to 1 specifies that the syntax elemnts num_units_in_tick, time_scale, and hrd_parameters_present_flag are present in the SPS RBSP syntax structure. timing_info_present_flag equal to 0 specifies that num_units_in_tick, time_scale, and hrd_parameters_present_flag are not present in the SPS RBSP syntax structure.

hrd_parameters_present_flag equal to 1 specifies that the syntax elements num_units_in_tick, time_scale, and syntax structure hrd_parameters( ) is present in the SPS RBSP syntax structure. hrd_parameters_present_flag equal to 0 specifies that the syntax elements num_units_in_tick, time_scale, and syntax structure hrd_parameters( ) is not present in the SPS RBSP syntax structure.

num_units_in_tick is the number of time units of a clock operating at the frequency time_scale Hz that corresponds to one increment (called a clock tick) of a clock tick counter. num_units_in_tick shall be greater than 0. A clock tick, in units of seconds, is equal to the quotient of num_units_in_tick divided by time_scale. For example, when the picture rate of a video signal is 25 Hz, time_scale may be equal to 27 000 000 and num_units_in_tick may be equal to 1 080 000, and consequently a clock tick may be equal to 0.04 seconds.

time_scale is the number of time units that pass in one second. For example, a time coordinate system that measures time using a 27 MHz clock has a time_scale of 27 000 000. The value of time_scale shall be greater than 0.

hrd_parameters_present_flag equal to 1 specifies that the syntax structure hrd_parameters( ) is present in the SPS RBSP syntax structure. hrd_parameters_present_flag equal to 0 specifies that the syntax structure hrd_parameters( ) is not present in the SPS RBSP syntax structure.

vui_parameters_present_flag equal to 1 specifies that the syntax structure vui_parameters( ) is present in the SPS RBSP syntax structure. vui_parameters_present_flag equal to 0 specifies that the syntax structure vui_parameters( ) is not present in the SPS RBSP syntax structure.

sps_extension_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. sps_extension_flag equal to 1 specifies that there are sps_extension_data_flag syntax elements present in the SPS RBSP syntax structure.

sps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_extension_data_flag syntax elements.

## Proposal 3

It is proposed to signal the syntax elements num_units_in_tick and time_scale with a minus1 coding because the value of 0 is not meaningful and is actually not allowed for both these syntax elements.

#### 7.3.2.4 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| num_units_in_tick_minus1 | u(32) |
| time_scale_minus1 | u(32) |
| hrd_parameters_present_flag | u(1) |
| if( hrd_parameters_present_flag ) |  |
| hrd_parameters( sps_max_sub_layers_minus1 ) |  |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |

num_units_in_tick_minus1 plus 1 is the number of time units of a clock operating at the frequency time_scale Hz that corresponds to one increment (called a clock tick) of a clock tick counter. num_units_in_tick shall be greater than 0. A clock tick, in units of seconds, is equal to the quotient of num_units_in_tick divided by time_scale. For example, when the picture rate of a video signal is 25 Hz, time_scale may be equal to 27 000 000 and num_units_in_tick may be equal to 1 080 000, and consequently a clock tick may be equal to 0.04 seconds.

time_scale_minus1 plus 1 is the number of time units that pass in one second. For example, a time coordinate system that measures time using a 27 MHz clock has a time_scale of 27 000 000. The value of time_scale shall be greater than 0.

# Conclusion

This document proposes modifications to HRD parameters signaling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-N1001-v8, Versatile Video Coding (Draft 5), Geneva, June 2019