| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0179-v2 |
| --- | --- |

| Title: | On NAL Unit Header Design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed to NAL unit header design.

It is proposed to redesign the NAL unit header as follows:

- The first byte consists of syntax element nuh_layer_id. For this syntax element, values 188 onwards are forbidden. Value 0 to 126, inclusive are defined to have meaning same as currently in VVC WD for independent layers. Additionally values 127 to 187, inclusive are reserved – to be possibly used for spatial/ view/ SNR scalability later.

- The second byte consists of 5-bit syntax element nal_unit_type, followed by 3-bit syntax element nuh_temporal_id_plus1.

# In v2 an analysis of extensibility is provided and an additional option is included.

# Introduction

It is asserted that the NAL unit header design in VVC WD5 has following limitations:

- The values only in the range of 16-159, inclusive are possible to be used in the first byte of the NAL unit header. Also there are certain restrictions in the NAL unit type assignments related to requiring zero TemporalId values for certain NAL unit types. As a result of these currently there are no reserved IRAP NAL unit types available.

- The derivation of NAL unit type (NalUnitType) requires a computation based on two different syntax elements (zero_tid_required_flag and nal_unit_type_lsb), where as this may not be too difficult, it could be seen as a minor issue for system level and also makes the specification a bit difficult to understand.

Main Proposal:

It is proposed to redesign the NAL unit header as follows:

- The first byte consists of syntax element nuh_layer_id. For this syntax element, values 188 onwards are forbidden (or are reserved). Value 0 to 126, inclusive are defined to have meaning same as currently in VVC WD for independent layers. Additionally values 127 to 187, inclusive are reserved – to be possibly used for spatial/ view/ SNR scalability

- The second byte consists of 5-bit syntax element nal_unit_type, followed by 3-bit syntax element nuh_temporal_id_plus1.

It is asserted that this redesign has following advantages over NAL unit header design in VVC WD5:

- Entire range of values 0-187 in the first byte which is allowed after disallowing MPEG PES start code emulation is used. Thus there is no code space wasted.

- The nal_unit_type field is contiguous and does not require a derivation based on two syntax elements.

Alternative Proposals:

If for some reason the above main proposal for NAL unit header design is not acceptable then the following modifications are proposed to the current NAL unit header as alternative proposals. These proposals do not need to be considered if the above proposal is adopted.

- Proposal 1: It is proposed to change nuh_temporal_id_plus1 to nuh_temporal_id.

- Proposal 2: It is proposed to use entire last byte for nuh_layer_id_plus1.

- Proposal 3: It is proposed to change zero_tid_required_flag to constrained_tid_flag. And allow TemporalID values 0, 1, 2 for NAL unit types RSV_NVCL21, RSV_NVCL22, RSV_NVCL23, UNSPEC28, UNSPEC29, UNSPEC30, UNSPEC31.

# Proposals

## Main Proposal

Option A:

It is proposed to redesign the NAL unit header as follows:

- The first byte consists of syntax element nuh_layer_id. For this syntax element, values 188 onwards are forbidden (or are reserved). Value 0 to 126, inclusive are defined to have meaning same as currently in VVC WD for independent layers. Additionally values 127 to 187, inclusive are reserved – to be possibly used for spatial/ view/ SNR scalability later.

- The second byte consists of 5-bit syntax element nal_unit_type, followed by 3-bit syntax element nuh_temporal_id_plus1.

The proposed changes are shown highlighted compared with VVC WD.

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| nuh_layer_id | u(78) |
| nuh_reserved_zero_bit | u(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

The clean version of the proposed NAL unit header syntax is as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| nuh_layer_id | u(8) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

#### NAL unit header semantics

nuh_layer_id specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id shall be in the range of 0 to 126, inclusive. The values in the range of 127 to 187, are reserved and The value of 127 may be specified in the future by ITU-T | ISO/IEC. The values in the range of 188 to 255, inclusive are forbidden. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the values 127-187, inclusive for nuh_layer_id in a NAL unit.

NOTE 7 – The values of 127-187, inclusive for nuh_layer_id may be used to indicate an extended layer identifier in a future extension of this Specification.

The value of nuh_layer_id shall be the same for all VCL NAL units of a coded picture. The value of nuh_layer_id of a coded picture is the value of the nuh_layer_id of the VCL NAL units of the coded picture.

zero_tid_required_flag equal to 0 specifies that zero_tid_required_flag does not impose any additional constraints on the value of nuh_temporal_id1_plus1.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

The value of nuh_temporal_id_plus1 shall not be equal to 0. When zero_tid_required_flag is equal to 1, the value of nuh_temporal_id_plus1 shall be equal to 1.

When nal_unit_type is in the range of IDR_W_RADL to RSV_IRAP_VCL13, inclusive, except GRA_NUT the coded tile group belongs to an IRAP picture, TemporalId shall be equal to 0.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

NOTE 1 – NAL unit types in the range of 16 to 31, inclusive, have zero_tid_required_flag equal to 1, and consequently have TemporalId equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of a layer access unit. The value of TemporalId of a coded picture or a layer access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the layer access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If NalUnitTypenal_unit_type is equal to SPS_NUT, TemporalId is equal to 0 and the TemporalId of the layer access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if NalUnitTypenal_unit_type is equal to APS_NUT, TemporalId shall be equal to that of the layer access unit containing the NAL unit.

–	Otherwise, when NalUnitTypenal_unit_type is not equal to EOS_NUT and not equal to EOB_NUT, TemporalId shall be greater than or equal to the TemporalId of the layer access unit containing the NAL unit.

NOTE 2 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all layer access units to which the non-VCL NAL unit applies. When NalUnitType is equal to PPS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing layer access unit, as all picture parameter sets (PPSs) may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When NalUnitType is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing layer access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes layer access units for which the TemporalId values are greater than the TemporalId of the layer access unit containing the SEI NAL unit.

nal_unit_type_lsb specifies the least significant bits for the NAL unit type.

The variable NalUnitType, which specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit as specified in Table 7-1, is derived as follows:

NalUnitType = ( zero_tid_required_flag  <<  4 ) + nal_unit_type_lsb	(7-2)

NAL units that have NalUnitType in the range of UNSPEC28..UNSPEC31, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 3 – NAL unit types in the range of UNSPEC28..UNSPEC31 may be used as determined by the application. No decoding process for these values of NalUnitType is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these NalUnitType values, and in the design of decoders that interpret the content of NAL units with these NalUnitType values. This Specification does not define any management for these values. These NalUnitType values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same NalUnitType value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of NalUnitType.

NOTE 4 – This requirement allows future definition of compatible extensions to this Specification.

Table 7-1 – NAL unit type codes and NAL unit type classes

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 1 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 2 3 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 4 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 6 5..7 | RSV_NVCL65.. RSV_NVCL7 | Reserved | non-VCL |
| 8 | TRAIL_NUT | Coded slice of a non-STSA trailing picture slice_layer_rbsp( ) | VCL |
| 9 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 10 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) | VCL |
| 11 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) | VCL |
| 12..15 | RSV_VCL_12.. RSV_VCL_15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 17 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 21..23 | RSV_NVCL21.. RSV_NVCL23 | Reserved | non-VCL |
| 24 25 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 26 | CRA_NUT | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 27 | GRA_NUT | Coded slice of a gradual random access picture slice_layer_rbsp( ) | VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified | non-VCL |

NOTE 5 – A clean random access (CRA) picture may have associated RASL or RADL pictures present in the bitstream. [Ed. (YK): The NOTE index is one greater than the value it should be. Removing of the table would make it correct when updating the fields, but ....]

NOTE 6 – An instantaneous decoding refresh (IDR) picture having NalUnitType equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having NalUnitType equal to IDR_W_RADL does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

<changes shown below for nal_unit_type are compared to M1001-v7:>

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

NAL units that have nal_unit_type in the range of UNSPEC28..UNSPEC31, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 1 – NAL unit types in the range of UNSPEC28..UNSPEC31 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these nal_unit_type values, and in the design of decoders that interpret the content of NAL units with these nal_unit_type values. This Specification does not define any management for these values. These nal_unit_type values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same nal_unit_type value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 2 – This requirement allows future definition of compatible extensions to this Specification.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded tile group of a non- STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2 | RASL_NUT | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 3 | RADL_NUT | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 4..7 | RSV_VCL_4.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 11 | GRA_NUT | Coded slice of a gradual random access picture slice_layer_rbsp( ) | VCL |
| 11 12 13 | RSV_IRAP_VCL11 RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 20 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 21 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 22, 23 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 2425..27 | RSV_NVCL2425.. RSV_NVCL27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified non-VCL NAL unit types | non-VCL |

NOTE 3 – A clean random access (CRA) picture may have associated RASL or RADL pictures present in the bitstream.

NOTE 4 – An instantaneous decoding refresh (IDR) picture having nal_unit_type equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having nal_unit_type equal to IDR_W_RADL does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

nuh_reserved_zero_bit shall be equal to '0'. The value 1 of nuh_reserved_zero_bit may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with nuh_reserved_zero_bit equal to '1'.

An example is provided below showing how the NAL unit header may be extended in future for more layers:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| nuh_layer_id | u(8) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| if(nuh_layer_id == 187) { |  |
| nuh_layer_id_ext | u(x) |
| nuh_reserved_bit | u(1) |
| } |  |
| } |  |

Option B:

In option B following changes are made compared to option A:

- The second byte is unchanged.

- In the first byte two bits are taken from nuh_layer_id and used as follows : First bit is assigned as forbidden_zero_bit, followed by a nuh_reserved_zero_bit, (followed by 6 bits for nuh_layer_id).

The Option B proposed NAL unit header is as follows:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_reserved_zero_bit | u(1) |
| nuh_layer_id | u(6) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

Semantics are as follows with change marks compared to VVC WD5.

forbidden_zero_bit shall be equal to 0.

nuh_reserved_zero_bit shall be equal to '0'. The value 1 of nuh_reserved_zero_bit may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with nuh_reserved_zero_bit equal to '1'.

nuh_layer_id_plus1 minus 1 specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id_plus1 shall be in the range of 10 to 12631, inclusive. The values in the range of 32 to 12763 may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the values 32-12763 for nuh_layer_id_plus1 in a NAL unit.

NOTE 7 – The values of 32-12763for nuh_layer_id_plus1 may be used to indicate an extended layer identifier in a future extension of this Specification.

The variable NuhLayerId is derived as follows:

NuhLayerId = nuh_layer_id_plus1 − 1		(7-3)

The value of NuhLayerIdnuh_layer_id shall be the same for all VCL NAL units of a coded picture. The value of NuhLayerIdnuh_layer_id of a coded picture is the value of the NuhLayerIdnuh_layer_id of the VCL NAL units of the coded picture.

nal_unit_type and nuh_temporal_id_plus1 have the same semantics as in option A above.

## Alternative Proposals

If for some reason the above main proposal for NAL unit header design is not acceptable then the following modifications are proposed to the current NAL unit header as alternative proposals. These proposals do not need to be considered if the above proposal is adopted.

- Proposal 1: It is proposed to change nuh_temporal_id_plus1 to nuh_temporal_id

- Proposal 2: It is proposed to use entire last byte for nuh_layer_id_plus1.

- Proposal 3: It is proposed to change zero_tid_required_flag to constrained_tid_flag. And allow TemporalID values 0, 1, 2 for NAL unit types RSV_NVCL21, RSV_NVCL22, RSV_NVCL23, UNSPEC28, UNSPEC29, UNSPEC30, UNSPEC31.

Proposal 1:

It is proposed to signal syntax element nuh_temporal_id_plus1 instead as syntax element nuh_temporal_id.

The changes corresponding to this aspect are shown highlighted below.

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| … |  |
| nuh_temporal_id_plus1 | u(3) |
| … |  |
| } |  |

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

The value of nuh_temporal_id_plus1 shall not be equal to 0. When zero_tid_required_flag is equal to 1, the value of nuh_temporal_id shall be equal to 0.

In one example: The value of nuh_temporal_id equal to 7 is kept reserved.

When nal_unit_type is in the range of IDR_W_RADL RSV_IRAP_VCL13, inclusive, except GRA_NUT the coded tile group belongs to an IRAP picture, TemporalId shall be equal to 0.

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

NOTE 1 – NAL unit types in the range of 16 to 31, inclusive, have zero_tid_required_flag equal to 1, and consequently have TemporalId equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of a layer access unit. The value of TemporalId of a coded picture or a layer access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the layer access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If NalUnitType is equal to SPS_NUT, TemporalId is equal to 0 and the TemporalId of the layer access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if NalUnitType is equal to APS_NUT, TemporalId shall be equal to that of the layer access unit containing the NAL unit.

–	Otherwise, when NalUnitType is not equal to EOS_NUT and not equal to EOB_NUT, TemporalId shall be greater than or equal to the TemporalId of the layer access unit containing the NAL unit.

NOTE 2 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all layer access units to which the non-VCL NAL unit applies. When NalUnitType is equal to PPS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing layer access unit, as all picture parameter sets (PPSs) may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When NalUnitType is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing layer access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes layer access units for which the TemporalId values are greater than the TemporalId of the layer access unit containing the SEI NAL unit.

Proposal 2:

It is proposed to use entire last byte for nuh_layer_id_plus1.

Since it is anticipated that the nuh_reserved_zero_bit will be used in future for creating a layer ID value, it is proposed to assign that bit to layer ID currently. And keep the remaining bits in it reserved.

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| nuh_layer_id_plus1 | u(78) |
| nuh_reserved_zero_bit | u(1) |
| } |  |

nuh_layer_id_plus1 minus 1 specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id_plus1 shall be in the range of 0 1 to 126, inclusive. The values in the range of 127 to 255, inclusive may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the value 127 to 255, inclusive for nuh_layer_id_plus1 in a NAL unit.

NOTE 7 – The value of 127 for nuh_layer_id may be used to indicate an extended layer identifier in a future extension of this Specification.

The variable NuhLayerID is derived as follows:

NuhLayerId = nuh_layer_id_plus1 − 1

The value of nuh_layer_id_plus1 shall be the same for all VCL NAL units of a coded picture. The value of nuh_layer_id_plus1 of a coded picture is the value of the nuh_layer_id_plus1 of the VCL NAL units of the coded picture.

Proposal 3:

It is proposed to change zero_tid_required_flag to constrained_tid_flag

It is asserted that some of the allowed code values – which could still be used while making sure there is no MPEG2 PES start code emulation are currently wasted by strictly requiring TemporalID to be zero for certain nal_unit_types. Thus it is proposed to change the zero_tid_required_flag to constrained_tid_flag and allow using the unused/ wasted TemporalID values for certain NAL unit types.

Also it is proposed to allow TemporalID values 0, 1, 2 for NAL unit types RSV_NVCL21, RSV_NVCL22, RSV_NVCL23, UNSPEC28, UNSPEC29, UNSPEC30, UNSPEC31.

It is asserted that use of TemporalID values other than 0 should be allowed for the above mentioned NAL units. Since this allows more freedom for use of those NAL unit types in future. It is asserted that allowing those additional TemporalID values still does not result in MPEG2 PES start code emulation.

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flagconstrained_tid_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| nuh_layer_id | u(7) |
| nuh_reserved_zero_bit | u(1) |
| } |  |

zero_tid_required_flagconstrained_tid_flag equal to 0 specifies that zero_tid_required_flagconstrained_tid_flag does not impose any additional constraints on the value of nuh_temporal_id1_plus1.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

The value of nuh_temporal_id_plus1 shall not be equal to 0. When zero_tid_required_flagconstrained_tid_flag is equal to 1, the value of nuh_temporal_id_plus1 shall be equal to 1constrained as follows:

- nuh_temporal_id_plus1 shall be equal to 1 for NAL units with NALUnitType DPS_NUT, SPS_NUT, EOS_NUT, EOB_NUT, VPS_NUT, IDR_W_RADL, IDR_N_LP, CRA_NUT, GRA_NUT

- nuh_temporal_id_plus1 shall be equal to 1 or 2 for NAL units with NALUnitType RSV_NVCL21, RSV_NVCL22, RSV_NVCL23, UNSPEC28, UNSPEC29, UNSPEC30, UNSPEC31

<When used with proposal 1>

zero_tid_required_flagconstrained_tid_flag is equal to 1, the value of nuh_temporal_id shall be equal to 1constrained as follows:

- nuh_temporal_id shall be equal to 0 for NAL units with NALUnitType DPS_NUT, SPS_NUT, EOS_NUT, EOB_NUT, VPS_NUT, IDR_W_RADL, IDR_N_LP, CRA_NUT, GRA_NUT

- nuh_temporal_id shall be equal to 0 or 1 or 2 for NAL units with NALUnitType RSV_NVCL21, RSV_NVCL22, RSV_NVCL23, UNSPEC28, UNSPEC29, UNSPEC30, UNSPEC31

The variable TemporalId is derived as follows:

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

NOTE 1 – NAL unit types in the range of 16 to 31, inclusive, have zero_tid_required_flagconstrained_tid_flag equal to 1, and consequently have TemporalId equal to 0 or 1.

<when used  with proposal 1>

NOTE 1 – NAL unit types in the range of 16 to 31, inclusive, have zero_tid_required_flagconstrained_tid_flag equal to 1, and consequently have TemporalId equal to 0 or 1 or 2.

The value of TemporalId shall be the same for all VCL NAL units of a layer access unit. The value of TemporalId of a coded picture or a layer access unit is the value of the TemporalId of the VCL NAL units of the coded picture or the layer access unit. [Ed. (YK): Check whether all layer AUs in an AU should be required to have the same value of TemporalId.]

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If NalUnitType is equal to SPS_NUT, TemporalId is equal to 0 and the TemporalId of the layer access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if NalUnitType is equal to APS_NUT, TemporalId shall be equal to that of the layer access unit containing the NAL unit.

–	Otherwise, when NalUnitType is not equal to EOS_NUT and not equal to EOB_NUT, TemporalId shall be greater than or equal to the TemporalId of the layer access unit containing the NAL unit.

NOTE 2 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all layer access units to which the non-VCL NAL unit applies. When NalUnitType is equal to PPS_NUT, TemporalId may be greater than or equal to the TemporalId of the containing layer access unit, as all picture parameter sets (PPSs) may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When NalUnitType is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing layer access unit, as an SEI NAL unit may contain information that applies to a bitstream subset that includes layer access units for which the TemporalId values are greater than the TemporalId of the layer access unit containing the SEI NAL unit.

nal_unit_type_lsb specifies the least significant bits for the NAL unit type.

The variable NalUnitType, which specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit as specified in Table 7-1, is derived as follows:

NalUnitType = ( zero_tid_required_flagconstrained_tid_flag  <<  4 ) + nal_unit_type_lsb	(7-2)

# Conclusion

This document proposes modifications to NAL unit header design for VVC. It is proposed to adopt the main proposal in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-N1001-v8, Versatile Video Coding (Draft 5), Geneva, June 2019