| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0184 |
| --- | --- |

| Title: | AHG8: On adaptive resolution change constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yuwen He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to signal constraint flag in SPS and slice header to constrain certain coding tools for ARC/RPR.

# Introduction

Adaptive resolution change (ARC) or reference picture resampling (RPR) approaches were studied in 13th and 14th JVET meeting for real-time communications, adaptive streaming, 360-degree viewport dependent streaming and rendering. High level syntax signaling, resampling filters, simulation results and implementation considerations have been proposed and discussed. An initial approach was agreed in principle to use “a range of a factor 4 in each dimension” and “have the resolution of the stored reference picture being different from that of the current picture”.

JVET-N0279 proposed following restrictions due to complexity and memory bandwidth concerns.

- When tile_group_temporal_mvp_enabled_flag is equal to 1, the current picture and its collocated picture shall have the same size.

- When resolution change is allowed within a sequence, decoder motion vector refinement shall be turned off.

- When resolution change is allowed within a sequence, sps_bdof_enabled_flag shall be equal to 0.

Besides complexity and bandwidth consideration, the pictures after resolution change may use a scaled picture as reference, the motion compensation error may impair the performance of certain coding tools that rely on accurate temporal information and reference samples. For example, the bi-directional optical flow (BDOF) needs to derive the delta motion vector for each 4x4 sub-block, such derivation is based on temporal prediction signal using the difference between two temporal predictions from forward and backward prediction and spatial gradients. However, both temporal difference and spatial gradients are changed after spatial scaling of reference pictures after ARC/RPR.

The decoder side motion vector refinement (DMVR) derives the delta motion vector for each sub-block (e.g. 16x16) inside the coding unit using two temporal predictions from two reference pictures, the derivation results would be changed when temporal reference pictures are scaled.

The ARC/RPR may happen at encoder side for applications like video conferencing, or at decoder side for the applications like viewport dependent streaming. In Figure 1, each viewport may be encoded into high resolution and low-resolution versions, high resolution viewport frame is always predicted from the same resolution frame, and low-resolution viewport frame is always predicted from the same resolution frame at encoder side. high resolution viewport is fetched to match the viewing orientation while low resolution viewports are fetched to cover rest 360-degree video. When a viewing orientation change is detected, viewport switching latency can be minimized if such switching can happen instantly on non-IRAP frame.

Figure 1 Viewport-dependent streaming ARC/RPR example

Disabling those coding tools for all frames at encoder side addresses the potential decoder-side ARC/RPR switching but may compromise coding performance for the entire sequence. When ARC/RPR happens at decoder side operation on a pre-encoded stream, the encoder cannot foresee ARC/RPR and apply the constraints in advance. A signaling in the bitstream may guide the ARC/RPR switching at decoder side.

This contribution proposes an ARC/RPR constraint flag in SPS and slice header to indicate potential ARC/RPR switching point where the relevant coding constraints are applied.

# Proposal

A sps_arc_constraint_flag is proposed to signal in SPS as shown in Table 1.

Table 1 proposed SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| …. | ue(v) |
| sps_arc_constraint_flag | u(1) |
| timing_info_present_flag | u(1) |
| …. |  |
| rbsp_trailing_bits( ) |  |
| } |  |

sps_arc_constraint_flag equal to 1 specifies that slice_temporal_mvp_enabled_flag, bdofFlag and dmvrFlag shall be equal to 0. slice_arc_constraint_flag equal to 0 does not impose a constraint.

A flag slice_arc_constraint_flag is proposed to signal in slice header as shown in Table 2.

Table 2 Proposed slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| slice_pic_order_cnt_lsb | u(v) |
| if ( !sps_arc_constraint_flag ) |  |
| slice_arc_constraint_flag | u(1) |
| … |  |
| if ( slice_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag && !slice_arc_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| … |  |

slice_arc_constraint_flag equal to 1 specifies that slice_temporal_mvp_enabled_flag, bdofFlag and dmvrFlag shall be equal to 0. slice_arc_constraint_flag equal to 0 does not impose a constraint. when slice_arc_constraint_flag is not present, it is inferred to be equal to 1.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).