| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0186 |
| --- | --- |

| Title: | AHG15: Signalling of chroma Qp mapping table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Roman Chernyak Elena Alshina | Tel: Email: | +7-906-035-18-23 sergey.ikonin@huawei.com chernyak.roman@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution proposes signaling mechanism for chroma Qp mapping table. It is also reported that usage of adjusted mapping table provides additional benefits in terms of coding performance and coding gain balancing across luma and chroma components. Informal subjective evaluation results are also reported demonstrating no noticeable difference for moving pictures and slight improvement for static pictures. No color issues or artifacts are detected.

# Introduction

In VVC 5 specification draft [1] chroma quantization parameter (QpC) is derived based on luma Qp using luma-to-chroma Qp mapping table:

Table 8 15 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

The table was inherited from HEVC and was mostly designed for SDR content. In contribution JVET-N0221 [2] it was proposed to introduce HDR specific table for chroma quantization parameter QpC. Another several proposals were targeted to redesign mapping table for SDR (JVET-K0252 [3], JVET- N0341 [4]) and demonstrated BD rate improvement. During meeting discussion in Geneva it was suggested to consider if it might be possible to signal such a table at some higher level.

This contribution proposes signalling mechanism for chroma Qp mapping table to enable adjustment of chroma quantization process addressing to specific properties of video content.

Since chroma compression efficiency has been significantly improved in VVC due to including chroma dedicated coding tools like chroma separate tree and CCLM, the chroma Qp mapping function may require adjustment. As a second aspect of contribution we demonstrate BD rate performance with adjusted mapping table.

# Proposal

The Figure 1 below demonstrates current VVC chroma Qp mapping table as a function of qPi derived based on luma Qp.

Figure 1 Current VVC chroma Qp mapping function

It can be seen that the luma-to-chroma mapping function is monotonically increasing (non-decreasing) function which can be divided on regions of two classes. Class A regions, where function is non-increasing (or flat) i.e. f(x) – f(x-1) = 0, and class B regions, where function is increasing, i.e. f(x) – f(x-1) = 1.

In method of signalling we propose to signal in the bitstream the points of class A (flat regions) using differential representation (e.g. cqp_delta_fpi = A[i] – A[i-1]). According to the proposed method decoder constructs mapping function using information about points of set A. For rest points of allowable Qp range the mapping function is assumed to be monotonically increased with step 1. For example, to reproduce current VVC mapping function following points need to be signalled: 30, 35, 37, 39, 41 and 43.

## Syntax and semantics

In SPS, we propose to add a new syntax element chroma_qp_mapping_flag. When the value of chroma_qp_mapping_flag is equal to 0 the default chroma Qp mapping table is used. When the value of chroma_qp_mapping_flag is equal to 1, a chroma Qp mapping table is signaled in a following way:

#### Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| chroma_qp_mapping_flag | u(1) |
| if( chroma_qp_mapping_flag ) |  |
| cqp_flat_points_minus1 | ue(v) |
| cqp_fp0 | ue(v) |
| for ( i = 1; i <= cqp_flat_points_minus1; i++ ) { |  |
| cqp_delta_fp_minus1[ i ] | ue(v) |
| … |  |
| } |  |

Following semantics are proposed to derive chroma Qp mapping table based on the signaled parameters:

chroma_qp_mapping_flag equal to 1 specifies that chroma Qp mapping table is signalled and overrides Table 8-15  by cqpMappingTable which is used to derive QpC. chroma_qp_mapping_flag equal to 0 specifies that default chroma Qp mapping table specified in is used to derive QpC. When chroma_qp_mapping_flag is not present, it is inferred to be equal to 0.

cqp_flat_points_minus1 plus 1 specifies the number of points of where mapping function is non-increasing.

cqp_fp0 specifies the fist element of set of points where mapping function is non-increasing

cqp_delta_fp_minus1[ i ] plus 1 specifies the delta value between i-th and (i-1)-th element of set of points where mapping function is non-increasing.

The variable cQpFlatSize is derived as follows:

cQpFlatSize = cqp_flat_points_minus1 + 1

The variable cQpFlat[] is derived as follows:

cQpFlat[ 0 ] = cqp_fp0;
for( i = 1; i < cQpFlatSize; i++ ) {
	cQpFlat[ i ] = cqp_delta_fp_minus1[ i ] + 1 + cQpFlat[ i - 1 ]
}

The chroma QP mapping table cqpMappingTable[] is derived as follows:

cqpMappingTable [ 0 ] = 0;
for( i = 1; i <= maxQP; i++ ) {
	incStep = 1
	for ( j = 0; j < cQpFlatSize; j++){
		if ( i = = cQpFlat[ j ] ) 
			incStep = 0
	}
	cqpMappingTable [ i ] = cqpMappingTable [ i – 1 ] + incStep;
}

## Adjusted chroma Qp mapping function

As a second aspect of the proposal we evaluated an adjusted mapping function having non-increasing regions at point 35, 39, 43. The adjusted mapping function is depicted on Figure 2.

Figure 2 Current and adjusted chroma Qp mapping function

The adjustment of mapping table can be enabled in cfg files by specifying non-increasing points:

CQPMap		: 1
	CQPMapPoints	: 35, 39, 43

Another option leading to same result is to update default chroma Qp mapping table.

## Simulation results

Following results were obtained using adjusted chroma Qp mapping function with flat regions at points 35, 39 and 43. The mapping table was adjusted using test configuration files using proposed signalling mechanism. In this experiment we keep chroma QP Offset equal to 1 for AI configuration only. For configurations RA, LDB and LDP the chroma QP Offset is set to 0.

Table 1 Coding performance of adjusted mapping table over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Proposal Over HM16.20 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.17% | 6.17% | 4.96% | 98% | 100% |
| Class A2 | -1.69% | 5.20% | 5.14% | 97% | 99% |
| Class B | -0.63% | 6.05% | 6.77% | 99% | 101% |
| Class C | -0.69% | 5.78% | 6.00% | 99% | 101% |
| Class E | -0.62% | 5.87% | 6.34% | 100% | 101% |
| Overall | -0.91% | 5.84% | 5.95% | 99% | 100% |
| Class D | -0.67% | 5.78% | 6.16% | 99% | 99% |
| Class F (optional) | -0.75% | 4.22% | 4.24% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Proposal Over HM16.20 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.34% | 2.56% | -0.45% | 98% | 100% |
| Class A2 | -1.18% | -2.76% | -2.97% | 98% | 100% |
| Class B | -0.21% | -3.75% | -3.11% | 99% | 100% |
| Class C | -0.38% | -1.69% | -0.76% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.88% | -1.74% | -1.92% | 99% | 101% |
| Class D | -0.23% | -2.44% | -1.47% | 100% | 101% |
| Class F (optional) | -0.20% | -2.26% | -1.65% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Proposal Over HM16.20 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -8.04% | -7.29% | 100% | 101% |
| Class C | 0.03% | -4.25% | -3.66% | 100% | 99% |
| Class E | 0.16% | -7.41% | -6.86% | 99% | 101% |
| Overall | 0.07% | -6.62% | -5.97% | 100% | 100% |
| Class D | 0.05% | -6.00% | -4.16% | 101% | 104% |
| Class F (optional) | -0.04% | -3.20% | -2.48% | 99% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Proposal Over HM16.20 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -8.18% | -7.40% | 99% | 100% |
| Class C | -0.07% | -4.69% | -3.63% | 99% | 100% |
| Class E | 0.08% | -7.54% | -8.13% | 100% | 101% |
| Overall | 0.02% | -6.86% | -6.33% | 99% | 100% |
| Class D | 0.13% | -6.53% | -5.06% | 100% | 101% |
| Class F (optional) | 0.01% | -2.69% | -2.81% | 100% | 100% |

It was noted that usage of VTM5.0 as an anchor leads crossing of RD curves for some sequences. That may make BD-rate numbers not relevant. To obtain correct numbers we estimate performance over VTM5.0 by taking difference in performance of VTM5.0 and test with adjusted mapping table over HM. On Table 2 below left part denotes the BD-rate difference with VTM5.0, middle part reports coding performance of adjusted table over HM, and right part is coding performance of VTM5.0 over HM given for comparison.

Table 2 Difference in coding performance of adjusted mapping table over VTM5.0 having HM as an anchor

# Subjective evaluation

Informal subjective evaluation was performed by proponents using sequences which have biggest difference in BD-rate performance. Slight subjective difference was observed if look at static pictures. At the same time looking at videos the difference can hardly be detected. No color issues or artifacts were observed during subjective evaluations. Some examples taken from coded videos are presented below.

| a) source | b) VTM5.0 | c) test |
| --- | --- | --- |

Figure 3 BQTerrace, RA, QP 32, #233, BD-rate: 0.13%/-12.66%/-12.52% (Y/Cb/Cr)

| a) source | b) VTM5.0 | c) test |
| --- | --- | --- |

Figure 4 CampfireParty2, RA, QP 37, #80, BD-rate: -5.96%/10.13%,/-1.41% (Y/Cb/Cr)

| a) source | b) VTM5.0 | c) test |
| --- | --- | --- |

Figure 5 ParkRunning3, RA, QP 37, #71, BD-rate: -2.85%/2.71%/2.80% (Y/Cb/Cr)

| a) source | b) VTM5.0 | c) test |
| --- | --- | --- |

Figure 6 CampfireParty2, AI, QP 37, #176, BD-rate: -1.64%/5.83%/3.52% (Y/Cb/Cr)

# Conclusion

This contribution proposes signalling mechanism for chroma Qp mapping table. In addition, simulation results with adjusted mapping table are reported. Using of adjusted table provides coding gain of 0.74%/0.13%/0.16% (Y/Cb/Cr) in RA configuration over VTM5.0 and reported more balanced VTM5.0 coding gain over HM across luma and chroma components, -23.90%/-22.43%/-24.96%, -33.88%/-35.35%/-36.61%, -24.69%/-25.18%/-27.13% and -28.19%/-27.87%/-29.70% (Y/Cb/Cr) in AI, RA, LDB and LDP configuration, respectively.

It is proposed to include signalling mechanism into next release of VVC; enable usage of adjusted chroma QP mapping table either via configuration scripts or via updating default mapping table; and keep chroma QP offset for AI configuration only in CTC.

# References

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, 19–27 March 2019

- T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen “Chroma Quantization Parameter QpC Table for HDR Signal”, JVET-N0221, Geneva, CH, 19–27 March 2019

- S. Hsiang, S. Lei “CE7.3.3: Derivation of chroma QP from luma QP”, JVET-K0252, Ljubljana, SI, 10–18 July 2018

- F. Bossen “On reporting combined YUV BD rates”, JVET- N0341, Geneva, CH, 19–27 March 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).