| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0186-v2 |
| --- | --- |

| Title: | AHG15: Signalling of chroma Qp mapping table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Roman Chernyak Timofey Solovyev Elena Alshina | Tel: Email: | +7-906-035-18-23 sergey.ikonin@huawei.com chernyak.roman@huawei.com solovyev.timofey@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution proposes signaling mechanism for chroma Qp mapping table. It is also reported that usage of adjusted mapping table provides additional benefits in terms of coding performance and coding gain balancing across luma and chroma components. Informal subjective evaluation results are also reported demonstrating no noticeable difference for moving pictures and slight improvement for static pictures. No color issues or artifacts are detected.

In a second revision of the document we modified signalling mechanism for chroma Qp mapping table to address the issues pointed during meeting discussion.

# Introduction

In VVC 5 specification draft [1] chroma quantization parameter (QpC) is derived based on luma Qp using luma-to-chroma Qp mapping table:

Table 8 15 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

The table was inherited from HEVC and was mostly designed for SDR content. In contribution JVET-N0221 [2] it was proposed to introduce HDR specific table for chroma quantization parameter QpC. Another several proposals were targeted to redesign mapping table for SDR (JVET-K0252 [3], JVET- N0341 [4]) and demonstrated BD rate improvement. During meeting discussion in Geneva it was suggested to consider if it might be possible to signal such a table at some higher level.

This contribution proposes signalling mechanism for chroma Qp mapping table to enable adjustment of chroma quantization process addressing to specific properties of video content.

Since chroma compression efficiency has been significantly improved in VVC due to including chroma dedicated coding tools like chroma separate tree and CCLM, the chroma Qp mapping function may require adjustment. As a second aspect of contribution we demonstrate BD rate performance with adjusted mapping table.

During the discussion on chroma QP mapping table signalling in Track A following recommendations on general signalling concept were given:

- Separate mapping tables for Cb and Cr – yes, should support this (also supporting a common table)

- SPS, PPS, or APS – SPS

- Default(s) – no default

- Additional offsets prior to or after the mapping table (currently we have PPS and slice level offsets before the mapping) – PPS and slice level, offsets applied after the mapping table (deblocking continues to use only the picture level offset)

- Joint chroma coding handling – use a third table if not using a common table, and support a third offset at PPS and slice level (similar to what we’re doing now at the PPS and slice level)

And the following items were identified as open:

- Degree of generality needed for the mapping table (slope constraints other than non-negative, and starting point constraints) – it was commented that we seem to have some use for a general non-negative slope, where we want slope of 1 at very low QP, a somewhat higher QP for chroma in some middle range, but then a somewhat lower QP for chroma at very high QP values – we prefer to have a general syntax, O0562 actually supports negative slopes, O0650 supports arbitrary non-negative slopes, so it was suggested to consider that syntax as a candidate for adoption; revisit after offline study.

- Syntax & semantics details (amount of bits, number of syntax elements, text description)

- What to do in CTC

Basic design of chroma mapping table proposed in a first revision of contribution does not satisfy to all issues listed above, however it can be easily extended to support all of them. The a second revision proposes such design.

# Proposal

The Figure 1 below demonstrates current VVC chroma Qp mapping table as a function of qPi derived based on luma Qp.

Figure 1 Current VVC chroma Qp mapping function

It can be seen that the luma-to-chroma mapping function is monotonically increasing (non-decreasing) function which can be divided on regions of two classes. Class A regions, where function is non-increasing (or flat) i.e. f(x) – f(x-1) = 0, and class B regions, where function is increasing, i.e. f(x) – f(x-1) = 1.

In method of signalling we propose to signal in the bitstream the points of class A (flat regions) using differential representation (e.g. cqp_delta_fpi = A[i] – A[i-1]). According to the proposed method decoder constructs mapping function using information about points of set A. For rest points of allowable Qp range the mapping function is assumed to be monotonically increased with step 1. For example, to reproduce current VVC mapping function following points need to be signalled: 30, 35, 37, 39, 41 and 43.

## Syntax and semantics

#### Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| same_сqp_table | u(1) |
| for( n = 0; n < same_qp_table_for_chroma ? 1 : 3; n++ ) { |  |
| cqp_set_num_m1[ n ] | ue(v) |
| for ( k = 0; k <= cqp_set_num_m1[ n ]; k++ ) { |  |
| cqp_set_inc[ n ][ k ] | ue(v) |
| cqp_set_size_m1[ n ][ k ] | ue(v) |
| for ( i = 0; i <= cqp_set_size_m1[ n ][ k ]; i++ ) |  |
| cqp_set_delta_m1[ n ][ k ][ i ] | ue(v) |
| } |  |
| } |  |

## Semantics: option 2, table-based

Following semantics are proposed to derive chroma Qp mapping table based on the signaled parameters:

same_сqp_table equal to 1 specifies that only one chroma QP mapping table is signalled and applies to both Cb and Cr components and joint Cb-Cr coding. same_сqp_table equal to 0 specifies that three chroma QP mapping tables are signalled in the SPS.

cqp_set_num_m1[ n ] plus 1 specifies number of sets of points on which n-th chroma Qp mapping function has delta value other than 1.

cqp_set_inc[ n ][ k ] specifies the chroma Qp mapping function increment at points of k-th set, and restricted not to be 1.

cqp_set_size_m1[ n ][ k ] plus 1 specifies the number of points of the set.

cqp_set_delta_m1[ n ][ k ][ i ] plus 1 specifies i-th element of the set if i equals 0, and delta value between i-th and (i-1)-th element otherwise.

The array cqp_set_point[ n ][ k ][ i ] specifies sets of points at which n-th chroma Qp mapping table has delta value other than 1, and is derived as follows:

cqp_set_point[ n ][ k ][ i ] = cqp_set_delta_m1[ n ][ k ][ i ] + 1 + ( i > 0 ? : cqp_set_point[ n ][ k ][ i - 1 ] : 0 )

The n-th chroma QP mapping table cqpMappingTable[ i ] for n = 0..same_qp_table_for_chroma ? 0 : 2 is derived as follows:

cqpMappingTable[ n ][ 0 ] = 0
cqpMappingTable[ n ][ i ] = cqpMappingTable[ n ][ i – 1 ] + incStep, with i = 1..63, where incStep is initialized to 1 and modifyed as follows, for k = 0..cqp_set_num_m1[ n ] and j = 0..cqp_set_size_m1[ n ][ k ]:

  - If ( i = = cqp_set_point[ n ][ k ][ j ] ) incStep = cqp_set_inc[ n ][ k ]

## Semantics: option 2, table-less

For some implementation it maybe be beneficial to avoid storing of entire mapping table to save memory. To achieve that derivation process for certain chroma Qp value is provided. Syntax elements and semantics for it the same as option 1, but maintaining of cqpMappingTable is not necessary:

same_сqp_table equal to 1 specifies that only one chroma QP mapping table is signalled and applies to both Cb and Cr components and joint Cb-Cr coding. same_сqp_table equal to 0 specifies that three chroma QP mapping tables are signalled in the SPS.

cqp_set_num_m1[ n ] plus 1 specifies number of sets of points on which n-th chroma Qp mapping function has delta value other than 1.

cqp_set_inc[ n ][ k ] specifies the chroma Qp mapping function increment at points of k-th set, and restricted not to be 1.

cqp_set_size_m1[ n ][ k ] plus 1 specifies the number of points of the set.

cqp_set_delta_m1[ n ][ k ][ i ] plus 1 specifies i-th element of the set if i equals 0, and delta value between i-th and (i-1)-th element otherwise.

The array cqp_set_point[ n ][ k ][ i ] specifies sets of points at which n-th chroma Qp mapping table has delta value other than 1, and is derived as follows:

cqp_set_point[ n ][ k ][ i ] = cqp_set_delta_m1[ n ][ k ][ i ] + 1 + ( i > 0 ? : cqp_set_point[ n ][ k ][ i - 1 ] : 0 )

## Derivation process

When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The array QpMapOffset[ n ], with n = 0..2 is initialized with 0. If ChromaArrayType is equal to 1, the QpMapOffset modifyed as follows, for k = 0..cqp_set_num_m1[ n ] and i = 0..cqp_set_size_m1[ n ][ k ]:

–	QpMapOffset[ n ] = QpMapOffset[ n ] + (cqp_set_point[ n ][ k ][ i ] <= QpY? cqp_set_inc[ n ][ k ] - 1 : 0 ).

–	The variables QpOffsetCb, QpOffsetCr and QpOffsetCbCr are derived as follows:

QpOffsetCb = QpMapOffset[0] + pps_cb_qp_offset + slice_cb_qp_offset

QpOffsetCr = QpMapOffset[1] + pps_cr_qp_offset + slice_cr_qp_offset

QpOffsetCbCr = QpMapOffset[2] + pps_cbcr_qp_offset + slice_cbcr_qp_offset

–	The variables qPCb, qPCr and qPCbCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset )	(8-928)

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset )	(8-929)

qPiCbCr = Clip3( −QpBdOffsetC, 69, QpY + pps_joint_cbcr_qp_offset + slice_joint_cbcr_qp_offset )	(8-930)

–	If ChromaArrayType is equal to 1, the variables qPCb, qPCr and qPCbCr are set equal to the value of QpC as specified in Table 8-15 based on the index qPi equal to qPiCb, qPiCr and qPiCbCr, respectively.

–	Otherwise, the variables qPCb, qPCr and qPCbCr are set equal to Min( qPi, 63 ), based on the index qPi equal to qPiCb, qPiCr and qPiCbCr, respectively.

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

Qp′Cb = Clip3( −QpBdOffsetC, 63, QpY + QpOffsetCb )+ QpBdOffsetC 	(8-931)

Qp′Cr = Clip3( −QpBdOffsetC, 63, QpY + QpOffsetCr ) + QpBdOffsetC	(8-932)

Qp′CbCr =  Clip3( −QpBdOffsetC, 63, QpY + QpOffsetCbCr )+ QpBdOffsetC	(8-933)

Table 8-15 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

## Bit amount estimation.

The amount of bits required for mapping function coding was estimated for following mapping function:

Figure 2 Current VVC chroma Qp mapping function

| Candidate default mapping tables | Bit amount |
| --- | --- |
| SDR (VTM5.0) | 33 |
| SDR (JVET-O0186) | 26 |
| HDR PQ (QpC_709Cr from JVET-N0221) | 59 |
| HDR HLG (JVET-O0433) | 55 |
| SDR (VTM5.0) + chroma QP increments | 49 |

## Syntax modification

To further optimize amount of bits required typical mapping tables coding it is proposed to modify the coding method for syntax elements in a following way:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| same_сqp_table | u(1) |
| for( n = 0; n < same_qp_table_for_chroma ? 1 : 3; n++ ) { |  |
| cqp_set_num_m1[ n ] | ue(v) |
| for ( k = 0; k <= cqp_set_num_m1[ n ]; k++ ) { |  |
| cqp_set_inc[ n ][ k ] | ue(v) |
| cqp_set_size_m1[ n ][ k ] | uek(v) |
| for ( i = 0; i <= cqp_set_size_m1[ n ][ k ]; i++ ) |  |
| cqp_set_delta_m1[ n ][ k ][ i ] | uek(v) |
| } |  |
| } |  |

cqp_set_size_m1[ n ][ k ] plus 1 specifies the number of points of the set. The order k of the exp-Golomb binarization uek(v) is 3.

cqp_set_delta_m1[ n ][ k ][ i ] plus 1 specifies i-th element of the set if i equals 0, and delta value between i-th and (i-1)-th element otherwise. The order k of the exp-Golomb binarization uek(v) is 3 for j = 0, and 0 otherwise.

With proposed modifications the following estimations for amount of bits were obtained:

| Candidate default mapping tables | Bit amount |
| --- | --- |
| SDR (VTM5.0) | 26 |
| SDR (JVET-O0186) | 22 |
| HDR PQ (QpC_709Cr from JVET-N0221) | 54 |
| HDR HLG (JVET-O0433) | 48 |
| SDR (VTM5.0) + chroma QP increments | 43 |

## Adjusted chroma Qp mapping function

As a second aspect of the proposal we evaluated an adjusted mapping function having non-increasing regions at point 35, 39, 43. The adjusted mapping function is depicted on Figure 2.

Figure 2 Current and adjusted chroma Qp mapping function

The adjustment of mapping table can be enabled in cfg files by specifying non-increasing points:

CQPMap		: 1
	CQPMapPoints	: 35, 39, 43

Another option leading to same result is to update default chroma Qp mapping table.

## Simulation results

Following results were obtained using adjusted chroma Qp mapping function with flat regions at points 35, 39 and 43. The mapping table was adjusted using test configuration files using proposed signalling mechanism. In this experiment we keep chroma QP Offset equal to 1 for AI configuration only. For configurations RA, LDB and LDP the chroma QP Offset is set to 0.

Table 1 Coding performance of adjusted mapping table over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Proposal Over HM16.20 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.17% | 6.17% | 4.96% | 98% | 100% |
| Class A2 | -1.69% | 5.20% | 5.14% | 97% | 99% |
| Class B | -0.63% | 6.05% | 6.77% | 99% | 101% |
| Class C | -0.69% | 5.78% | 6.00% | 99% | 101% |
| Class E | -0.62% | 5.87% | 6.34% | 100% | 101% |
| Overall | -0.91% | 5.84% | 5.95% | 99% | 100% |
| Class D | -0.67% | 5.78% | 6.16% | 99% | 99% |
| Class F (optional) | -0.75% | 4.22% | 4.24% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Proposal Over HM16.20 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.34% | 2.56% | -0.45% | 98% | 100% |
| Class A2 | -1.18% | -2.76% | -2.97% | 98% | 100% |
| Class B | -0.21% | -3.75% | -3.11% | 99% | 100% |
| Class C | -0.38% | -1.69% | -0.76% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.88% | -1.74% | -1.92% | 99% | 101% |
| Class D | -0.23% | -2.44% | -1.47% | 100% | 101% |
| Class F (optional) | -0.20% | -2.26% | -1.65% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Proposal Over HM16.20 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -8.04% | -7.29% | 100% | 101% |
| Class C | 0.03% | -4.25% | -3.66% | 100% | 99% |
| Class E | 0.16% | -7.41% | -6.86% | 99% | 101% |
| Overall | 0.07% | -6.62% | -5.97% | 100% | 100% |
| Class D | 0.05% | -6.00% | -4.16% | 101% | 104% |
| Class F (optional) | -0.04% | -3.20% | -2.48% | 99% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Proposal Over HM16.20 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -8.18% | -7.40% | 99% | 100% |
| Class C | -0.07% | -4.69% | -3.63% | 99% | 100% |
| Class E | 0.08% | -7.54% | -8.13% | 100% | 101% |
| Overall | 0.02% | -6.86% | -6.33% | 99% | 100% |
| Class D | 0.13% | -6.53% | -5.06% | 100% | 101% |
| Class F (optional) | 0.01% | -2.69% | -2.81% | 100% | 100% |

It was noted that usage of VTM5.0 as an anchor leads crossing of RD curves for some sequences. That may make BD-rate numbers not relevant. To obtain correct numbers we estimate performance over VTM5.0 by taking difference in performance of VTM5.0 and test with adjusted mapping table over HM. On Table 2 below left part denotes the BD-rate difference with VTM5.0, middle part reports coding performance of adjusted table over HM, and right part is coding performance of VTM5.0 over HM given for comparison.

Table 2 Difference in coding performance of adjusted mapping table over VTM5.0 having HM as an anchor

# Subjective evaluation

Informal subjective evaluation was performed by proponents using sequences which have biggest difference in BD-rate performance. Slight subjective difference was observed if look at static pictures. At the same time looking at videos the difference can hardly be detected. No color issues or artifacts were observed during subjective evaluations. Some examples taken from coded videos are presented below.

| a) source | b) VTM5.0 | c) test |
| --- | --- | --- |

Figure 3 BQTerrace, RA, QP 32, #233, BD-rate: 0.13%/-12.66%/-12.52% (Y/Cb/Cr)

| a) source | b) VTM5.0 | c) test |
| --- | --- | --- |

Figure 4 CampfireParty2, RA, QP 37, #80, BD-rate: -5.96%/10.13%,/-1.41% (Y/Cb/Cr)

| a) source | b) VTM5.0 | c) test |
| --- | --- | --- |

Figure 5 ParkRunning3, RA, QP 37, #71, BD-rate: -2.85%/2.71%/2.80% (Y/Cb/Cr)

| a) source | b) VTM5.0 | c) test |
| --- | --- | --- |

Figure 6 CampfireParty2, AI, QP 37, #176, BD-rate: -1.64%/5.83%/3.52% (Y/Cb/Cr)

# Conclusion

This contribution proposes signalling mechanism for chroma Qp mapping table. In addition, simulation results with adjusted mapping table are reported. Using of adjusted table provides coding gain of 0.74%/0.13%/0.16% (Y/Cb/Cr) in RA configuration over VTM5.0 and reported more balanced VTM5.0 coding gain over HM across luma and chroma components, -23.90%/-22.43%/-24.96%, -33.88%/-35.35%/-36.61%, -24.69%/-25.18%/-27.13% and -28.19%/-27.87%/-29.70% (Y/Cb/Cr) in AI, RA, LDB and LDP configuration, respectively.

It is proposed to include signalling mechanism into next release of VVC; enable usage of adjusted chroma QP mapping table either via configuration scripts or via updating default mapping table; and keep chroma QP offset for AI configuration only in CTC.

# References

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, 19–27 March 2019

- T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen “Chroma Quantization Parameter QpC Table for HDR Signal”, JVET-N0221, Geneva, CH, 19–27 March 2019

- S. Hsiang, S. Lei “CE7.3.3: Derivation of chroma QP from luma QP”, JVET-K0252, Ljubljana, SI, 10–18 July 2018

- F. Bossen “On reporting combined YUV BD rates”, JVET- N0341, Geneva, CH, 19–27 March 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).