| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0188 |
| --- | --- |

| Title: | Non-CE5: Non-linear ALF simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Victor Stepin Elena Alshina | Tel: Email: | +7-906-035-18-23 sergey.ikonin@huawei.com stepin.victor@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes a simplifications of the non-linear adaptive loop filter adopted to VTM5.0 where adaptive clipping operation is applied during the filtering process. For the worst case scenario non-linear adaptive loop filter requires 12 multiplications of 8-bit filter coefficient by (BitDepth+3)-bit value equals to sum of clipped difference between filtering sample and neighboring sample. The proposed method modifies formula of clipping value calculation and applies secondary clipping to guarantee bit depth of multiplication operand after summation. The method allows to save 2 bits of multiplication bit depth from 8x(BitDepth+3) to 8x(BitDepth+1) for each of 12 multiplications in the worst case scenario. Additionally using the same clipping level equation for luma and chroma components is proposed to unify the calculation. The proposed simplifications demonstrate negligible performance change, namely: 0.00%/-0.02%	/-0.01% and 0.02%/-0.04%/-0.04% (Y/Cb/Cr) in AI and RA configurations correspondingly.

# Introduction

The filtering process of the Adaptive Loop Filter, is performed as follows:

Ox,y=i,jwi,j.Ix+i,y+j,

where samples I(x+i,y+j) are input reconstructed samples, O(x,y) is the filtered output reconstructed sample (i.e. filter result), and wi, j denotes the filter coefficients. The described above equation can be reformulated (according to Non-linear Adaptive Loop Filter method, introduced in JVET-M0385 and adopted to VTM5.0), without coding efficiency impact, in the following expression:

Ox,y=Ix,y+(i,j)≠(0,0)wi, j.Ix+i,y+j-Ix,y,

O'x,y=Ix,y+

i,j)≠(0,0wi, j×Clip∆x,y,i,j,Limi,j+Clip∆x,y,-i,-j,Limi,j

where wi, j are the same filter coefficients as in first equation [(i,j)wi, j=1]. According to JVET-M0385, the ALF filter is modified as follows:

∆x,y,i,j=Ix+i,y+j-I(x,y)

Clipd,b=min(b,max-b,d

Limi,j=2BitDepthLuma*4-Idxi,j4 , for luma filtering

Limi,j=2BitDepthChroma-8+8*3-Idxi,j3,  for chroma filtering

Idxi,j=0,1,2 or 3

If maximal weight coefficient w(i,j) is 8-bits integer value, BitDepthLuma is luma component bit depth and BitDepthChroma is chroma component bit depth then filter implementation requires N integer multiplication of 8-bit value by (BitDepthLuma+3)-bit values for luma component filtering and by (BitDepthChroma+3) for chroma component filtering, wherein N is amount of filter coefficients (12 for luma and 6 for chroma). For example if BitDepth equals to 10, then maximum value after clipping is Lim(i,j)=2bitDepth=210=1024, that requires 12 bits if take into account clipping by positive and negative value of Lim(i,j). After sum of clipped values the bit depth is increased to 13. In the worst case scenario it would require 12 multiplications of 8-bit by 13-bit values.

# Proposal

## Aspect 1: fix of clipping value calculation formula

As a first aspect of this contribution we propose to modify formula of clipping value calculation to ensure the output after clipping by negative and positive value of Limi,j is in 11 bit range (for internal BitDepth of 10 bit):

Limi,j=2BitDepthLuma*4-Idxi,j4-1, for luma filtering

Limi,j=2BitDepthChroma-8+8*3-Idxi,j3-1, for chroma filtering

Idxi,j=0,1,2,3

According to the formula the maximum clipping level is achieved at the Idx = 0. Supposing BitDepthLuma =10, the maximum clipping value is Lim(i,j)=2BitDepthLuma-1=210-1=1023. That fits into 10 bits for positive value, and would require 11 bits if also consider negative clipping value.

The following modifications in specification text are required:

AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ] = filterClips[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ]

filterClips[ sfIdx ][ j ] = Round( 2( BitDepthY * ( 4 − alf_luma_clip_idx[ sfIdx ][ j ] ) / 4 ) - 1)

…

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2( BitDepthC − 8 ) * 2( 8 * ( 3 − alf_chroma_clip_idx[ j ] ) / 3 ) - 1)

This modification saves 1 bit of filtering multiplication bit depth.

## Aspect 2: secondary clipping

∆x,y,i,j=Ix+i,y+j-I(x,y)

According to the second aspect we propose to add secondary clipping to sum of clipped differences between filtering sample and neighbor sample to ensure the sum of clipped differences is still in 11 bit range (for internal BitDepth of 10 bit):

Clipd,b=min(b,max-b,d

O'x,y=Ix,y+

i,j)≠(0,0wi, j×Clip{Clip∆x,y,i,j,Limi,j+Clip∆x,y,-i,-j,Limi,j,Thr}

Thr=2BitDepthLuma-1 for luma filtering

Thr=2BitDepthChroma-1 for chroma filtering

Proposed modifications of specification are as follows:

–ThrL = ( 1 << BitDepthY ) − 1

sum = f[ idx[ 0 ] ]   * Clip3(-ThrL, ThrL, Clip3( −c[ idx[ 0 ] ], c[ idx[ 0 ] ],     recPictureL[ hx, vy + r3 ] − curr ) + 
		Clip3( −c[ idx[ 0 ] ], c[ idx[ 0 ] ],     recPictureL[ hx, vy − r3 ] − curr ) ) + 
	f[ idx[ 1 ] ]   * Clip3(-ThrL, ThrL, Clip3( −c[ idx[ 1 ] ], c[ idx[ 1 ] ],     recPictureL[ hx + 1, vy + r2 ] − curr ) + 
		Clip3( −c[ idx[ 1 ] ], c[ idx[ 1 ] ],     recPictureL[ hx − 1, vy − r2 ] − curr ) ) + 
	f[ idx[ 2 ] ]   * Clip3(-ThrL, ThrL,  Clip3( −c[ idx[ 2 ] ], c[ idx[ 2 ] ],     recPictureL[ hx, vy + r2 ] − curr ) + 
		Clip3( −c[ idx[ 2 ] ], c[ idx[ 2 ] ],     recPictureL[ hx, vy − r2 ] − curr ) ) + 
	f[ idx[ 3 ] ]   * Clip3(-ThrL, ThrL,(  Clip3( −c[ idx[ 3 ] ], c[ idx[ 3 ] ],     recPictureL[ hx − 1, vy + r2 ] − curr ) + 
		Clip3( −c[ idx[ 3 ] ], c[ idx[ 3 ] ],     recPictureL[ hx + 1, vy − r2 ] − curr ) ) + 
	f[ idx[ 4 ] ]   * Clip3(-ThrL, ThrL, Clip3( −c[ idx[ 4 ] ], c[ idx[ 4 ] ],     recPictureL[ hx + 2, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 4 ] ], c[ idx[ 4 ] ],     recPictureL[ hx − 2, vy − r1 ] − curr ) ) + 
	f[ idx[ 5 ] ]   * Clip3(-ThrL, ThrL,  Clip3( −c[ idx[ 5 ] ], c[ idx[ 5 ] ],     recPictureL[ hx + 1, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 5 ] ], c[ idx[ 5 ] ],     recPictureL[ hx − 1, vy − r1 ] − curr ) ) + 
	f[ idx[ 6 ] ]   * Clip3(-ThrL, ThrL,  Clip3( −c[ idx[ 6 ] ], c[ idx[ 6 ] ],     recPictureL[ hx, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 6 ] ], c[ idx[ 6 ] ],     recPictureL[ hx, vy − r1 ] − curr ) ) + 		f[ idx[ 7 ] ]   * Clip3(-ThrL, ThrL,  Clip3( −c[ idx[ 7 ] ], c[ idx[ 7 ] ],     recPictureL[ hx − 1, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 7 ] ], c[ idx[ 7 ] ],     recPictureL[ hx + 1, vy − r1 ] − curr ) ) + 
	f[ idx[ 8 ] ]   * Clip3(-ThrL, ThrL,  Clip3( −c[ idx[ 8 ] ], c[ idx[ 8 ] ],     recPictureL[ hx − 2, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 8 ] ], c[ idx[ 8 ] ],     recPictureL[ hx + 2, vy − r1 ] − curr ) ) + 
	f[ idx[ 9 ] ]   * Clip3(-ThrL, ThrL,  Clip3( −c[ idx[ 9 ] ], c[ idx[ 9 ] ],     recPictureL[ hx + 3, vy ] − curr ) + 
		Clip3( −c[ idx[ 9 ] ], c[ idx[ 9 ] ],     recPictureL[ hx − 3, vy ] − curr ) ) + 
	f[ idx[ 10 ] ] * Clip3(-ThrL, ThrL,  Clip3( −c[ idx[ 10 ] ], c[ idx[ 10 ] ], recPictureL[ hx + 2, vy ] − curr ) + 
		Clip3( −c[ idx[ 10 ] ], c[ idx[ 10 ] ], recPictureL[ hx − 2, vy ] − curr ) ) + 
	f[ idx[ 11 ] ] * Clip3(-ThrL, ThrL,  Clip3( −c[ idx[ 11 ] ], c[ idx[ 11 ] ], recPictureL[ hx + 1, vy ] − curr ) + 
		Clip3( −c[ idx[ 11 ] ], c[ idx[ 11 ] ], recPictureL[ hx − 1, vy ] − curr ) )

, and for chroma components:

  - Thrc = ( 1 << BitDepthC ) − 1

sum = f[ 0 ] * Clip3(-Thrc , Thrc,  Clip3( −c[ 0 ], c[ 0 ], recPicture[ hx, vy + r2 ] − curr ) + 
		Clip3( −c[ 0 ], c[ 0 ], recPicture[ hx, vy − r2 ] − curr ) ) + 
	f[ 1 ] * Clip3(-Thrc, Thrc, Clip3( −c[ 1 ], c[ 1 ], recPicture[ hx + 1, vy + r1 ] − curr ) + 
		Clip3( −c[ 1 ], c[ 1 ], recPicture[ hx − 1, vy − r1 ] − curr ) ) + 
	f[ 2 ] * Clip3(-Thrc, Thrc, Clip3( −c[ 2 ], c[ 2 ], recPicture[ hx, vy + r1 ] − curr ) + 
		Clip3( −c[ 2 ], c[ 2 ], recPicture[ hx, vy − r1 ] − curr ) ) + 		f[ 3 ] * Clip3(-Thrc, Thrc, Clip3( −c[ 3 ], c[ 3 ], recPicture[ hx − 1, vy + r1 ] − curr ) + 
		Clip3( −c[ 3 ], c[ 3 ], recPicture[ hx + 1, vy − r1 ] − curr ) ) + 
	f[ 4 ] * Clip3(-Thrc, Thrc, Clip3( −c[ 4 ], c[ 4 ], recPicture[ hx + 2, vy ] − curr ) + 
		Clip3( −c[ 4 ], c[ 4 ], recPicture[ hx − 2, vy ] − curr ) ) + 
	f[ 5 ] * Clip3(-Thrc, Thrc, Clip3( −c[ 5 ], c[ 5 ], recPicture[ hx + 1, vy ] − curr ) + 
		Clip3( −c[ 5 ], c[ 5 ], recPicture[ hx − 1, vy ] − curr ) )

This modification additionally saves 1 bit of filtering multiplication bit depth.

## Aspect 3: use same formula for luma and chroma clipping value calculation

According to the third aspect we propose to use same formula for clipping value calculation both for luma and chroma:

Limi,j=2BitDepthLuma*4-Idxi,j4-1, for luma filtering

Limi,j=2BitDepthChroma*4-Idxi,j4-1, for chroma filtering

The following changes in specification would be required:AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ] = filterClips[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ]

filterClips[ sfIdx ][ j ] = Round( 2( BitDepthY * ( 4 − alf_luma_clip_idx[ sfIdx ][ j ] ) / 4 ) - 1)

…

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2(BitDepthC * ( 4 − alf_chroma_clip_idx ][ j ] ) / 4 ) - 1)

It bring additional simplification due to unification of clipping value calculation for luma and chroma components

# Complexity reduction for worst case

In the worst case non-linear adaptive loop filter of VTM5.0 requires 12 multiplications of 8-bit integer filter coefficient by (BitDepth+3)-bit integer value equals to sum of clipped difference between filtering sample and neighbor sample. The proposed method requires 12 multiplication of 8- bit integer filter coefficient by (BitDepth+1)-bit integer value equals to clipped sum of clipped difference between filtering sample and neighbor samples that saves 2 bits for each multiplication which is beneficial for hardware implementation due to saving of square area.

# Simulation Results

The proposed non-linear adaptive loop filters simplifications are implemented on top of VTM-5.0 and the simulation is performed under common test conditions as described in JVET-N1010 [3].

## Test 1: Aspect 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.03% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | 0.01% | 101% | 101% |
| Class B | 0.00% | 0.01% | 0.02% | 100% | 100% |
| Class C | 0.01% | 0.04% | 0.07% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Overall | 0.00% | 0.02% | 0.03% | 100% | 101% |
| Class D | 0.00% | 0.02% | 0.03% | 101% | 102% |
| Class F (optional) | 0.02% | 0.04% | 0.06% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.01% | 98% | 99% |
| Class A2 | 0.01% | -0.04% | 0.04% | 99% | 100% |
| Class B | 0.02% | -0.01% | -0.07% | 99% | 100% |
| Class C | 0.00% | -0.06% | -0.10% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.04% | -0.04% | 99% | 100% |
| Class D | 0.02% | 0.04% | 0.26% | 100% | 100% |
| Class F (optional) | 0.03% | 0.11% | 0.09% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.03% | -0.05% | -0.28% | 99% | 101% |
| Class E | 0.13% | 0.22% | -0.64% | 99% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | -0.09% | 0.58% | 100% | 102% |
| Class F (optional) | 0.01% | 0.30% | -0.09% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.03% | -0.28% | -0.10% | 99% | 101% |
| Class E | 0.00% | 0.44% | -0.12% | 100% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | -0.06% | 0.48% | 100% | 102% |
| Class F (optional) | 0.00% | 0.52% | 0.50% | 100% | 100% |

## Test 2: Aspect 1 and Aspect 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.03% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | 0.01% | 100% | 102% |
| Class B | 0.00% | 0.02% | 0.02% | 100% | 101% |
| Class C | 0.01% | 0.04% | 0.07% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.02% | 101% | 106% |
| Overall | 0.00% | 0.02% | 0.03% | 100% | 102% |
| Class D | 0.01% | 0.02% | 0.03% | 100% | 100% |
| Class F (optional) | 0.01% | 0.04% | 0.05% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.03% | 99% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.03% | -0.01% | -0.06% | 99% | 100% |
| Class C | -0.02% | -0.06% | -0.14% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.06% | 0.02% | 0.18% | 100% | 101% |
| Class F (optional) | 0.01% | 0.13% | 0.11% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.02% | -0.09% | -0.10% | 100% | 102% |
| Class E | -0.02% | -0.13% | -0.13% | 100% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.07% | 0.26% | 0.69% | 100% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E | 0.00% | 0.64% | -0.06% | 100% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.06% | -0.44% | 0.43% | 100% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Test 3: Aspect 1, Aspect 2 and Aspect 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.04% | 100% | 101% |
| Class A2 | 0.00% | -0.02% | 0.01% | 100% | 102% |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class C | 0.01% | -0.01% | -0.01% | 100% | 101% |
| Class E | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F (optional) | 0.01% | 0.00% | 0.05% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.08% | 100% | 102% |
| Class A2 | 0.02% | 0.01% | 0.01% | 100% | 101% |
| Class B | 0.04% | 0.02% | -0.08% | 99% | 101% |
| Class C | -0.01% | -0.13% | -0.10% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.04% | 100% | 101% |
| Class D | 0.05% | 0.12% | 0.40% | 100% | 101% |
| Class F (optional) | 0.02% | 0.10% | 0.09% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.05% | -0.05% | 0.05% | 99% | 102% |
| Class E | 0.05% | 0.21% | 0.04% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.08% | 0.44% | 0.81% | 99% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.02% | -0.38% | 0.13% | 99% | 101% |
| Class E | -0.03% | 0.99% | -0.14% | 100% | 105% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.04% | 0.08% | -0.37% | 100% | 103% |
| Class F (optional) | -0.11% | 0.84% | -0.12% | 100% | 100% |

# Conclusion

This contribution proposes three aspects for simplification of non-linear adaptive loop filter: clipping level formula modification to ensure result is within 11 bit range (for 10 bit internal bit depth); apply secondary clipping to ensure sum of clipped values is still in 11 bit range; use same formula for luma and chroma clipping value calculation. The proposed method allows to save 2 bits of multiplication bit depth from 8x(BitDepth+3) to 8x(BitDepth+1) for each of 12 multiplications in the worst case scenario. The coding performance impact is negligible. We suggest to adopt proposed modifications in next release of VVC

# References

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, 19–27 March 2019

- J. Taquet, C. Gisquet, G. Laroche, P. Onno “Non-linear Adaptive Loop Filter”, JVET-M0385, Marrakech, MA, 9–18 Jan. 2019

- J. Taquet, P. Onno, C. Gisquet, G. Laroche “CE5: Results of tests CE5-3.1 to CE5-3.4 on Non-Linear adaptive loop filter”, JVET-N0242, Geneva, CH, 19–27 March 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET common test conditions and software reference configurations”, JVET-N1010, Geneva, CH, 19–27 March 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).