| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0189 |
| --- | --- |

| Title: | AHG17: Support of decoding unit in HRD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui | Tel: Email: | +81 44 754 2049 kazui.kimihiko@fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

- Abstract

This contribution proposes to introduce the concept of decoding unit (DU) to the hypothetical reference decoder (HRD). Parameters and operation of the coded picture buffer (CPB) and decoded picture buffer (DPB) specified in the HEVC standard are suggested as the starting point. Support of emerging elements possibly to be adopted, such as sub-picture, will be investigated once such elements are adopted.

Need of supporting decoding unit for lowering end to end delay

The HEVC standard adopted the concept of decoding unit (DU) in the design to support very low end-to-end delay (i.e. less than one picture interval) operation. Specifically, a set of syntax elements for signalling DU-related parameters is introduced and operation of the coded picture buffer (CPB) and decoded picture buffer (DPB) for DU is defined in Annex C.

It is the author’s understanding that very low delay coding plays an important roll in emerging applications of modern video coding standard, such as remote vehicle control. Moreover, in the development of the VVC standard, several tools for supporting very low delay coding (a.k.a. gradual intra refresh) with better coding efficiency were proposed and tested in CE11 established at the 14th JVET meeting [1].

At the 14th JVET meeting, there were multiple proposals on the initial design of the HRD. One of such proposal [2] proposed to include DU-based operation in addition to AU-based operation, as specified in the HEVC standard. The conclusion at the 14th meeting was to adopt the HRD of HEVC, but to exclude DU in the initial version of the HRD in the VVC draft.

The purpose of this contribution is to propose DU-based HRD operation in the VVC standard.

Proposal

The proposed text changes are in the attachment to this contribution, with change marks relative to JVET-N1001-v8. It is noted that support of emerging elements possibly to be adopted, such as sub-picture, will be investigated once such elements are adopted.

Syntax elements for DU-based operation

hrd_parameters()

- sub_pic_hrd_params_present_flag

- tick_divisor_minus2

- sub_pic_cpb_params_in_pic_timing_sei_flag

- cpb_size_du_scale

sub_layer_hrd_parameters()

- cpb_size_du_value_minus1[ i ]

- bit_rate_du_value_minus1[ i ]

buffering_period()

- du_cpb_removal_delay_increment_length_minus1

- dpb_output_delay_du_length_minus1

- nal_initial_alt_cpb_removal_delay[ i ]

- nal_initial_alt_cpb_removal_offset[ i ]

- vcl_initial_alt_cpb_removal_delay[ i ]

- vcl_initial_alt_cpb_removal_offset[ i ]

picture_timimg()

- pic_dpb_output_du_delay

- num_decoding_units_minus1

- du_common_cpb_removal_delay_flag

- du_common_cpb_removal_delay_increment_minus1

- num_nalus_in_du_minus1[ i ]

- du_cpb_removal_delay_increment_minus1[ i ]

decoding_unit_info()

- decoding_unit_idx

- du_spt_cpb_removal_delay_increment

- dpb_output_du_delay_present_flag

- pic_spt_dpb_output_du_delay

Hypothetical Reference Decoder for DU-based operation

C.1 “General”

- Set of bitstream conformance test

C.2 “Operation of coded picture buffer”

- CPB removal time of a decoding unit

C.3 “Operation of decoded picture buffer”

- DPB output time of a decoding unit

C.4 Bitstream conformance

- Constraints on bitstream with sub_pic_hrd_params_present_flag equal to 1

Conclusion

This contribution proposes to introduce DU-based HRD operation for supporting very low delay coding in the VVC standard.

References

- “Description of Core Experiment 11 (CE11): Gradual random access”, JVET-N1031, March 2019

- K.Kazui, “Hypothetical Reference Decoder in VVC”, JVET-N0077, March 2019

Patent rights declaration(s)

Fujitsu Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).