| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0199 |
| --- | --- |

| Title: | AHG12: On rectangular slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Vadim Seregin Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes rectangular slice ordering in pictures and signalling of slice IDs.

# Problem statement and proposal

When rectangular slice extraction happens to form a new extracted bitstream, then the order of the slices may not be in increasing order of their slice_address values.

An example is illustrated below where a full resolution and reduced resolution representation of 360 video cubemap faces are packed as shown where each face is a rectangular slice with corresponding ids:

Figure 1: cubemap with 6 full resolution and 6 half resolution faces.

A viewport dependent streaming server packs the main viewport 9 with accompanying 6 low resolution faces into a new picture using bitstream extraction process where slice (header) data is unmodified.

Figure 4 Extracted viewport dependent bitstream picture.

In this packing scheme for example where the full resolution face (i.e front face) is always packed at the beginning of the picture, then slice_id 9 comes before other slice ids in the picture. In VVC spec. , the signalling of slice IDs and slice addressing described as follows.

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| ….. |  |
| } |  |

#### 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| …. |  |

#### 7.4.6.1 General slice header semantics

……

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the brick ID as specified by Equation 6-8.

- The length of slice_address is Ceil( Log2 ( NumBricksInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumBricksInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the slice ID of the slice.

- The length of slice_address is signalled_slice_id_length_minus1 + 1 bits.

- If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- The value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- The slices of a picture shall be in increasing order of their slice_address values.

….

So instead of restricting the slice addresses to be increasing order as stated in 7.4.6.1, it is proposed to state that “The slices of a picture shall be in increasing order of brick IDs.” Basically, the slice_address that is assigned to slice_id in case of rectangular slices could be in arbitrary order but the corresponding rectangular slices indicated by slice_id should follow the order of regular slices or tiles or bricks in pictures. The bitstream order for above example would be content from slices with IDs 9,2,3,10,7,6,11.

# Slice ID signalling in PPS

In PPS, the slice_ids of rectangular slices are signalled as absolute id value give by slice_id[i].

slice_id[ i ] specifies the slice ID of the i-th slice. The length of the slice_id[ i ] syntax element is signalled_slice_id_length_minus1 + 1 bits.  When not present, the value of slice_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_slices_in_pic_minus1, inclusive.

Instead of signaling slice_ids in absolute value, the delta between slice_ids can be signalled as delta_slice_id_minus1 and the slice_id can be derived from the signaled delta slice_id_minus1[i] values.

This can be expressed as

| if( rect_slice_flag ) { |  |
| --- | --- |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| slice_id[0] | u(v) |
| for( i = 1; i <= num_slices_in_pic_minus1; i++ ){ |  |
| delta_slice_id_minus1[ i ] | ue(v) |
| sign_delta_slice_id[i] | u(1) |
| } |  |
| } |  |
| } |  |

delta_slice_id_minus1[ i ] specifies the absolute difference between slice ID of the i-th slice and i-1th slice minus 1.

sign_delta_slice_id[ i ] specifies the sign of delta_slice_id_minus1[i] syntax element plus 1. Value of 0 indicates positive difference between consecutive slice_id values, and value of 1 indicates negative difference.

slice_id[ i ] specifies the slice ID of the i-th slice. The length of the slice_id[ i ] syntax element is signalled_slice_id_length_minus1 + 1 bits.  When not present, the value of slice_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_slices_in_pic_minus1, inclusive. The value of slide_id[i] for i greater than 0 are derived by slice_id[i] = slice_id[i-1] + (1-2*sign_delta_slice_id[i]) * (delta_slice_id_minus1[i] + 1).

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).