| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0202 |
| --- | --- |

| Title: | Non-CE3: MIP Modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong-Uk Yoon Do-Hyeon Park Jae-Gon Kim Jinho Lee Jungwon Kang | Tel: Email: | +82-2-300-0414 yuyoon@kau.kr dhpark@kau.kr jgkim@kau.ac.kr jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Korea Aerospace University (KAU), ETRI |  |  |

_____________________________

- Abstract

This contribution proposes two modifications of matrix-weighted intra prediction (MIP). The first modification is changing the positions of the neighboring blocks to be referred deriving MIP MPM. The second modification is removing inverse mapping table which maps MIP matrix modes to regular intra modes. This contribution reports the performances on the two tests below:

- Test 1: modified positions for deriving MIP MPM candidates

- Test 2: remove inverse mapping table that assigns MIP modes to regular intra prediction modes

The experiment results for test 1 reportedly show 0.00% and 0.XX% luma BD-rates performance with 101% and, XX% encoding time for the AI and RA, respectively.

The experiment results for test 2 reportedly show 0.01% and 0.XX% luma BD-rates performance with 99% and, XX% encoding time for the AI and RA, respectively.

Introduction

At the last meeting, MIP was adopted as a new intra coding tool. In VTM 5.0, if the MIP mode is applied, the index of the MIP mode is signaled using 3 MPMs which are derived from neighboring blocks. Unlike the neighboring positions which are referred to derive the regular 6 MPMs, which are top-right and bottom-left, 3 MPMs of MIP are derived from neighboring positions of top-left corner as shown in Figure 1. The derivation of the MPMs is harmonized with regular intra modes by performing mode mapping between regular intra modes and MIP modes based on the predefined mapping tables which depend on the block size. The forward mapping table maps regular intra modes to MIP modes, and the inverse mapping table maps MIP modes to regular intra modes. The inverse mapping table is used for the derivation of regular MPM modes and a chroma DM mode. In other words, when deriving regular MPM modes or a chroma DM mode, if the referenced block is a MIP mode, the MIP mode is mapped to a regular intra mode by using the inverse mapping table.

Proposed modifications

Modification of positions deriving MIP MPM candidates

In VTM5.0, the MIP MPM is derived from positions shown in Figure 1 (right) while the regular MPM is derived from positions shown in Figure 1 (left). To keep the consistency of the reference positions deriving MPM, the first modification proposes to change the reference positions deriving MIP MPM as the same positions which derive regular MPM.

Remove inverse mapping table that assigns MIP mode to regular intra mode

This contribution proposes to remove the inverse mapping table. When deriving regular MPM or chroma DM mode, if the referenced block is a MIP mode, the mode of the reference block is considered as planar mode. Therefore, the cases where the inverse mapping table is used are eliminated, so that the inverse mapping table can be removed.

Figure 1. Referenced positions for MPM (left: regular MPM, right: MIP MPM)

Experimental results

The proposed modifications are implemented on top of VTM5.0 and evaluated with VTM configurations according to the common test conditions. This contribution reports on the performance on the two tests below:

- Test 1: modified positions for deriving MIP MPM candidates

- Test 2: remove the inverse mapping table that assigns MIP modes to regular intra prediction modes

Test 1 results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.11% | 104% | 101% |
| Class A2 | 0.01% | -0.02% | -0.08% | 102% | 100% |
| Class B | 0.00% | -0.02% | 0.03% | 101% | 101% |
| Class C | 0.01% | -0.12% | -0.01% | 98% | 100% |
| Class E | 0.01% | -0.11% | -0.06% | 101% | 100% |
| Overall | 0.00% | -0.05% | 0.00% | 101% | 100% |
| Class D | -0.01% | 0.02% | 0.00% | 99% | 102% |
| Class F | -0.01% | 0.02% | -0.02% | 102% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.15% | 0.04% | 100% | 100% |
| Class A2 | 0.01% | -0.10% | 0.04% | 100% | 100% |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.12% | 0.44% | 98% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test 2 results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.04% | 101% | 100% |
| Class A2 | 0.03% | 0.02% | -0.02% | 94% | 100% |
| Class B | 0.01% | -0.04% | 0.09% | 100% | 100% |
| Class C | 0.00% | -0.12% | 0.05% | 99% | 101% |
| Class E | 0.03% | 0.01% | 0.03% | 101% | 100% |
| Overall | 0.01% | -0.02% | 0.04% | 99% | 100% |
| Class D | -0.03% | 0.19% | 0.00% | 100% | 104% |
| Class F | -0.02% | -0.02% | 0.08% | 102% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.22% | 100% | 100% |
| Class A2 | 0.07% | 0.22% | 0.06% | 101% | 100% |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

This contribution presents two modifications of MIP. First, the positions deriving MPM candidates of MIP are modified to be the same as the positions deriving regular MPM candidates. Secondly, the inverse mapping table which maps MIP modes to regular intra modes is removed. The three tests are performed in this contribution. The experimental results report 0.00% and 0.01% luma BD-rates performance without change of encoding/decoding running time for Test 1 and Test 2, respectively.

Patent rights declaration(s)

KAU and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).