| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 |  |  |  | Document: JVET-O0203-v1 |  |
| --- | --- | --- | --- | --- | --- |
| Title: | CE3- related: Simplification of Matrix-based Intra Prediction (MIP) |  |  |  |  |  |
| Status: | Input document to JVET |  |  |  |  |  |
| Purpose: | Proposal |  |  |  |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Biao Wang, Semih Esenlik, Elena Alshina | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com biao.wang@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com |  |  |  |
| Source: | Huawei Technologies Co. |  |  |  |  |  |

_____________________________

# Abstract

In this document, we propose a set of mechanisms for reducing computational complexity and memory consumption of Matrix-based Intra Prediction (MIP). In Test 1, we align the width of MIP multipliers with the 16-bit multipliers used for interpolation filtering in the conventional intra prediction mechanism. In addition to reducing the multiplier width, multiplication by the MIP coefficients used in Test 1 can be implemented using add-and-shift technique (with no more than either 1 addition or 1 subtraction operation). Test 1 reportedly shows the following overall results over VTM-5.0: 0.03% (Y), -0.02% (U), 0.03% (V) with encoding time of 100% and decoding time of 100% for AI configuration, 0.02% (Y), -0.01% (U), 0.00% (V) with encoding time of 99% and decoding time of 100% for RA configuration. Besides, we test MIP with 6-bit multipliers on top of JVET-O0084. This test (Test 2) reportedly shows the following overall results over VTM-5.0: -0.05% (Y), -0.10% (U), -0.09% (V) with encoding time of 100% and decoding time of 99% for AI configuration, -0.03% (Y), -0.17% (U), -0.07% (V) with encoding time of 100% and decoding time of 99% for RA configuration.

# Introduction

In proposal JVET-N0217 [1] adopted at the 14th JVET meeting into VTM-5.0 and the VVC spec draft, Matrix-based Intra Prediction (MIP) also known as affine linear weighted intra prediction (ALWIP) is proposed to generate the luma intra prediction signal out of one line of reference samples left and above a current block by a matrix vector multiplication and the addition of an offset. In the VTM-5.0 design, MIP coefficients A (mWeight according to the VVC spec draft) have bit-depth of 10 bits, i.e. their bit-depth is not aligned with interpolation filtering coefficient bit-depth (6 bits) for conventional intra-prediction. Thus, 19-bit multipliers (10-bit input value x 9-bit MIP coefficient magnitude,) are required to perform matrix-vector multiplication. Even in the design with reduced bit-depth of MIP coefficients [2], 18-bit multipliers (10-bit input value magnitude x 8-bit MIP coefficient) are required to perform matrix-vector multiplication.

However, as pointed out in the JVET meeting notes [3] (p. 66), “implementability in 16 bit logic” was one of the criteria that should be met by intra-prediction tools (e.g., interpolation filters). Thus, it would be desirable if MIP could also fulfill this requirement.

# Proposal

## This proposal contains two aspects. The first aspect relates to aligning the width of MIP multipliers with 16-bit multipliers used for interpolation filtering in the conventional intra prediction mechanism by decomposing magnitudes of original MIP coefficients into a 6-bit mantissa and a 2-bit exponent. In the second aspect, MIP coefficients are represented in such a way that multiplication operations can be replaced by cheaper shift and addition/subtraction operations.

## Bit-depth reduction of MIP coefficients

As shown in Fig. 1, MIP coefficients in VTM-5.0 have 9-bit magnitude prepended by a sign.

Fig. 1. Representation of MIP coefficients in VTM-5.0

We propose to reduce their bit-depth to enable performing matrix-vector multiplication with 6-bit multipliers. So, magnitudes of all MIP coefficients are quantized subject to the positions of most significant bits as presented in Fig. 2. In this case, the quantized 9-bit coefficient magnitude can be replaced with 6-bit field as a base magnitude (mantissa) and 2-bit field for a left shift (exponent). Fig. 3 illustrates the proposed representation of MIP coefficients. If this approach is implemented on top of MIP with 8-bit coefficients [2], the sign bit shown in Fig. 3 can be omitted as MIP coefficients always are non-negative. As a concrete example, we can use an original MIP coefficients with magnitude of 130. It is quantized to 132 that can be represented as 33 in the 6-bit field (base magnitude or exponent) and 2 in the 2-bit filed (exponent or left shift value).

Fig. 2. Quantization of 10-bit MIP coefficients

Fig. 3. Proposed representation of MIP coefficients

Thus, MIP coefficients with 9-bit magnitude used in VTM-5.0 are replaced by coefficients Coef8bit with 8-bit magnitude that are represented as follows:

Coef8bit= sign∙Base6bit≪ shift2bit,

where sign is the sign of a quantized coefficient Coef8bit; Base6bit and shift2bit are 6-bit base magnitude and 2-bit shift, respectively. Multiplication in MIP is performed using the following formula:

predredi=j=0inSize-1signij∙Base6bitij⋅inputred[j]≪shift2bitij+oW≫sW+b,

where inputred is input signal value; predred is reduced prediction signal; bdryred is input signal value; b is a bias value; oW and sW are rounding offset and shift, respectively. Similarly, coefficients Coef8bit with 8-bit magnitude for the MIP design presented in [2] are computed as follows:

Coef8bit= Base6bit≪ shift2bit.

Taking into account the changes in [2], the reduced prediction signal predred is calculated according to the following formula:

predredi=j=0inSize-1Base6bitij⋅inputred[j]≪shift2bitij+oW≫sW+dcVal,

where inputred is input signal value; oW and sW are rounding offset and shift, respectively; dcVal is mean value (a detailed description of these variables can be found in [2]).

## Computational complexity reduction for matrix-vector multiplication

In the second approach, an original MIP coefficient is quantized into a coefficient, which can be represented using the following formula:

C= ak∙2k+aj∙2j,

where ak = {-1, 0, 1} and aj = {-1, 0, 1}; k ≠ j.

For example, an original MIP coefficients with magnitude of 131 would be quantized to 130 or 132, which can be represented as 128+2 or 128+4, respectively. Using this approach, the coefficients are quantized in a row-by-row fashion in original coefficient matrix. Meanwhile, we try to keep the difference between the sum of quantized coefficients in a row and the sum of original coefficient as less as possible.

# Experiment results

## Simulation result

The results of testing the above-described aspects implemented on top of VTM-5.0 and evaluated under the JVET CTC conditions [4] are presented in Tables 1 for AI and RA configurations. It is worth noting that the results presented in Table 1 are obtained for such a set of quantized MIP coefficients that are derived by using both approaches described in section 2.1 and 2.2.

Table 1. Experimental results for Test 1 (includes both aspects described in Section 2.1 and Section 2.2 on top of VTM-5.0)

|  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM-5.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | 0.07% |  | 0.04% |  | 0.11% | 99% |  | 98% |  |
| Class A2 | 0.03% |  | 0.01% |  | -0.06% | 101% |  | 105% |  |
| Class B | 0.03% |  | -0.03% |  | 0.02% | 100% |  | 100% |  |
| Class C | 0.02% |  | -0.03% |  | 0.11% | 99% |  | 98% |  |
| Class E | 0.04% |  | -0.07% |  | -0.03% | 100% |  | 100% |  |
| Overall | 0.03% |  | -0.02% |  | 0.03% | 100% |  | 100% |  |
| Class D | 0.01% |  | 0.32% |  | -0.02% | 100% |  | 101% |  |
| Class F | 0.01% |  | -0.02% |  | 0.05% | 99% |  | 100% |  |
|  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  | Over VTM-5.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |  |
| Class A1 | 0.07% |  | -0.01% |  | 0.25% | 100% |  | 101% |  |
| Class A2 | 0.04% |  | -0.10% |  | 0.05% | 99% |  | 100% |  |
| Class B | 0.03% |  | -0.05% |  | -0.12% | 99% |  | 100% |  |
| Class C | -0.04% |  | 0.09% |  | -0.09% | 99% |  | 99% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% |  | -0.01% |  | 0.00% | 99% |  | 100% |  |
| Class D | 0.01% |  | -0.20% |  | 0.00% | 99% |  | 99% |  |
| Class F | 0.00% |  | -0.06% |  | 0.10% | 99% |  | 100% |  |

## Additional result based on JVET-O0084

In Table 2, we present the results obtained by testing approach described in Section 2.1 on top of JVET- O0084 [2] according the JVET CTC conditions [4] for AI and RA configurations. Note that the anchor for this test is VTM-5.0.

Table 2. Experimental results for Test 2 (includes only aspect described in Section 2.1 on top of JVET- O0084 [2])

|  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM-5.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  |  | DecT |
| Class A1 | -0.09% |  | -0.11% |  | -0.17% | 100% |  |  | 100% |
| Class A2 | -0.05% |  | -0.11% |  | -0.10% | 101% |  |  | 102% |
| Class B | -0.05% |  | -0.13% |  | -0.13% | 100% |  |  | 100% |
| Class C | -0.01% |  | -0.04% |  | 0.08% | 100% |  |  | 97% |
| Class E | -0.05% |  | -0.08% |  | -0.14% | 100% |  |  | 100% |
| Overall | -0.05% |  | -0.10% |  | -0.09% | 100% |  |  | 99% |
| Class D | 0.00% |  | 0.04% |  | -0.13% | 100% |  |  | 99% |
| Class F | 0.02% |  | 0.04% |  | 0.09% | 100% |  |  | 99% |
|  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  | Over VTM-5.0 |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  |  | DecT |
| Class A1 | -0.05% |  | -0.35% |  | -0.08% | 100% |  |  | 100% |
| Class A2 | -0.02% |  | 0.05% |  | 0.03% | 100% |  |  | 100% |
| Class B | -0.03% |  | -0.08% |  | -0.15% | 100% |  |  | 100% |
| Class C | -0.02% |  | -0.31% |  | -0.06% | 99% |  |  | 99% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.03% |  | -0.17% |  | -0.07% | 100% |  |  | 99% |
| Class D | -0.03% |  | 0.06% |  | 0.17% | 100% |  |  | 100% |
| Class F | -0.01% |  | -0.25% |  | 0.07% | 99% |  |  | 100% |

# Conclusion

We suggest adopting the proposed modification of MIP that is implementable using 16-bit multipliers as well as equivalently one shift and one addition / subtraction, into the VTM software and the VVC spec draft.

# References

- J. Pfaff, et al, “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, 14th JVET Meeting, Geneva, CH, March 2019.

- J. Pfaff, et al, “8-bit implementation and simplification of MIP”, JVET-O0084, 15th JVET Meeting, Geneva, SE, July 2019.

- G. J. Sullivan, J.-R. Ohm, “Meeting Report of the 12th JVET Meeting (Macao, 3–12 Oct. 2018)”, JVET-L1000, 12th JVET Meeting, Macao, CN, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Huawei Technologies Co. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).