| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0205_v2 |
| --- | --- |

| Title: | CE8-related : Improvements on BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Junghak Nam Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {sunmi.yoo, junghak.nam, jungah.choi, jin78.heo jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

Two methods for improving BDPCM are proposed in this contribution as follows.

- Intra mode derivation for BDPCM

- Line-shaped CG for BDPCM block

In the current VVC, when the BDPCM mode is enabled for a block, its prediction mode is set to MODE_INTRA and its intra mode is inferred to be the first candidate in the MPM list, which is planar mode. Considering that BDPCM actually uses one of intra prediction between horizontal and vertical modes, this proposal suggests to derive the BDPCM prediction mode based on actual prediction mode. Another aspect is to use line-shaped CG for BDPCM. In the BDPCM process, once the DPCM direction is indicated to a CU, the transform skipped residuals are processed/predicted by the line. Therefore, combining the residuals with a line CG seems to be desirable and it also provides coding performance. As a result, coding gains of the proposed scheme is observed to be -0.18% / -0.28% in Class F and TGM in AI, -0.12% / 0.00% in Class F and TGM in RA, and -0.24% / -0.23% in Class F and TGM in LDB, respectively.

Introduction

In the current VVC, the BDPCM can be enabled for intra prediction blocks, and its prediction mode is specified by the syntax element intra_bdpcm_dir. This direction indicates both the intra prediction mode and the residual prediction direction – 0 specifies the horizontal prediction and the residual DPCM is applied in the horizontal direction to the block, and 1 for the vertical direction, respectively. However, in the current design, the intra mode for the BDPCM is inferred equal to the first MPM candidate, and it is always planar mode. It may decrease the coding performance since the BDPCM block is considered as an intra block and its intra prediction mode can be referred by the spatial neighbor blocks or co-located neighboring chroma components. Moreover, the BDPCM is applied to each line in the TS residual block, by subtracting the current row or column with the previous one (left for columns and above for rows). Since the DPCM residual can be dependent on its prediction, grouping the residuals similar to the predicted unit can enhance the coding performance.

Proposed method

Intra mode derivation for BDPCM block

In the proposed method, the intra prediction mode for the BDPCM is set to the one used for the actual prediction, either horizontal or vertical modes. If the BDPCM mode is set to 1, the syntax elements for the intra prediction modes remain not parsed, and inferred one of those two modes. Because the intra prediction mode is directly specified by intra_bdpcm_dir, there is no parsing dependency. The proposed spec text change is shown below.

### 8.4.1. General decoding process for coding units coded in intra prediction mode

(…)

- If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( cbHeight * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ),	(8-6)
		with i = 0..cbWidth − 1, j = 0..cbHeight − 1

- Otherwise, the following applies:

- The variable MipSizeId[ x ][ y ] for x = xCb..xCb + cbWidth − 1 and  y = yCb..yCb + cbHeight − 1 is derived as follows:

- If both cbWidth and cbHeight are equal to 4, MipSizeId[ x ][ y ] is set equal to 0.

- Otherwise, if both cbWidth and cbHeight are less than or equal to 8, MipSizeId[ x ][ y ] is set equal to 1.

- Otherwise, MipSizeId[ x ][ y ] is set equal to 2.

- The luma intra prediction mode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, the derivation process for matrix-based intra prediction mode as specified in clause 8.4.2 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- Otherwise, the derivation process for the luma intra prediction mode as specified in clause 8.4.3 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- The variable predModeIntra is

- -

- set equal to IntraPredModeY[ xCb ][ yCb ].

- The general decoding process for intra blocks as specified in clause 8.4.5.1 is invoked with the sample location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, predModeIntra, and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before in-loop filtering.

(…)

### 8.4.3 Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-3 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table  8-3 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived as follows:

- If BdpcmFlag[ xCb ][ yCb ] is equal to 1, IntraPredModeY[ xCb ][ yCb ] is derived as follows :

IntraPredModeY[ xCb ][ yCb ] = BdpcmDir[ xCb ][ yCb ] ? INTRA_ANGULAR50 : INTRA_ANGULAR18

- Otherwise, if BdpcmFlag[ xCb ][ yCb ] is equal to 0 and intra_luma_not_planar_flag[ xCb ][ yCb ] is equal to 0, IntraPredModeY[ xCb ][ yCb ] is set equal to INTRA_PLANAR .

- Otherwise (BdpcmFlag[ xCb ][ yCb ] is equal to 0 and intra_luma_not_planar_flag[ xCb ][ yCb ] is equal to 1), the following ordered steps apply:

(…)

Line-shaped CG for BDPCM block

In this method, the line-shaped CG for the BDPCM is proposed. The number of residuals in a CG is kept 16, as it is for the luma component, but the CG can be rectangular or line. The CG size is decided adaptively according to the direction of the BDPCM and the block size, as shown in Table 1. The scanning order for the proposed scheme remains the same as well, from the top left to the bottom right, with a diagonal scan order. Also, the non-square CGs are not new at all – those CGs are already introduced for ISP splits.

Table 1 CG size for BDPCM direction and block size

|  | Block Size | Coefficient group Size |
| --- | --- | --- |
| Horizontal BDPCM | M×N, N≥16 | 1×16 |
|  | M×N, N=8 | 2×8 |
|  | M×N, N=4 | 4×4 |
| Vertical BDPCM | M×N, M≥16 | 16×1 |
|  | M×N, M=8 | 8×2 |
|  | M×N, M=4 | 4×4 |

Experimental result

The proposed change is implemented in VTM-5.0 and evaluated under common test conditions. The coding performance shown below remains the same in Class A1 to Class E, since the BDPCM is enabled only for Class F and TGM in the current CTC.

Method 1

The proposed method has been implemented using VTM-5.0 and under the common test conditions. The coding performance shown below remains the same in Class A1 to Class E, since the BDPCM is enabled only for Class F and TGM in the current CTC.

Table 1 The coding performance of the proposed method (AI)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.12% | 0.04% | -0.08% | 99% | 98% |
| TGM | -0.18% | -0.06% | -0.07% | 100% | 99% |

Table 2 The coding performance of the proposed method (RA)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.09% | 0.08% | 0.10% | 100% | 98% |
| TGM | -0.18% | -0.02% | -0.13% | 99% | 98% |

Table 3 The coding performance of the proposed method (LDB)

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.02% | -0.21% | -0.44% | 100% | 100% |
| TGM | -0.04% | 0.01% | 0.04% | 100% | 102% |

Method 1 + Method 2

The coding performance of the combined BDPCM intra mode align and line CG is shown below.

Table 5 The combined result of line CG and intra mode align in AI

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.18% | -0.11% | -0.08% | 101% | 103% |
| Class SCC | -0.28% | -0.14% | -0.15% | 102% | 102% |

Table 6 The combined result of line CG and intra mode align in RA

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.12% | 0.07% | 0.10% | 101% | 105% |
| Class SCC | 0.00% | 0.09% | 0.03% | 100% | 102% |

Table 7 The combined result of line CG and intra mode align in LDB

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.24% | -0.30% | -0.31% | 99% | 98% |
| Class SCC | -0.23% | -0.17% | -0.16% | 99% | 100% |

Conclusion

In conclusion, it is proposed to derive the intra prediction mode for the BDPCM based on the actual prediction mode, which is either the horizontal or vertical mode. Furthermore, the use of line CG for the BDPCM block is proposed. The first proposed method is aligned with the existing intra prediction derivation process and also improves the coding performance. On top of that, the proposed CG for the BDPCM brings coding efficiency itself, and the additive coding gain can be achieved if the proposed methods are considered together. It is recommended for the proposed methods to be adopted in the next version of VVC and VTM.

Reference

- M. Karczewicz, M. Coban, “CE8-related: Quantized residual BDPCM,” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N0413, 14th Meeting: Geneva, CH, 19–27 March 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1001, 14th Meeting: Geneva, CH, 19–27 March 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).