| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0207-v1 |
| --- | --- |

| Title: | Non-CE9: A bug fix of BDOF specification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

At the 19th Geneva meeting, there was a discussion about the BDOF specifications when bitdepth is greater than 12-bit. However, since there was not enough time, the verification of specification was insufficient. This contribution proposes a bug fix for the BDOF specification, when bitdepth is greater than 12-bit. The current WD specification seems to be designed to keep internal bit precision constantly even if bitdepth is greater than 12-bit. However, it is argued that the gradient precision is not appropriate. Also, it is suggested that the threshold value of the vector should be fixed value if the internal bit precision is constant.  Unfortunately since VTM Software has not supported operations over 13-bit of bitdepth, it cannot be experimented. The specification and experimental results in bitdepth from 8-bit to 12-bit including CTC are not changed.

Problem Statement

In this section, we point out a problem of the current BDOF specification [1] and proposes a solution. Table 1 and Table 2 show the value and precision per bitdepth each variable in the current specification. A problem is that since the precision of variables tempH and tempV decreases as the variable bitDepth increases, the solution of the optical flow (vx, .vy) has been broken, when the variable bitDepth is greater than 12-bit. In our understanding, the precision of variables tempH and tempV should be constant, even if the variable bitDepth is greater than 12-bit to derive the optical flow correctly. If the precisions of variables tempH, tempV and Diff per bitdepth are constant, the precision of optical flow (vx, .vy) is constant. Therefore, the variable mvRefileThres shall be fixed to 32.

As a result, the following modification should be introduced.

- Fix the variable shift1 to 6.

- Fix the variable mvRefineThres to 32.

Table 1	bitDepth of each variable specified in the WD specification

|  | bitDepth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| shift1 | 6 | 6 | 6 | 6 | 6 | 7 | 8 | 9 | 10 |
| shift2 | 4 | 4 | 4 | 4 | 4 | 5 | 6 | 7 | 8 |
| shift3 | 1 | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 5 |
| shift4 | 7 | 6 | 5 | 4 | 3 | 3 | 3 | 3 | 3 |
| mvRefineThres | 32 | 32 | 32 | 32 | 32 | 64 | 128 | 256 | 512 |

Table 2	The precision (dynamic range) per bitDepth of each variable derived by the WD specification

|  | bitDepth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| predSamplesL0 predSamplesL1 | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 15 (17) | 16 (18) | 17 (19) | 18 (20) |
| gradientHL0 gradientVL0 gradientHL1 gradientVL1 | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) |
| Diff | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) |
| tempH tempV | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 8 (10) | 7 (9) | 6 (8) | 5 (7) |
| bdofOffset | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 15 (17) |  |  |  |  |
| pbSamples | 8 | 9 | 10 | 11 | 12 |  |  |  |  |

Table 3 and Table 4 show the value and precision per bitdepth of each variable in proposed specification. As the variable shift1 is fixed to 6, the precision of variables tempH and tempV per bitdepth becomes constant, when the variable bitDepth is greater than 12-bit, the optical flow (vx, .vy) is derived correctly. Since the variable mvRefineThres is fixed to 32, the precision of optical flow (vx, .vy) is constant. On the other hand, as the precision of variables grandientHL0, VL0, HL1 and VL1 is proportional to that of variables predSamplesL0 and L1, the variable bdofOffset is derived correctly.

Table 3 	The precision (dynamic range) per bitDepth of each variable in proposed bugfix

|  | bitDepth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| shift1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| shift2 | 4 | 4 | 4 | 4 | 4 | 5 | 6 | 7 | 8 |
| shift3 | 1 | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 5 |
| shift4 | 7 | 6 | 5 | 4 | 3 | 3 | 3 | 3 | 3 |
| mvRefineThres | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 32 |

Table 4 	The precision (dynamic range) per bitDepth of each variable in proposed bugfix

|  | bitDepth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| predSamplesL0 predSamplesL1 | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 15 (17) | 16 (18) | 17 (19) | 18 (20) |
| gradientHL0 gradientVL0 gradientHL1 gradientVL1 | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 10 (12) | 11 (13) | 12 (14) | 13 (16) |
| Diff | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) |
| tempH tempV | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) |
| bdofOffset | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 16 (18) | 17 (19) | 18 (20) | 19 (21) |
| pbSamples | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

Proposal changes of specification

The working draft [2] should be changed by the followings.

#### 8.5.6.4	Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1.

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 6, bitDepth − 6 )6.

- The variable shift2 is set to equal to Max( 4, bitDepth − 8 ).

- The variable shift3 is set to equal to Max( 1, bitDepth − 11 ).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to 1 << Max( 5, bitDepth − 7 )32.

For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1.

- If bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 +	(8-766)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift4 )

- Otherwise (bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

  - For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(8-767)

vy = Clip3( 1, nCbH, y )		(8-768)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1	(8-769)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1	(8-770)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1	(8-771)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1	(8-772)

- The variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 )	(8-773)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3	(8-774)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3	(8-775)

  - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-776)

sGy2 = ij( tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-777)

sGxGy = ij( tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-778)

sGxGym = sGxGy >> 12		(8-779)

sGxGys =  sGxGy & ( ( 1 << 12 ) − 1 )		(8-780)

sGxdI = ij( −tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-781)

sGydI = ij( −tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-782)

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	(8-783)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	(8-784)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGx2 ) ) )  :  0

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset = Round( ( vx * ( gradientHL1[ x + 1 ][ y + 1 ] − gradientHL0[ x + 1 ][ y + 1 ] ) ) >> 1 )	(8-785)
	+ Round( ( vy * (gradientVL1[ x + 1 ][ y + 1 ] − gradientVL0[ x + 1 ][ y + 1 ] ) ) >> 1 )

[Ed. (JC): Round() operation is defined for float input. The Round() operation seems redundant here since the input is an integer value. To be confirmed by the proponent]

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	(8-786)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

Conclusion

In this contribution, we proposed a bug fix for BODF specification. We would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang, CE9-related: CE9-related: Improvements on bi-directional optical flow (BDOF), Joint Video Experts Team Document, JVET-N0325r2, Geneva, April 2019.

[2]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 5), Joint Video Experts Team Document, JVET-N1001-v8, Geneva, April 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).