| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0212-v1 |
| --- | --- |

| Title: | On an enable condition for LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

LMCS (luma mapping with chroma scaling) is a coding tool that performs in-loop mapping for luma and in-loop scaling for chroma; however, it seems to be assumed that the histogram of sequence is constant for a certain period. As an experimental result, LMCS was not effective for fade sequences that is changed the histogram temporally. Therefore, in this contribution, an enable condition on the syntax of LMCS is proposed. If any flag of WP (weighted prediction), which is an effective tool for fading sequences, is not true, LMCS can be applied. As experimental results using black and white fade sequences proposed at the Macao meeting, compared to VTM-5.0 when WP is applied, average of 2.66%/6.49% improvements on RA/LB configurations for black fade and average of 0.30%/1.61% improvements on RA/LB configurations for white fade have been achieved.

Introduction

Although LMCS performs in-loop mapping for luma and in-loop scaling for chroma, it seems to be assumed that the histogram of sequence is constant for a certain period. In fact, as experimental results using fade sequences that are changed the histogram temporally, there was no gain. Since WP is an effective tool for fade sequences, we would like to propose a solution that both LMCS and WP should not be applied simultaneously at this time. In current specification [1], slice_lmcs_enabled_flag on slice header which is an enable flag of LMCS and weighted_pred_flag and weighted_bipred_flag on PPS are enable flags of WP. Since PPS is a higher layer than slice header, slice_lmcs_enabled_flag should be applied when neither weighted_pred_flag nor weighted_bipred_flag is true. In this contribution, an enable condition on the syntax of LMCS is proposed and the improvement of coding efficiency is shown by using black and white fade sequences [2] proposed at the Macao meeting.

Proposal changes of specification

The working draft [1] should be changed by the followings.

7.3.5	Slice header syntax

7.3.5.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| if( sps_lmcs_enabled_flag && !( weighted_pred_flag ¦ ¦ weighted_bipred_flag ) ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(5) |
| if( !( qtbtt_dual_tree_intra_flag && slice_type = = I ) ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |

Experimental results

Fade sequences

Fade sequences are generated with a fading tool in JVET-L0201-v1 [2]. In our experiments, a linear fade is applied to the about first 2 seconds of the regular sequence set and the experiments are conducted with them. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half. The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs. The number of fade sequence frames is decided by following Table 1.

Table 1	The number of fade sequence frames against frame rate

| Frame Rate | The number of fade sequence frames |
| --- | --- |
| 20 | 33 |
| 30 | 65 |
| 50 | 97 |
| 60 | 129 |

Experimental results

The proposed method has been implemented on VTM-5.0 software. The simulation was based on VTM configuration JVET-L1010 [3] with above black and white fade sequences. Regarding WP parameters on VTM-5.0, we set the following parameters.

--WeightedPredMethod=0 --WeightedPredB=1 --WeightedPredP=1

The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

As a first experiment, we have tested three cases, which are LMCS off, WP and our proposed condition that if any flag of WP is not true, LMCS is not applied, compared to VTM-5.0. As a second experiment, we have compared our proposed condition to VTM-5.0 when WP is on.

Table 2  Summary of experimental results for black fade sequences on RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| LMCS off | -1.86% | -2.82% | -2.43% | 93% | 99% |
| WP | -6.38% | -10.30% | -9.75% | 176% | 84% |
| Proposed | -8.94% | -13.84% | -12.98% | 166% | 80% |

Table 3  Summary of experimental results for white fade sequences on RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| LMCS off | 0.31% | -2.27% | -2.09% | 95% | 100% |
| WP | -8.08% | -12.39% | -12.02% | 171% | 83% |
| Proposed | -8.39% | -15.19% | -14.63% | 164% | 79% |

Table 4  Summary of experimental results for black fade sequences on LB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| LMCS off | -4.54% | -3.40% | -3.60% | 91% | 102% |
| WP | -27.30% | -37.60% | -36.69% | 105% | 87% |
| Proposed | -31.95% | -40.40% | -39.90% | 98% | 80% |

Table 5  Summary of experimental results for white fade sequences on LB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| LMCS off | -0.88% | -1.89% | -2.27% | 95% | 102% |
| WP | -28.89% | -38.08% | -37.78% | 99% | 85% |
| Proposed | -29.93% | -39.97% | -39.72% | 95% | 79% |

Table 6  Summary of experimental results for black fade sequences (compared to WP)

|  | Over VTM-5.0_WP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |  |
| Proposed (RA) | -2.66% | -3.91% | -3.50% | 94% | 95% |
| Proposed (LB) | -6.49% | -4.51% | -5.36% | 93% | 92% |

Table 7  Summary of experimental results for white fade sequences (compared to WP)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0_WPM0+WPB+WPP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Proposed (RA) | -0.30% | -3.06% | -2.85% | 96% | 96% |
| Proposed (LB) | -1.61% | -3.28% | -3.38% | 96% | 93% |

Conclusion

In this contribution, an enable condition for LMCS has been proposed. As experimental results for fade sequences, the coding efficiency has been improved by introducing a proposed method. We would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 5), Joint Video Experts Team Document, JVET-N1001-v8, Geneva, June 2019.

[2]	P. Bordes, E. François and F. L. Léannec, AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences, Joint Video Exploration Team Document, JVET-L0201, Macao, Oct. 2018.

[3]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-N1010, Geneva, April 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).