| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0215 |
| --- | --- |

| Title: | AHG12: Clean up on wavefront parallel processing signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Eiichi Sasaki Takeshi Chujoh | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

At the last meeting, the wavefront parallel processing has been adopted. This contribution proposes a syntax restriction and provides range of num_entry_point_offsets as follows.

- entropy_coding_sync_enabled_flag is only signalled when rect_slice_flag is equal to 1

- num_entry_point_offsets shall be 0 or total number of CTU row minus 1

Introduction

At the last meeting discussion in HLG BoG, wavefront parallel processing has been discussed and it was agreed that entropy_coding_sync_enabled_flag should be included in picture parameter set only and raster-scan-order tile’s support for single tile in picture. However, it seemed some confusion. Specifically single tile in picture and single tile group in tile is actually rectangular tile. The author believes using raster-scan-order tile (segment can be divided in different positions of CTU row) and wavefront processing (CTU row base parallelism) brings unnecessary confusions without no obvious benefit. Thus we propose a syntax restriction as follows.

In addition, in the discussion with the editors on the adopted wavefront proessing in JVET-N0150, the range of a syntax element of num_entry_point_offsets is missing. We propose a range.

Proposal

Firstly, we propose a syntax restriction as follows.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rect_slice_flag ) { |  |
| entropy_coding_sync_enabled_flag | u(1) |
| } |  |

It is noted that rect_slice_flag is equal to 1 when single_tile_in_pic_flag is equal to 1 or single_brick_per_slice_flag is equal to 1. Thus entropy_coding_sync_enabled_flag usage in use case would not be affected.

Secondly, we propose the following range as suggested in the last meeting.

- num_entry_point_offsets shall be 0 or total number of CTU row minus 1

Proposed text changes

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_brick_idx[ i ] | u(v) |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| if( rect_slice_flag ) { |  |
| entropy_coding_sync_enabled_flag | u(1) |
| } |  |
| cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| rpl1_idx_present_flag | u(1) |
| init_qp_minus26 | se(v) |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_entry_point_offsets is used to specify the variable NumEntryPoints, which specifies the number of entry points in the current slice as follows:

NumEntryPoints = entropy_coding_sync_enabled_flag ? num_entry_point_offsets :
			NumBricksInCurrSlice − 1		(7-62)

[Ed. (YK): The value range of the ue(v)-coded num_entry_point_offsets is missing.]

It is a requirement of bitstream conformance that the value of num_entry_point_offsets shall be either 0 or the total number of brick-specific luma CTU rows in the slice – 1.

References

[1] T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono, “AHG12: Wavefront processing in a tile group”, JVET-M0070

[2] T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono, “AHG12: Signalling on wavefront parallel processing”, JVET-N0149

[3] T. Ikai, “AHG12: One CTU delay wavefront parallel processing”, JVET-N0150

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).