| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0221 |
| --- | --- |

| Title: | Non-CE4: On merge data syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Du Liu Torshamnsgatan 23, 164 80 Stockholm, Sweden | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In the merge data syntax of VTM-5.0, the signalling order of different merge alternatives is regular merge, MMVD, subblock merge, CIIP and triangle merge. It is asserted that there are two drawbacks with such design: 1) The merge alternatives such as MMVD, subblock merge, CIIP and triangle can all be turned off either on a sequence level or on a slice level. With the existing VTM-5.0 syntax, redundant bits would be spent for the merge alternatives that precede the alternatives that get disabled. 2) The behavior of decoding is not properly defined when all the merge alternatives are signalled to be off for a CU. The proposal discusses around these drawbacks and presents several options for consideration.

# Proposal

In the merge data syntax of VTM-5.0, the syntax signalling order of different merge alternatives is regular merge, MMVD, subblock merge, CIIP and triangular merge. It is reported that there are two drawbacks with such design:

1) The merge alternatives such as MMVD, subblock merge, CIIP and triangle can all be turned off either on a sequence level or a slice level. With the existing VTM-5.0 syntax, redundant bits would be spent for the merge alternatives that have their corresponding syntax signalled preceding the syntax of the alternatives that gets disabled.

2) The behavior of decoding is not properly defined when all the merge alternatives are signalled to be off for a CU.

## On drawback #1

To resolve the drawback #1, one option would be to modify the presence check of the preceding merge alternatives to also consider the enabling flags or presence criterions of the succeeding merge alternatives.

One example shown below demonstrates redundant bits that could be saved for subblock merge, CIIP and MMVD by utilizing the knowledge that triangle merge is not present when P slice is used:

- The CIIP flag can be inferred to 1 when

  - The CU is in non-skip mode, regular_merge_flag = 0, mmvd_merge_flag = 0 and merge_subblock_flag = 0.

- The merge_subblock_flag can be inferred to 1 when either the following is true

  - The CU is in skip mode, regular_merge_flag = 0 and mmvd_merge_flag = 0

  - The CU is in non-skip mode, regular_merge_flag = 0, mmvd_merge_flag = 0 and the width of the CU equals 128 or the height of the CU equals 128.

- The mmvd_merge_flag can be inferred to 1 when

  - The CU is in skip mode, regular_merge_flag = 0 and the width of the CU equals 4 or the height of the CU equals 4.

The table below shows the BD-rate impact of the above changes under the LDP common test configurations. The anchor is VTM-5.0. The encoder as well as decoder run time are not accurate. As the changes only related to P picture, the performance under RA and LDB configuration are not affected.

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.18% | -0.33% | 102% | 113% |
| Class C | -0.23% | -0.37% | -0.06% | 101% | 111% |
| Class E | -0.28% | -0.01% | -0.43% | 103% | 127% |
| Overall | -0.22% | -0.20% | -0.26% | 102% | 116% |
| Class D | -0.29% | -0.97% | 0.25% | 101% | 113% |
| Class F | -0.02% | -0.31% | -0.09% | 103% | 124% |

The following shows the specification text changes.

7.3.7.7	Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag ¦ ¦ cbWidth * cbHeight != 32 ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag && cbWidth * cbHeight != 32 && ! (slice_type == P && cu_skip_flag[ x0 ][ y0 ] == 1 && ( cbWidth == 4 ¦¦ cbHeight == 4 ) ) ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 && !( slice_type == P && (cu_skip_flag[ x0 ][ y0 ] == 1 ¦¦ (cu_skip_flag[ x0 ][ y0 ] == 0 && (cbWidth == 128 ¦¦ cbHeight == 128 ) ) ) ) ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 && ! ( slice_type == P ) ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.8.7 Merge data semantics … mmvd_merge_flag[ x0 ][ y0 ] equal to 1 specifies that merge mode with motion vector difference is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When mmvd_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows: - If all the following conditions are true, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1: - sps_mmvd_enabled_flag is equal to 1. - general_merge_flag[ x0 ][ y0 ] is equal to 1. - At least one of the following conditions is true: - cbWidth*cbHeight is equal to 32. - slice_type is equal to P, cu_skip_flag[ x0 ][ y0 ] is euqal to 1, cbWidth is equal to 4 or cbHeight is equal to 4. - regular_merge_flag[ x0 ][ y0 ] is equal to 0. - Otherwise, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0. … merge_subblock_flag[ x0 ][ y0 ] specifies whether the subblock-based inter prediction parameters for the current coding unit are inferred from neighbouring blocks. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When merge_subblock_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0 as follows: - If all the following conditions are true, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 1: - MaxNumSubblockMergeCand is greater than 0. - general_merge_flag[ x0 ][ y0 ] is equal to 1. - regular_merge_flag[ x0 ][ y0 ] is equal to 0. - mmvd_merge_flag[ x0 ][ y0 ] is equal to 0. - slice_type is equal to P. - At least one of the following conditions is true - cu_skip_flag[ x0 ][ y0 ] is equal to 1 - cu_skip_flag[ x0 ][ y0 ] is equal to 0, cbWidth is equal to 128 or cbHeight is equal to 128. - Otherwise, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 0. … ciip_flag[ x0 ][ y0 ] specifies whether the combined inter-picture merge and intra-picture prediction is applied for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When ciip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0 as follows: - If all the following conditions are true, ciip_flag [ x0 ][ y0 ] is inferred to be equal to 1: - sps_ciip_enabled_flag is equal to 1 - cu_skip_flag[ x0 ][ y0 ] is equal to 0 - general_merge_flag[ x0 ][ y0 ] is equal to 1 - regular_merge_flag[ x0 ][ y0 ] is equal to 0 - mmvd_merge_flag[ x0 ][ y0 ] is equal to 0 - merge_subblock_flag[ x0 ][ y0 ] is equal to 0 - cbWidth * cbHeight is greater than 64 - cbWidth is smaller than 128 and cbHeight is smaller than 128 - slice_type is equal to P. - Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be 0. …

Furthermore, to throughout solve the redundant bits drawback, the sequence level enabling flags as well as slice level enabling flags also need to be considered. It is further asserted that it would introduce large amount of conditional checks when checking the presence of flags. The changes that shown above only resolves one part of the drawback, but the specification text modification is already quite tedious.

## On drawback #2

To resolve the drawback #2, another option would be to add a bitstream conformance constraint that when general_merge_flag[ x0 ][ y0 ] is equal to 1, there should be one of the following flags equal to 1:

- regular_merge_flag[ x0 ][ y0 ]

- mmvd_merge_flag[ x0 ][ y0 ]

- merge_subblock_flag[ x0 ][ y0 ]

- ciip_flag[ x0 ][ y0 ]

- MergeTriangleFlag[ x0 ][ y0 ]

## Move the regular merge to the end of merge data syntax to resolve both drawbacks

The third option would be to move syntax related to regular merge mode to the end of the merge data syntax.

It is asserted that the approach would not introduce the two drawbacks discussed above. This is because that the regular merge mode is always an available option, i.e., it cannot be switched off either on sequence level or slice level.

The table below shows the BD-rate impact of the moving the regular merge to the end of merge data syntax under the RA and LDB common test configurations. The anchor is VTM-5.0. The encoder as well as decoder run time are not accurate.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.29% | -0.07% | 101% | 111% |
| Class A2 | -0.02% | -0.02% | -0.02% | 102% | 109% |
| Class B | 0.05% | 0.12% | -0.15% | 99% | 107% |
| Class C | 0.09% | 0.01% | -0.06% | 99% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.02% | -0.08% | 100% | 107% |
| Class D | 0.20% | 0.27% | 0.52% | 100% | 103% |
| Class F | 0.15% | 0.20% | 0.15% | 104% | 109% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.17% | 0.22% | 101% | 110% |
| Class C | 0.05% | 0.03% | 0.39% | 103% | 115% |
| Class E | 0.40% | 0.86% | 0.39% | 96% | 138% |
| Overall | 0.14% | 0.30% | 0.32% | 100% | 118% |
| Class D | -0.15% | -0.16% | 1.07% | 101% | 115% |
| Class F | 0.66% | 0.97% | 0.22% | 102% | 133% |

# Conclusion

This contribution discusses several options for resolving the drawbacks of merge data signalling in VTM-5.0. Among these options, the option which moves the regular merge to the end of merge data syntax signalling is the most straightforward.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).