| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0222 |
| --- | --- |

| Title: | Non-CE4: Modification of triangle merge mode enabling criterion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu Ruoyang Yu | Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-5.0, the triangular merge mode divides a coding unit into two triangular partitions. Uni-directional motion compensation is applied for each triangular partition and the diagonal edge between the two triangular partitions is blended by weighting matrices. The triangular merge mode is allowed for CU with either of its dimension greater than 64. To accommodate VPDU pipeline design but still realize the correct blending, different weighting matrices need to be applied for each VPDU since the blending area for each VPDU is different. The contribution proposes to disable the triangular merge mode when width equals 128 or height equals 128, such that the handling for each VPDU is unified and storage of weighting matrices for different VPDU is saved. The proposed method was tested under the VTM-5.0 random access (RA) and low delay (LD) under common test configuration. The BD-rate impact is -0.02% for RA and 0.03% for LD.

# Introduction

In VTM-5.0, triangle merge mode allows a merge coding unit (CU) into two triangular partitions. Each triangular partition is associated with a uni-directional MV for motion compensation. When a CU fulfills the following conditions, triangular merge mode is applied:

- Triangle merge mode is enabled in SPS

- Slice type is B.

- Merge mode is enabled.

- The maximum number of triangular merge candidate is at least 2.

- width * height is greater than or equal to 64.

- The CU is not coded using regular merge, MMVD merge, merge subblock, or CIIP.

To reduce the edge effect, the diagonal edge between the two triangular partitions needs to be blended. Fig. 1 depicts an example of triangular partitioning from top left to bottom right for a square block. The green area, i.e., the diagonal location ±3 for each row, corresponds to the pixel locations that need blending, and the weighting matrix in Fig. 1 (b) and (c) are the weights that are used to blend. Let the top right triangle be associated with MV0, a reference block Ref0, and a weighting matrix w0, and bottom left triangle associated with MV1, a reference block Ref1, and a weighting matrix w1. The predicted value is given by

pi,j=Ref0i,j * w0i,j+Ref1i,j * w1i,j/8

= Ref0i,j * w0i,j+Ref1i,j * (8-w1i,j)/8.

This equation holds for pixel locations both inside and outside the green area.

(a)                              (b)                          (c)

Fig.1. An example of triangular partitioning from top-left to bottom-right and the corresponding weighting matrices w0 and w1.

(a)                            (b)

(c)                           (d)

Fig. 2.  Partition of 128x128 weighting matrices into four 64x64 matrices.

In VPDU pipeline design, the virtual 64x64 data unit affects the processing of blocks with width or height larger than 64. Since the large blocks need to be divided into multiple 64x64 blocks, processing across the 64x64 boundary is not preferred.

For triangular merge mode, the size condition is only width * height larger than or equal to 64, thus, it is possible to have CUs of sizes 128xN and Nx128 that exceed the VPDU size. Different weighting matrices need to be stored and used for each VPDU, as the blending area are different.

For example, when a CU is in size 128x128, it needs to be splitted into four 64x64 units to accommodate the VPDU pipeline. For each VPDU, different weighting matrices are required. For the partition in Fig. 2(a), it needs four different 64x64 weighting matrices for w0 (as shown in Fig. 2(b)), and for the partition in Fig. 2(c), it needs another four weighting matrices (as shown in Fig. 2(d)). Thus, eight 64x64 weighting matrices need to be stored.

Similarly, for a CU of size 128xN (or Nx128), N<128, w0 is splitted into two 64xN (or Nx64) matrices. With two possible triangular partitions, w0 need to be stored in four different matrices, i.e., four matrices of size 64xN (or Nx64).

# Proposed Method

To unify the handling of each VPDU and reduce the memory requirement for saving different weighting matrices, it is proposed to disable triangular merge mode when the CU width or height is equal to 128.

One condition marked with yellow is added to the specific text as:

#### 7.4.8.7 Merge data semantics

…

- If all the following conditions are true, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 1:

- sps_triangle_enabled_flag is equal to 1.

- slice_type is equal to B.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- MaxNumTriangleMergeCand is greater than or equal to 2.

- cbWidth * cbHeight is greater than or equal to 64.

- cbWidth is smaller than 128 and cbHeight is smaller than 128.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- ciip_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 0.

# Results

The proposed method was implemented in VTM-5.0 reference software [1]. The simulations were run using the VTM common test configurations [2]. The encoder and decoder run time were not accurate.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.15% | 0.06% | 98% | 98% |
| Class A2 | -0.03% | 0.12% | 0.02% | 100% | 101% |
| Class B | 0.00% | 0.03% | -0.03% | 100% | 102% |
| Class C | -0.01% | -0.04% | -0.04% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 107% |
| Class F | 0.02% | 0.02% | 0.03% | 96% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.19% | -0.03% | 101% | 102% |
| Class C | 0.04% | -0.10% | -0.07% | 102% | 103% |
| Class E | 0.07% | 0.11% | -0.26% | 93% | 95% |
| Overall | 0.03% | 0.07% | -0.10% | 99% | 100% |
| Class D | -0.01% | 0.02% | 0.11% | 101% | 101% |
| Class F | 0.03% | -0.25% | -0.23% | 98% | 101% |

# Conclusion

The proposed method disables triangular merge mode for CU width or height equals 128. The results show that the BD-rate impact for RA is -0.02% and for LDB is 0.03%. It is worth noticing that for 4K resolution class A1 and A2, the BD-rate impact is -0.05% and -0.03%, respectively. We recommend adopting the proposed method into the next VTM software and VVC working draft.

# Reference

[1]. VTM-5.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).