| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0223-v2 |
| --- | --- |

| Title: | Non-CE7: Quantization matrices with single identifier and prediction from larger ones |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Fabrice Leleannec, Edouard François, Karam Naser | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify quantization matrix (QM) signalling and prediction, while enhancing them so that any matrix can be predicted from any previously signalled one, by: identifying a QM with a single index, transmitting QMs in decreasing block size order, and specifying the prediction process as either a copy or decimation process.

It also describes a unified QM derivation process that encompasses selecting a decoded QM for a given transform block, up-sampling for blocks larger that 8x8, and down-sampling for rectangular blocks.

These modifications are said to reduce by half the text needed to support QMs, and to save about 50% of the bits needed to signal QMs from a provided test set.

# Introduction

Quantization matrices (QMs) were introduced in VVC during the 14th JVET meeting, based on contribution JVET-N0847. Compared to HEVC, VVC needs more quantization matrices due to a higher number of block sizes. Custom QMs can be signaled in SPS or PPS, and prediction can be used to reduce bit cost. However, QM prediction is limited to same block-size matrices, which can result in wasted bits. Moreover, the syntax is more complex because of the usage in VVC of block size 2x2 for chroma only, and of block size 64x64 for luma only. Also, JVET-N0847, as HEVC, describes a specific derivation process for each size.

This contribution proposes to simplify QM signalling and prediction process, while enhancing them so that any matrix can be predicted from any previously signalled one, by:

- Unifying matrix index to encompass both size and type, so that a reference index difference can address any previously transmitted one;

- Transmitting quantization matrices in decreasing block size order;

- Specifying the prediction process as either a copy or decimation process, as needed.

Besides, a unified QM derivation process is described, that encompasses selecting a decoded QM for a given transform block, up-sampling for blocks larger that 8x8, and down-sampling for rectangular blocks.

# Proposal description

The proposal contains changes related to:

- The derivation and usage of matrix index for identifying a QM

- The QM prediction process

- The transmission of all coefficients for size 64

- The QM selection and resize process

These aspects are presented in the sub-sections below.

## Unified matrix index

In the current VVC design (VTM-5.0, after adoption of JVET-N0847), a QM is identified by two parameters, matrixId and sizeId.

In the current proposal, the QM is identified by one single parameter matrixId. The proposed unified matrixId (matrix index) is a compound of:

- a size identifier (sizeId in Table 3) which relates to transform “unit” size rather than “block” size (or enclosing square shape, because only square-size matrices are transmitted). QM size identifier then relates to size of luma block, or chroma block projected on luma plane.

- a matrix type identifier (matrixTypeId in Table 4) which first lists luma QMs, because they are larger than chroma ones (QMs are transmitted in 4:2:0 chroma format)

The proposed modifications are illustrated in tables 1 to 4 and equation (2-1).

Table 1: Size identifier (VTM-5.0)

| Luma | Chroma | sizeId |
| --- | --- | --- |
| - | - | 0 |
| - | 2x2 | 1 |
| 4x4 | 4x4 | 2 |
| 8x8 | 8x8 | 3 |
| 16x16 | 16x16 | 4 |
| 32x32 | 32x32 | 5 |
| 64x64 | - | 6 |

Table 2: Matrix type identifier (VTM-5.0)

| CuPredMode | cIdx (Colour component) | matrixId |
| --- | --- | --- |
| MODE_INTRA | 0 (Y) | 0 |
| MODE_INTRA | 1 (Cb) | 1 |
| MODE_INTRA | 2 (Cr) | 2 |
| MODE_INTER | 0 (Y) | 3 |
| MODE_INTER | 1 (Cb) | 4 |
| MODE_INTER | 2 (Cr) | 5 |

Table 3: Size identifier (proposed)

| Luma | Chroma | sizeId |
| --- | --- | --- |
| 64x64 | 32x32 | 0 |
| 32x32 | 16x16 | 1 |
| 16x16 | 8x8 | 2 |
| 8x8 | 4x4 | 3 |
| 4x4 | 2x2 | 4 |

Table 4: Matrix type identifier (proposed)

| CuPredMode | cIdx (Colour component) | matrixTypeId |
| --- | --- | --- |
| MODE_INTRA | 0 (Y) | 0 |
| MODE_INTER | 0 (Y) | 1 |
| MODE_INTRA | 1 (Cb) | 2 |
| MODE_INTER | 1 (Cb) | 3 |
| MODE_INTRA | 2 (Cr) | 4 |
| MODE_INTER | 2 (Cr) | 5 |

The proposed unified matrixId is derived as follows:

matrixId = 6 * sizeId + matrixTypeId		(2-1)

A summary of quantization matrix identification changes is illustrated in the following two tables:

Table 5: (matrixId, sizeId) combinations (current)

| INTRA | Y |  | 0, 2 | 0, 3 | 0, 4 | 0, 5 | 0, 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cb | 1, 1 | 1, 2 | 1, 3 | 1, 4 | 1, 5 |  |
|  | Cr | 2, 1 | 2, 2 | 2, 3 | 2, 4 | 2, 5 |  |
| INTER | Y |  | 3, 2 | 3, 3 | 3, 4 | 3, 5 | 3, 6 |
|  | Cb | 4, 1 | 4, 2 | 4, 3 | 4, 4 | 4, 5 |  |
|  | Cr | 5, 1 | 5, 2 | 5, 3 | 5, 4 | 5, 5 |  |
| Block size: max(width,height) |  | 2 | 4 | 8 | 16 | 32 | 64 |
| Signalled QM size |  | 2x2 | 4x4 | 8x8 | 8x8 + DC |  |  |

Table 6: unified matrixId (proposed)

| Y | INTRA | 0 |  | 6 |  | 12 |  | 18 |  | 24 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | INTER | 1 |  | 7 |  | 13 |  | 19 |  | 25 |  |
| Cb | INTRA |  | 2 |  | 8 |  | 14 |  | 20 |  | 26 |
|  | INTER |  | 3 |  | 9 |  | 15 |  | 21 |  | 27 |
| Cr | INTRA |  | 4 |  | 10 |  | 16 |  | 22 |  | 28 |
|  | INTER |  | 5 |  | 11 |  | 17 |  | 23 |  | 29 |
| TU size: luma max(width,height) |  | 64 |  | 32 |  | 16 |  | 8 |  | 4 |  |
| Block size: max(width,height) |  | 64 | 32 |  | 16 |  | 8 |  | 4 |  | 2 |
| Signalled QM size |  | 8x8 + DC |  |  |  |  | 8x8 |  | 4x4 |  | 2x2 |

Note: adding QMs for other transform types (MTS) is made easy, as they can be inserted after the row “Y INTER”, with minor impact on draft text.

Note2: for the following, we refer to QMs for a given family of block sizes (square or rectangular) as size-N, in relation to sizeId and the square block size it is used for: for example, for TU sizes 16x16 or 16x4, the QMs are identified as size-16 (sizeId 2 in this proposal). The size-N notation is used to differentiate from exact block shape, and from the number of signaled QM coefficients (limited to 8x8, as shown in tables 5 and 6).

## QM prediction process

When signaling custom QMs in scaling_list_data syntax, instead of transmitting QM coefficients, it is possible to predict a QM either from default values, or from any previously transmitted one.

- When the reference QM is of the same size, it is copied, otherwise it is decimated by the relevant ratio, as illustrated in example 1 below

- When the reference QM has a DC value

  - If the current QM needs a DC value, it is copied as DC value

  - Otherwise, it is copied to the top-left QM coefficient

Corresponding draft text is given in §3.4.

### Example 1: prediction of size-4 QM from size-8

In this example, a luma size-4 QM (which is a 4x4 array) is predicted from a luma size-8 QM, which is an 8x8 array. One line out of two and one column out of two are dropped to generate the 4x4 array.

Figure 1: prediction from larger size (decimation)

## Transmission of all coefficients for size-64

To enable prediction of smaller QMs from size-64, all coefficients for size-64 are transmitted in the scaling list syntax, even though the bottom-right quadrant is never used by VVC transformation and quantization process.

This means the specific syntax and zero-out process for size-64 present in VTM-5.0 / JVET-N0847 is removed.

However, it is worth noting that the overhead related to this increased number of transmitted coefficients compared to VTM-5.0 / JVET-N0847 can be limited to 2x16 bits in the worst case, since the syntax element scaling_list_delta_coef can be set to zero for the bottom-right quadrant of a size-64 QM when it is not used as a predictor: for bottom-right quadrant, 4x4 = 16 delta-coefficients are signaled, each taking 1-bit if forced to zero (coded with exp-Golomb), and there are two size-64 QMs (luma intra/inter).

Tests reported in section 4 show that this overhead is marginal compared to the gains brought by prediction improvement.

## Unified QM derivation process

The proposed derivation process for quantization matrix first selects the right QM index depending on block parameters, as described in §2.1 (unified matrix index), then unifies the processes of decimation for rectangular blocks, repetition for blocks larger than 8x8, and chroma format adaptation into one single process.

The proposed process is based on bit-shifts of x and y output coordinates, as described at the end of §3.5. To select the right line/column of the reference matrix, only left shift followed by right shift of x/y output coordinates are required, as illustrated in equation below (excerpt from §3.5).

m[ x ][ y ] = ScalingMatrix[ i ][ j ]		(2-2)
	with i = ( x << log2MatrixSize ) >> log2( blkWidth ),
	and j = ( y << log2MatrixSize ) >> log2( blkHeight )

### Example 2: QM for 16x8 luma block

In this example, the QM for a luma 16x8 block is derived from a luma size-16 QM, which is actually an 8x8 array plus a DC coefficient; blkWidth equals 16, blkHeight equals 8, and log2MatrixSize equals 3, hence equation (2-2) takes the following form:

m[ x ][ y ] = ScalingMatrix[ matrixId ][ i ][ j ]
	with i = ( x << 3 ) >> 4,
	and j = ( y << 3 ) >> 3

where x = 0..15 and y = 0..7

x is right-shifted by 1, and y is unchanged (i.e., column i in the decoded QM is copied to columns 2*i and 2*i + 1 in the current QM). Additionally, since the decoded QM has a DC coefficient, it is copied to m[0][0].

Figure 2: combining repetition for block larger than 8x8 and decimation for rectangular block

### Example 3: chroma block for 8x4 TU (4:2:0 format)

In this example, the QM for a chroma 4x2 block is generated. This matches 8x4 TU size, which enclosing square is 8x8. The selected QM is thus the size-8 one, where the chroma QMs are coded as 4x4 arrays; blkWidth equals 4, blkHeight equals 2, and log2MatrixSize equals 2, hence equation (2-2) takes the following form:

m[ x ][ y ] = ScalingMatrix[ matrixId ][ i ][ j ]
	with i = ( x << 2 ) >> 2, and
	j = ( y << 2 ) >> 1

where x = 0..3 and y = 0..1

x is unchanged, and y is left-shifted by 1.

Figure 3: rectangular block for chroma

# Draft text

## Complexity comparison with JVET-N0847

The following table illustrates the amount of text needed to specify the support of QMs in VVC (statistics from Microsoft Word with all changes accepted):

|  | JVET-M1001-v7 | JVET-N0847 | Proposed |
| --- | --- | --- | --- |
| Pages | 300 | +5 | +3 |
| Paragraphs | 11386 | +502 | +102 |
| Lines | 16922 | +609 | +165 |
| Words | 150667 | +2660 | +1353 |
| Characters | 709054 | +10854 | +6804 |

The following table details the amount of text needed for each syntax structure of VVC to specify the support of QMs:

|  | JVET-N0847 | Proposed |
| --- | --- | --- |
| SPS syntax | +6 lines | +6 lines (same text) |
| SPS semantics | +5 lines | +5 lines (same text) |
| PPS syntax | +3 lines | +3 lines (same text) |
| PPS semantics | +8 lines | +8 lines (slight change) |
| Scaling list syntax | +28 lines, more complex than HEVC | +20 lines, simpler than HEVC (single loop, simple conditions) |
| Scaling list semantics | +145 lines | +64 lines |
| Scaling matrix derivation process | Included in semantics, with specific text for each matrix size | +20 lines |
| Decoding process (scaling) | -1/+7 lines | -1/+7, similar (adapted) |
| Other remarks | - Complex rules and dependency between sizeId and matrixId - Single default matrix for intra/inter - Specific text for rectangular block - New Raster2Diag scanning - 5-dimensional ScalingFactor - Zero-out process for 64x64 | - matrixId alone, single array of ScalingList - Prediction from any previously signalled QM with decimation - Separate intra/inter default matrices - Unified process for upscale / rectangular / 4:x:x chroma - No new scanning - No zero-out process |

## SPS syntax and semantics (same as adopted and HEVC)

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| … |  |
| } |  |

scaling_list_enabled_flag equal to 1 specifies that a scaling list is used for the scaling process for transform coefficients. scaling_list_enabled_flag equal to 0 specifies that scaling list is not used for the scaling process for transform coefficients.

sps_scaling_list_data_present_flag equal to 1 specifies that the scaling_list_data( ) syntax structure is present in the SPS. sps_scaling_list_data_present_flag equal to 0 specifies that the scaling_list_data( ) syntax structure is not present in the SPS. When not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0.

## PPS syntax and semantics (minor adaptation)

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| pps_extension_flag | u(1) |
| … |  |
| } |  |

pps_scaling_list_data_present_flag equal to 1 specifies that the scaling list data used for the pictures referring to the PPS are derived based on the scaling lists specified by the active SPS and the scaling lists specified by the PPS. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling list data used for the pictures referring to the PPS are inferred to be equal to those specified by the active SPS. When scaling_list_enabled_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enabled_flag is equal to 1, sps_scaling_list_data_present_flag is equal to 0 and pps_scaling_list_data_present_flag is equal to 0, the default scaling matrices are used to derive the array ScalingMatrix as described in the scaling list data semantics as specified in clause 7.4.5.

## Scaling list data syntax/semantics (simplified)

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( matrixId = 0; matrixId < 30; matrixId++ ) { |  |
| scaling_list_pred_mode_flag[ matrixId ] | u(1) |
| if ( !scaling_list_pred_mode_flag[ matrixId ] ) |  |
| scaling_list_pred_matrix_id_delta[ matrixId ] | ue(v) |
| else { |  |
| nextCoef = 8 |  |
| coefNum = (matrixId < 20) ? 64 : (matrixId < 26) ? 16 : 4 |  |
| if ( matrixId < 14 ) { |  |
| scaling_list_dc_coef_minus8[ matrixId ] | se(v) |
| nextCoef = scaling_list_dc_coef_minus8[ matrixId ] + 8 |  |
| } |  |
| for( i = 0; i < coefNum; i++ ) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| ScalingList[ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |

scaling_list_pred_mode_flag[ matrixId ] equal to 0 specifies that the scaling matrix is derived from the values of a reference scaling matrix. The reference scaling matrix is specified by scaling_list_pred_matrix_id_delta[ matrixId ]. scaling_list_pred_mode_flag[ matrixId ] equal to 1 specifies that the values of the scaling list are explicitly signalled.

scaling_list_pred_matrix_id_delta[ matrixId ] specifies the reference scaling matrix used to derive the scaling matrix, as follows. The value of scaling_list_pred_matrix_id_delta[ matrixId ] shall be in the range of 0 to matrixId, inclusive.

When scaling_list_pred_mode_flag[ matrixId ] is equal to zero:

- The variable refMatrixSize and the array refScalingMatrix are first derived as follows:

  - If scaling_list_pred_matrix_id_delta[ matrixId ] is equal to zero, the following applies to set default values:

    - refMatrixSize is set equal to 8,

    - if matrixId is an even number,

refScalingMatrix =		(3-1)

{

{ 16, 16, 16, 16, 16, 16, 16, 16 }		// placeholder for INTRA default values

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

},

    - otherwise

refScalingMatrix =		(3-2)

{

{ 16, 16, 16, 16, 16, 16, 16, 16 }		// placeholder for INTER default values

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

{ 16, 16, 16, 16, 16, 16, 16, 16 }

},

  - otherwise (if scaling_list_pred_matrix_id_delta[ matrixId ] is greater than zero), the following applies:

refMatrixId = matrixId - scaling_list_pred_matrix_id_delta[ matrixId ]	(3-3)

refMatrixSize = (refMatrixId < 20) ? 8 : (refMatrixId < 26) ? 4 : 2 )	(3-4)

refScalingMatrix = ScalingMatrix[ refMatrixId ]		(3-5)

- The array ScalingMatrix[ matrixId ] is then derived as follows:

ScalingMatrix[ matrixId ][ x ][ y ] = refScalingMatrix[ i ][ j ]	(3-6)
	with matrixSize = (matrixId < 20) ? 8 : (matrixId < 26) ? 4 : 2 )
	x = 0 .. matrixSize – 1, y = 0 .. matrixSize – 1,
	i = x << ( log2(refMatrixSize) - log2( matrixSize ) ), and
	j = y << ( log2(refMatrixSize) - log2( matrixSize ) )

scaling_list_dc_coef_minus8[ matrixId ] plus 8 specifies the first value of the scaling matrix when relevant, as described in clause xxx. The value of scaling_list_dc_coef_minus8[ matrixId ] shall be in the range of −7 to 247, inclusive.

When scaling_list_pred_mode_flag[ matrixId ] is equal to zero, scaling_list_pred_matrix_id_delta[ matrixId ] is greater than zero, and refMatrixId < 14, then the following applies:

- if matrixId < 14, scaling_list_dc_coef_minus8[ matrixId ] is inferred to be equal to scaling_list_dc_coef_minus8[ refMatrixId ],

- otherwise, ScalingMatrix[ matrixId ][ 0 ][ 0 ] is set equal to scaling_list_dc_coef_minus8[ refMatrixId ] + 8

When scaling_list_pred_mode_flag[ matrixId ] is equal to zero, scaling_list_pred_matrix_id_delta[ matrixId ] is equal to zero (indicating default values), and matrixId < 14, then scaling_list_dc_coef_minus8[ matrixId ] is inferred to be equal to 8

scaling_list_delta_coef specifies the difference between the current matrix coefficient ScalingList[ matrixId ][ i ] and the previous matrix coefficient ScalingList[ matrixId ][ i − 1 ], when scaling_list_pred_mode_flag[ matrixId ] is equal to 1. The value of scaling_list_delta_coef shall be in the range of −128 to 127, inclusive. The value of ScalingList[ matrixId ][ i ] shall be greater than 0.

When present (i.e. scaling_list_pred_mode_flag[ matrixId ] is equal to 1), the array ScalingMatrix[ matrixId ] is derived as follows:

ScalingMatrix[ matrixId ][ i ][ j ] = ScalingList[ matrixId ][ k ]	(3-7)
	with k = 0 .. coefNum – 1,
	i = diagScanOrder[ log2(coefNum)/2 ][ log2(coefNum)/2 ][ k ][ 0 ], and
	j = diagScanOrder[ log2(coefNum)/2 ][ log2(coefNum)/2 ][ k ][ 1 ]

Note: the number of coefficients to be signaled coefNum can be expressed mathematically instead of a sequence of comparisons, with identical result: coefNum = Min( 64, 4096 >> ( ( matrixId + 4 ) / 6 ) * 2 )

## Scaling matrix derivation process (new: partly replaces description in scaling list semantics – this is section xxx)

Input to this process is a prediction mode predMode, a colour component variable cIdx, a block width blkWidth and a block height blkHeight.

Output of this process is the (blkWidth)x(blkHeight) array m[ x ][ y ] (scaling matrix), with x and y the horizontal and vertical coefficient position.

The variable matrixId is derived as follows:

matrixId = 6 * sizeId + matrixTypeId		(xxx-1)
	with subWidth = (cIdx > 0) ? SubWidthC : 1,
	subHeight = (cIdx > 0) ? SubHeightC : 1,
	sizeId = 6 – max( log2( blkWidth * subWidth ), log2( blkHeight * subHeight ) ), and
	matrixTypeId = ( 2 * cIdx + ( predMode = =  MODE_INTER ? 1 : 0 ) )

The variable log2MatrixSize is derived as follows:

log2MatrixSize = (matrixId < 20) ? 3 : (matrixId < 26) ? 2 : 1	(xxx-2)

The output array m[ x ][ y ] is derived by applying the following, for x ranging from 0 to blkWidth – 1 included, and y ranging from 0 to blkHeight – 1 included:

m[ x ][ y ] = ScalingMatrix[ matrixId ][ i ][ j ]		(xxx-3)
	with i = ( x << log2MatrixSize ) >> log2( blkWidth ), and
	j = ( y << log2MatrixSize ) >> log2( blkHeight )

If matrixId is lower than 14, m[ 0 ][ 0 ] is further modified as follows:

m[ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ matrixId ] + 8	(xxx-4)

Note: the selected QM size log2MatrixSize can be expressed mathematically instead of a sequence of comparisons, with identical result: log2MatrixSize = min( 3, 6 – ( matrixId + 4 ) / 6 )

## Scaling process for transform coefficients (adapted)

[…]

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The (nTbW)x(nTbH) intermediate scaling factors array m is derived as follows:

  - If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

  - scaling_list_enabled_flag is equal to 0.

  - transform_skip_flag[ xTbY ][ yTbY ] is equal to 1.

  - Otherwise, m is the output of the scaling matrix derivation process as specified in clause xxx, invoked with the prediction mode CuPredMode[ xTbY ][ yTbY ], the colour component variable cIdx, the block width nTbW and the block height nTbH as inputs.

- The scaling factor ls[ x ][ y ] is derived as follows:

[…]

# Performance test

## Test set description

### HEVC test set

Test matrices used in related HEVC quantization core experiments, JCTVC-G1204 and JCTVC-H1104, have been reused, and augmented with other QM sets derived from default or recommended QMs of popular standards (JPEG, MPEG2, h264, HEVC), and a real DTT h264 broadcast.

Matrices from JCTVC that are larger than 8x8 have been downscaled to make HEVC-compliant tests (maximum 8x8 coefficients are coded), and copied from smaller size when very little difference was found (i.e. interpolation was used originally to generate the larger size).

For standards specifying only an 8x8 intra matrix (JPEG), 8x8 inter matrix was derived by applying a scale (0.67014) and offset (5.95569) computed from the relation between 8x8 intra and 8x8 inter h264 default matrices.

For missing 4x4 matrices, these were downsampled from 8x8 matrices.

For missing chroma matrices, these were copied from luma, except for DTT-like test.

For DTT-like intra 8x8 chroma: luma matrix up-right diagonals were first averaged to make constant diagonals, then an affine rule was applied (scale 2, same DC, clipping to 80). For DTT-like inter 8x8 chroma: from averaged up-right diagonals, stretched interpolation (pchip) from 4x4 size was applied along the down-right diagonal.

To extend the test matrices to HEVC needs, the missing size-16 and size-32 matrices have been copied from size-8.

### VVC test set

To extend the test matrices to VVC needs, size-2 matrices have been downsampled from size-4 ones, size-32 has been moved to size-64, and size-32 has be copied from size-16.

## Results

The following table reports the number of bits needed to encode scaling_list_data using three different methods: HEVC, JVET-N0847, and proposed; HEVC uses the HEVC test set (24 matrices per test), and the two other use the VVC test set (30 matrices per test).

| Test name | HEVC | N0847 | N0847 vs HEVC | Proposed | Proposed vs N0847 | Proposed vs HEVC |
| --- | --- | --- | --- | --- | --- | --- |
| JPEG-like | 3007 | 3729 | +722 (+24%) | 1059 | -2670 (-72%) | -1948 (-65%) |
| MPEG2-like | 1067 | 1333 | +266 (+25%) | 489 | -844 (-63%) | -578 (-54%) |
| h264-like | 950 | 1224 | +274 (+29%) | 480 | -744 (-61%) | -470 (-49%) |
| DTT-like | 2264 | 2940 | +676 (+30%) | 1142 | -1798 (-61%) | -1122 (-50%) |
| HEVC | 1281 | 1554 | +273 (+21%) | 535 | -1019 (-66%) | -746 (-58%) |
| symmetry1 | 2090 | 2658 | +568 (+27%) | 892 | -1766 (-66%) | -1198 (-57%) |
| symmetry2 | 1272 | 1620 | +348 (+27%) | 997 | -623 (-38%) | -275 (-22%) |
| symmetry3 | 2660 | 3220 | +560 (+21%) | 2094 | -1126 (-35%) | -566 (-21%) |
| symmetry4 | 2542 | 3146 | +604 (+24%) | 2064 | -1082 (-34%) | -478 (-19%) |
| symmetry7 | 3366 | 4264 | +898 (+27%) | 2675 | -1589 (-37%) | -691 (-21%) |
| asymmetry1 | 2414 | 3078 | +664 (+28%) | 1004 | -2074 (-67%) | -1410 (-58%) |
| asymmetry2 | 1394 | 1778 | +384 (+28%) | 1057 | -721 (-41%) | -337 (-24%) |
| asymmetry3 | 2962 | 3624 | +662 (+22%) | 2272 | -1352 (-37%) | -690 (-23%) |
| asymmetry4 | 3560 | 4344 | +784 (+22%) | 2828 | -1516 (-35%) | -732 (-21%) |
| asymmetry7 | 3946 | 5406 | +1460 (+37%) | 3268 | -2138 (-40%) | -678 (-17%) |
| Average | 2318 | 2928 | +610 (+26%) | 1524 | -1404 (-48%) | -795 (-34%) |

For this test set, it can be seen that the proposed technique saves a significant amount of bits, even compared to HEVC whereas it encodes more matrices.

If QMs are sent every picture, these bit savings translate to about 2.8% BDrate gain in RA for class B; when focusing on QP32 and comparing with average picture cost (still class B, encoded in RA), this translates to 3.4% gain.

If QMs are sent every I-period, these bit savings translate to about 0.06% BDrate gain in RA.

# Conclusion

A simplified quantization matrix signalling is proposed, that allows QM prediction from any previously signalled one. This is made possible by the use of a single QM identifier, transmission of QMs in decreasing size order, and a prediction process which is either a copy or decimation.

Additionally, a unified QM derivation process is described, that encompasses selecting a decoded QM for a given transform block, up-sampling for blocks larger that 8x8, and down-sampling for rectangular blocks.

These modifications reduce by half the draft text to support QMs.

Prediction improvement is tested, and show approximately 50% bits saving for the signalling of QMs from a provided test set, which is derived from a JCTVC test set, default QMs from popular standards, and a real DTT bitstream.

# Annex: chroma format adaptation

As a side effect illustrated in examples 4 (§6.1.1) and 5 (§6.1.2), the proposed adaptation to 4:2:2 and 4:4:4 formats is different from what is currently adopted. Instead of looking for a matrix that matches chroma block size (except for 64x64 where no chroma matrix exists), the size matching is always based on the same (luma) TU size, and coefficients are repeated if needed. This makes the matrix design independent from the chroma format.

### Example 4: chroma block for 8x4 TU (4:2:2 format)

The selected QM is the same as in previous example, but 4:2:2 format needs twice more columns. Here, columns must be repeated, thus x is right-shifted by 1, and y is still left-shifted by 1.

Indeed, blkWidth equals 8, blkHeight equals 2, and log2MatrixSize equals 2, hence equation (2-2) takes the following form:

m[ x ][ y ] = ScalingMatrix[ matrixId ][ i ][ j ]
	with i = ( x << 2 ) >> 3,
	and j = ( y << 2 ) >> 1

where x = 0..7 and y = 0..1

Figure 4: rectangular block for chroma (adaptation to 4:2:2 format)

### Example 5: chroma block for 8x4 TU (4:4:4 format)

The selected QM is still the same as in example 3 and 4, but the 4:4:4 format needs twice as many rows and columns as 4:2:0. Here, reference columns must be repeated, thus x is right-shifted by 1, but the decimation of rows (because of rectangular shape) can be skipped, thus y no more shifted.

Indeed, blkWidth equals 8, blkHeight equals 4, and log2MatrixSize equals 2, hence equation (6-3) takes the following form:

m[ x ][ y ] = ScalingMatrix[ matrixId ][ i ][ j ]
	with i = ( x << 2 ) >> 3, and
	j = ( y << 2 ) >> 2

where x = 0..7 and y = 0..3

Figure 5: rectangular block for chroma (adaptation to 4:4:4 format)

# References

[JVET-N1001]	B.Bross, J.Chen, S.Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, March 2019

[JVET-N0847]	O.Chubach, T.Toma, S.C.Lim, et al., “CE7-related: Support of quantization matrices for VVC”, JVET-N0847, Geneva, CH, March 2019

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).