| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0228 |
| --- | --- |

| Title: | AHG17: Harmonized HRD parameters signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

It is suggested to modify the syntax and semantics of the buffering period SEI message by:

- Adding a loop on temporal sub-layers in the buffering period SEI message;

- Adding requirements that syntax elements repeated in the buffering period SEI message shall be equal to the corresponding syntax elements from the active SPS.

The author believes that the suggested modifications would harmonize the syntax of HRD parameters, buffering period SEI message and picture timing SEI message.

# Introduction

Information related to the HRD (Hypothetical Reference Decoder) are encoded within the HRD parameters syntax structure (which is itself contained within the Sequence Parameter Set), the buffering period SEI message and the picture timing SEI message.

In VVD D5, both the HRD parameters and the picture timing SEI message contain a loop on temporal sub-layers (see Figure 1 and Figure 2 for the syntax of the HRD parameters). However, this is not the case for the buffering period SEI message, which means that for bitstreams using temporal sub-layers (for example for the purpose of frame rate scalability), one buffering period SEI message per sub-layer needs to be signaled, which is inconsistent with what needs to be done for the picture timing SEI message and the HRD parameters. In addition, in order to avoid parsing dependencies between the buffering period SEI message and the HRD parameters, the number of schedules for which bit rate, CPB size and initial CPB removal delays are signalled is signalled independently in both the HRD parameters and the buffering period SEI message. However, there is currently no constraint that the number of schedules signalled in the two structures should be consistent, which may lead to problematic bitstreams.

| hrd_parameters( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| vui_nal_hrd_parameters_present_flag | u(1) |
| vui_vcl_hrd_parameters_present_flag | u(1) |
| if( vui_nal_hrd_parameters_present_flag ¦ ¦ vui_vcl_hrd_parameters_present_flag ){ |  |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| } |  |
| for( i = 0; i <= maxNumSubLayersMinus1; i++ ) { |  |
| fixed_pic_rate_general_flag[ i ] | u(1) |
| if( !fixed_pic_rate_general_flag[ i ] ) |  |
| fixed_pic_rate_within_cvs_flag[ i ] | u(1) |
| if( fixed_pic_rate_within_cvs_flag[ i ] ) |  |
| elemental_duration_in_tc_minus1[ i ] | ue(v) |
| else |  |
| low_delay_hrd_flag[ i ] | u(1) |
| if( !low_delay_hrd_flag[ i ] ) |  |
| vui_cpb_cnt_minus1[ i ] | ue(v) |
| if( vui_nal_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| if( vui_vcl_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| } |  |
| } |  |

Figure 1: Syntax of HRD parameters in VVC D5

| sub_layer_hrd_parameters( subLayerId ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= CpbCnt; i++ ) { |  |
| bit_rate_value_minus1[ i ] | ue(v) |
| cpb_size_value_minus1[ i ] | ue(v) |
| cbr_flag[ i ] | u(1) |
| } |  |
| } |  |

Figure 2: Syntax of sub-layers HRD parameters

# Harmonize the syntax of the buffering period SEI message with the HRD parameters and picture timing SEI message

## Modify the syntax of the buffering period SEI message

In order to harmonize the syntax of HRD parameters, picture timing SEI message and buffering period SEI message, it is suggested to include a loop on temporal sub-layers within the buffering period SEI message as well. To avoid introducing a parsing dependency on the Sequence Parameter Set, the number of temporal sub-layers of the bitstream also has to be signalled within the buffering period SEI message. In addition, the number of schedules per temporal sub-layer signalled in the buffering period SEI message is constrained to be equal to the number of schedules signalled in the HRD parameters of the active Sequence Parameter Set for consistency between the buffering period SEI message and the Sequence Parameter Set.

## Specification text

In order to clean-up the current specification text, it is suggested to remove the prefix „vui_“ from all the syntax elements of the HRD parameters, since the HRD parameters are not part of the VUI syntax structure anymore.

The syntax of the buffering period SEI message is modified as follows with the additions marked in yellow:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_seq_parameter_set_id | ue(v) |
| bp_nal_hrd_parameters_present_flag | u(1) |
| bp_vcl_hrd_parameters_present_flag | u(1) |
| if( bp_nal_hrd_parameters_present_flag ¦ ¦ bp_ vcl_hrd_parameters_present_flag ) { |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| } |  |
| concatenation_flag | u(1) |
| cpb_removal_delay_delta_minus1 | u(v) |
| cpb_removal_delay_deltas_present_flag | u(1) |
| if( cpb_removal_delay_deltas_present_flag ) { |  |
| num_cpb_removal_delay_deltas_minus1 | ue(v) |
| for( i = 0; i <= num_cpb_removal_delay_deltas_minus1; i++ ) |  |
| cpb_removal_delay_delta[ i ] | u(v) |
| } |  |
| bp_max_sub_layers_minus1 | u(3) |
| for( tid = 0; tid <= bp_max_sub_layers_minus1; tid++ ) { |  |
| bp_cpb_cnt_minus1[ tid ] | ue(v) |
| if( bp_nal_hrd_parameters_present_flag ) |  |
| for( i = 0; i < bp_cpb_cnt_minus1[ tid ] + 1; i++ ) { |  |
| nal_initial_cpb_removal_delay[ tid ][ i ] | u(v) |
| nal_initial_cpb_removal_offset[ tid ] [ i ] | u(v) |
| } |  |
| if( bp_vcl_hrd_parameters_present_flag ) |  |
| for( i = 0; i < bp_cpb_cnt_minus1[ tid ] + 1; i++ ) { |  |
| vcl_initial_cpb_removal_delay[ tid ] [ i ] | u(v) |
| vcl_initial_cpb_removal_offset[ tid ] [ i ] | u(v) |
| } |  |
| } |  |
| } |  |

The semantics of the buffering period SEI message is modified as follows:

bp_nal_hrd_parameters_present_flag equal to 1 specifies that a list of syntax element pairs nal_initial_cpb_removal_delay[ tid ][ i ] and nal_initial_cpb_removal_offset[ tid ][ i ] are present in the buffering period SEI message. bp_nal_hrd_parameters_present_flag equal to 0 specifies that no syntax element pairs nal_initial_cpb_removal_delay[ tid ][ i ] and nal_initial_cpb_removal_offset[ tid ][ i ] are present in the buffering period SEI message.

It is a requirement of bitstream conformance that the value of bp_nal_hrd_parameters_present_flag in the buffering period SEI message assocaited with an access unit is equal to the value of vui_nal_hrd_parameters_present_flag in the VUIHRD parameters of the active SPS.

bp_vcl_hrd_parameters_present_flag equal to 1 specifies that a list of syntax element pairs vcl_initial_cpb_removal_delay[ tid ][ i ] and vcl_initial_cpb_removal_offset[ tid ][ i ] are present in the buffering period SEI message. bp_vcl_hrd_parameters_present_flag equal to 0 specifies that no syntax element pairs vcl_initial_cpb_removal_delay[ tid ][ i ] and vcl_initial_cpb_removal_offset[ tid ][ i ] are present in the buffering period SEI message.

It is a requirement of bitstream conformance that the value of bp_vcl_hrd_parameters_present_flag in the buffering period SEI message associated with an access unit is equal to the value of vui_vcl_hrd_parameters_present_flag in the VUIHRD parameters of the active SPS.

initial_cpb_removal_delay_length_minus1 plus 1 specifies the length, in bits, of the syntax elements nal_initial_cpb_removal_delay[ tid ][ i ], nal_initial_cpb_removal_offset[ tid ][ i ], vcl_initial_cpb_removal_delay[ tid ][ i ], and vcl_initial_cpb_removal_offset[ tid ][ i ] of the buffering period SEI message. When not present, the value of initial_cpb_removal_delay_length_minus1 is inferred to be equal to 23.

...

bp_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the access units associated with the buffering period SEI message. The value of bp_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

It is a requirement of bitstream conformance that the value of bp_max_sub_layers_minus1 in the buffering period SEI message is equal to the value of sps_max_sub_layers_minus1 in the active SPS.

bp_cpb_cnt_minus1[ tid ] plus 1 specifies the number of syntax element pairs nal_initial_cpb_removal_delay[ tid ][ i ] and nal_initial_cpb_removal_offset[ tid ][ i ] when bp_nal_hrd_parameters_present_flag is equal to 1, and the number of syntax element pairs vcl_initial_cpb_removal_delay[ tid ][ i ] and vcl_initial_cpb_removal_offset[ tid ][ i ] when bp_vcl_hrd_parameters_present_flag is equal to 1. The value of bp_cpb_cnt_minus1 shall be in the range of 0 to 31, inclusive.

It is a requirement of bitstream conformance that the value of bp_cpb_cnt_minus1[ tid ] in the buffering period SEI message is equal to the value of vui_cpb_cnt_minus1[ tid ] in the active SPS for temporal sub-layer tid.

nal_initial_cpb_removal_delay[ tid ][ i ] specify the i-th initial CPB removal delay for the NAL HRD in units of a 90 kHz clock for temporal sub-layer tid. The length of nal_initial_cpb_removal_delay[ tid ][ i ] is initial_cpb_removal_delay_length_minus1 + 1 bits. The value of nal_initial_cpb_removal_delay[ tid ][ i ] shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ i ]  BitRate[ i ] ), the time-equivalent of the CPB size in 90 kHz clock units. [Ed. (YK): Add a clarification of CpbSize[ i ]  BitRate[ i ] at the beginning of this clause.]

nal_initial_cpb_removal_offset[ tid ][ i ] specify the i-th initial CPB removal offset for the NAL HRD in units of a 90 kHz clock for temporal sub-layer tid. The length of nal_initial_cpb_removal_offset[ tid ][ i ] is initial_cpb_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, the sum of nal_initial_cpb_removal_delay[ tid ][ i ] and nal_initial_cpb_removal_offset[ tid ][ i ] shall be constant for each value of i and tid.

vcl_initial_cpb_removal_delay[ tid ][ i ] specify the i-th initial CPB removal delay for the VCL HRD in units of a 90 kHz clock for temporal sub-layer tid. The length of vcl_initial_cpb_removal_delay[ tid ][ i ] is initial_cpb_removal_delay_length_minus1 + 1 bits. The value of vcl_initial_cpb_removal_delay[ tid ][ i ] shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ i ]  BitRate[ i ] ), the time-equivalent of the CPB size in 90 kHz clock units.

vcl_initial_cpb_removal_offset[ tid ][ i ] specify the i-th initial CPB removal offset for the VCL HRD in units of a 90 kHz clock for temporal sub-layer tid. The length of vcl_initial_cpb_removal_offset[ tid ][ i ] is initial_cpb_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, the sum of vcl_initial_cpb_removal_delay[ tid ][ i ] and vcl_initial_cpb_removal_offset[ tid ][ i ] shall be constant for each value of I and tid.

In C.2.2	 Timing of decoding unit arrival and C.2.3 Timing of access unit removal and decoding of access unit:

The variables InitCpbRemovalDelay[ OpTid ][ SchedSelIdx ] and InitCpbRemovalDelayOffset[ OpTid ][ SchedSelIdx ] are derived as follows:

–	InitCpbRemovalDelay[ OpTid ][ SchedSelIdx ] and InitCpbRemovalDelayOffset[ OpTid ][ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ OpTid ][ SchedSelIdx ] and nal_initial_cpb_removal_offset[ OpTid ][ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ OpTid ][ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ OpTid ][ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause C.1.

# Conclusion

It is suggested to harmonize the syntax of the HRD parameters, the buffering period SEI message and the picture timing SEI message by:

- Adding a loop on temporal sub-layers in the buffering period SEI message;

- Adding requirements that syntax elements repeated in the buffering period SEI message shall be equal to the corresponding syntax elements from the active SPS.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).