| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0230-r1 |
| --- | --- |

| Title: | Non-CE3: Interpolation filtering for intra prediction in non-diagonal directions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath, Fabien Racapé, Fabrice Urban, Fabrice Le Léannec | Tel: Email: | +33.2.56.85.62.76 gagan.rath@InterDigital.com fabien.racape@InterDigital.com fabrice.urban@InterDigital.com fabrice.leleannec@InterDigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

For intra prediction along non-diagonal directions, this contribution reports the results of using simple linear approximations for interpolation with four reference samples instead of the 4-tap Gaussian and cubic filters. Since models using four samples cannot model object boundaries accurately, it is also proposed to use the conventional interpolation with two reference samples if their magnitudes are relatively close. BD-rate gains of 0.09% for Luma and 0.25% for chroma are reported. Besides BD-rate performance gain, the advantages of the proposed method are lower complexity, no memory storage of filter coefficients, and a uniform method for both Luma and Chroma components.

# Introduction

# In VTM 5.0, for intra prediction along non-diagonal directions except the horizontal and vertical directions, the predicting reference samples are interpolated using either a 4-tap Gaussian filter or a 4-tap cubic filter. Though such an interpolation leads to higher coding gain than the conventional linear interpolation, it comes at the expense of additional complexity, and memory requirements for storing the 32x4 cubic and Gaussian filter coefficient arrays. This contribution proposes to use simpler linear models with four reference samples instead of the 4-tap Gaussian and the cubic filters. As the VTM5.0 already uses the conventional linear interpolation for chroma components, the proposed method is applied to both Luma and chroma components.

# Problem definition

For the sake of generality, a rectangular target block with width W samples and height H samples is considered. Let refMain and refSide represent the reference arrays on the top and the left of the target block, respectively. For horizontal prediction modes, that is, if mode < DIA_IDX,  refMain and the refSide denote the top and the left reference arrays after they have been swapped. Since this contribution deals with only non-diagonal directions, both refMain and refSide consist of unfiltered decoded samples in VTM 5.0.

For any target pixel at (x,y), the predictor reference sample on refMain is identified through two parameters deltaInt and deltraFract, which are determined as deltaPos = (1+y) * predIntraAngle, deltaInt = deltaPos >> 5, and deltraFract = deltaPos & 31, where predIntraAngle corresponds to the prediction mode.

Let  P0, P1, P2, and P3 denote the four reference samples nearest to the predictor reference sample, as shown in Figure 1.  They are identified as:P0 ≡ refMain[x+deltaInt],  P1 ≡ refMain[x+deltaInt+1],  P2 ≡ refMain[x+deltaInt + 2],  and P3 ≡refMainx+deltaInt +3.

Figure 1: interpolation of predictor for intra prediction in non-diagonal directions. P0, P1, P2, and P3 are the nearest reference samples which are used to interpolate the predictor with either a Gaussian or a cubic filter for the Luma component. For the Chroma component, the predictor is linearly interpolated using P1 and P2.

For a Luma target block, the predictor sample value is computed as:

Ppred(x,y) = (F[0] * P0 + F[1] * P1 + F[2] * P2 + F[3] * P3 + 32) >> 6 …… (1)

where F[i] denotes the ith filter coefficient. The chosen filter depends on the value deltaFract and a Boolean parameter called filterFlaguseFilter. If filterFlaguseFilter is enabled, then a Gaussian filter is used, else a cubic filter is used. The value of the filterFlaguseFilter is decided based on several intra prediction parameters such as the block size, the prediction direction, MRL index, etc. As the specified cubic filter has both negative and positive coefficient values, when filterFlaguseFilter is disabled, a clipping operation is additionally performed to keep the predicted value within the valid dynamic range of the Luma component:

Ppred(x,y) = Clip((F[0] * P0 + F[1] * P1 + F[2] * P2 + F[3] * P3 + 32) >> 6) …… (2)

For a chroma target block, only the two reference samples P1 and P2 are used for interpolating the predictor. The predictor value is linearly interpolated, as in HEVC:

Ppred(x,y) = ((32-deltaFract) * P1 +deltaFract * P2+  16) >> 5; …… (3)

Since deltaFract has integral values in the range [0-31], VTM 5.0 includes 32 Gaussian and 32 cubic filters corresponding to  32 deltaFract values, which are pre-computed and stored in memory.  It is clear that each Luma target block requires four multiplications per sample.

# Proposed solutions

refMainrefSideuseFilterP1P2 useFilter

useFilter

P1 = (refMain[x + deltaInt] + 2*refMain[x + deltaInt + 1] + refMain[x + deltaInt + 2] + 2) >> 2 ;

P2 = (refMain[x + deltaInt+1] + 2* refMain[x + deltaInt + 2]+ refMain[x + deltaInt + 3] + 2) >> 2 ;

refMainrefSideuseFilter

In the first proposed method, the predictor value is always computed as:

Ppred(x,y) = Clip(P1  + P2 – P1<< 2* deltaFract + P1 + P2 – P0 – P3 * deltaFract2  + 64>> 7)

where deltaFract2=deltaFract  if deltaFract<16, deltaFract2= 32-deltaFract , otherwise.

In the second proposed method, the absolute difference between P1 and P2 is compared with a pre-set threshold. If the value is less than the threshold, the conventional linear interpolation is performed using P1 and P2, else the predictor is interpolated as above. Denoting the pre-set threshold by Th,

If |(P1-P2)| <= Th

Ppred(x,y) = (P1 * (32 – deltaFract)   + P2 * deltaFract + 16) >> 5;

Else

Ppred(x,y) = Clip(P1  + P2 – P1<< 2* deltaFract + P1 + P2 – P0 – P3 * deltaFract2  + 64>> 7)

The interpolation is derived from the piece-wise linear approximation of a 2nd order polynomial that uses four samples p0, p1, p2, and p3, as shown in Figure 2.

Figure 2: Piece-wise linear approximation of a quadratic function. In each half of the interval [0, 1], the function is approximated with a straight line, where the slope of the line depends on the four given sample values p0, p1, p2, and p3.

In VTM 5.0,  the selection between the filtered and unfiltered reference samples is made depending on the value of a flag called refFilterFlag,  For non-diagonal directions, the refFilterFlag is always disabled, that is, for non-diagonal directions, the unfiltered reference samples are always used for prediction. Another flag called interpolationFlag  is used to select between the Gaussian and cubic filters.

In the above proposed methods, there is no need to distinguish between the Gaussian and cubic filters. Thus, for prediction along non-diagonal directions, it is  proposed to enable/disable the refFilterFlag depending on if the reference samples need to be filtered or not.  As an alternative implementation in VTM-5.0,  as reference sample arrays refMain and refSide are always unfiltered for prediction along non-directional directions, P1 and P2 can be filtered  if interpolationFlag flag is true:

:

P1 = (refMain[x + deltaInt] + 2*refMain[x + deltaInt + 1] + refMain[x + deltaInt + 2] + 2) >> 2 ;

P2 = (refMain[x + deltaInt+1] + 2* refMain[x + deltaInt + 2]+ refMain[x + deltaInt + 3] + 2) >> 2 ;

# Experimental Results

Simulations were performed according to CTC described in [1]. AI and RA results are reported below.  Test 1 and Test 2 results correspond to the first and the second methods, respectively. In Test 2, the threshold values used for the Luma and Chroma components were 16 and 8 respectively. It is interesting to observe that Method 2 results in BD-rate gain for all test classes. Looking at the results for individual test sequences, it is also observed that, Method 2 yields Luma BD-rate gain for almost all test sequences under AI configuration.

## Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.00% | 0.02% | 94% | 94% |
| Class A2 | 0.04% | -0.21% | -0.18% | 95% | 96% |
| Class B | 0.03% | -0.31% | -0.24% | 98% | 93% |
| Class C | -0.21% | -0.51% | -0.50% | 94% | 90% |
| Class E | 0.02% | -0.07% | -0.12% | 98% | 95% |
| Overall | -0.01% | -0.25% | -0.23% | 96% | 93% |
| Class D | -0.13% | -0.05% | -0.41% | 98% | 94% |
| Class F | -0.06% | -0.37% | -0.29% | 100% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.34% | 0.00% | 102% | 102% |
| Class A2 | 0.03% | -0.17% | -0.30% | 100% | 99% |
| Class B | 0.03% | -0.62% | -0.69% | 96% | 99% |
| Class C | -0.08% | -0.58% | -0.76% | 95% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.46% | -0.49% | 98% | 98% |
| Class D | -0.06% | -0.24% | -0.19% | 95% | 92% |
| Class F |  |  |  |  |  |

## Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.09% | 97% | 96% |
| Class A2 | -0.10% | -0.24% | -0.26% | 97% | 98% |
| Class B | -0.06% | -0.29% | -0.18% | 100% | 93% |
| Class C | -0.17% | -0.54% | -0.49% | 102% | 94% |
| Class E | -0.10% | -0.07% | -0.14% | 99% | 98% |
| Overall | -0.09% | -0.25% | -0.24% | 99% | 95% |
| Class D | -0.14% | 0.00% | -0.40% | 99% | 93% |
| Class F | -0.20% | -0.38% | -0.42% | 99% | 97% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.06% | -0.04% | 102% | 97% |
| Class A2 | -0.05% | -0.12% | -0.18% | 101% | 97% |
| Class B | 0.00% | -0.31% | -0.45% | 95% | 96% |
| Class C | -0.07% | -0.58% | -0.43% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.29% | -0.31% | 99% | 97% |
| Class D | -0.01% | -0.23% | -0.28% | 98% | 93% |
| Class F |  |  |  |  |  |

# Specification text changes

##### Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable refIdx specifying the intra prediction reference line index,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable refFilterFlag specifying the value of reference filter flag,

- a variable cIdx specifying the colour component of the current block,

- the neighbouring samples p[ x ][ y ], with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1.

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0.

- refFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is greater than or equal to INTRA_ANGULAR34 and nTbW is less than or equal to 8

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is less than INTRA_ANGULAR34 and nTbH is less than or equal to 8.

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-9.

- The variable filterFlag is derived as follows:

- If one more of the following conditions is true, filterFlag is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- minDistVerHor is greater than intraHorVerDistThres[ nTbS ] and refFilterFlag is equal to 0

- Otherwise, filterFlag is set equal to 0.

Table 8-9 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 16 | 14 | 2 | 0 | 0 | 0 |

Figure 8-1 – Intra prediction directions (informative)

Figure 8-1 illustrates the 93 prediction directions, where the dashed directions are associated with the wide-angle modes that are only applied to non-square blocks.

Table 8-10 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Table 8-10 – Specification of intraPredAngle

| predModeIntra | −14 | −13 | −12 | −11 | −10 | −9 | −8 | −7 | −6 | −5 | −4 | −3 | −2 | −1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | 512 | 341 | 256 | 171 | 128 | 102 | 86 | 73 | 64 | 57 | 51 | 45 | 39 | 35 | 32 | 29 | 26 |
| predModeIntra | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| intraPredAngle | 23 | 20 | 18 | 16 | 14 | 12 | 10 | 8 | 6 | 4 | 3 | 2 | 1 | 0 | −1 | −2 | −3 |
| predModeIntra | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| intraPredAngle | −4 | −6 | −8 | −10 | −12 | −14 | −16 | −18 | −20 | −23 | −26 | −29 | −32 | −29 | −26 | −23 | −20 |
| predModeIntra | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
| intraPredAngle | −18 | −16 | −14 | −12 | −10 | −8 | −6 | −4 | −3 | −2 | −1 | 0 | 1 | 2 | 3 | 4 | 6 |
| predModeIntra | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 |
| intraPredAngle | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 51 | 57 | 64 |
| predModeIntra | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |  |  |  |  |  |  |  |  |  |
| intraPredAngle | 73 | 86 | 102 | 128 | 171 | 256 | 341 | 512 |  |  |  |  |  |  |  |  |  |

The inverse angle parameter invAngle is derived based on intraPredAngle as follows:

invAngle = Round256*32intraPredAngle		(8-118)

-

Table 8-11 – Specification of interpolation filter coefficients fC and fG

| Fractional sample position p | fC interpolation filter coefficients |  |  |  | fG interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] | fG[ p ][ 0 ] | fG[ p ][ 1 ] | fG[ p ][ 2 ] | fG[ p ][ 3 ] |
| 0 | 0 | 64 | 0 | 0 | 16 | 32 | 16 | 0 |
| 1 | −1 | 63 | 2 | 0 | 15 | 29 | 17 | 3 |
| 2 | −2 | 62 | 4 | 0 | 15 | 29 | 17 | 3 |
| 3 | −2 | 60 | 7 | −1 | 14 | 29 | 18 | 3 |
| 4 | −2 | 58 | 10 | −2 | 13 | 29 | 18 | 4 |
| 5 | −3 | 57 | 12 | −2 | 13 | 28 | 19 | 4 |
| 6 | −4 | 56 | 14 | −2 | 13 | 28 | 19 | 4 |
| 7 | −4 | 55 | 15 | −2 | 12 | 28 | 20 | 4 |
| 8 | −4 | 54 | 16 | −2 | 11 | 28 | 20 | 5 |
| 9 | −5 | 53 | 18 | −2 | 11 | 27 | 21 | 5 |
| 10 | −6 | 52 | 20 | −2 | 10 | 27 | 22 | 5 |
| 11 | −6 | 49 | 24 | −3 | 9 | 27 | 22 | 6 |
| 12 | −6 | 46 | 28 | −4 | 9 | 26 | 23 | 6 |
| 13 | −5 | 44 | 29 | −4 | 9 | 26 | 23 | 6 |
| 14 | −4 | 42 | 30 | −4 | 8 | 25 | 24 | 7 |
| 15 | −4 | 39 | 33 | −4 | 8 | 25 | 24 | 7 |
| 16 | −4 | 36 | 36 | −4 | 8 | 24 | 24 | 8 |
| 17 | −4 | 33 | 39 | −4 | 7 | 24 | 25 | 8 |
| 18 | −4 | 30 | 42 | −4 | 7 | 24 | 25 | 8 |
| 19 | −4 | 29 | 44 | −5 | 6 | 23 | 26 | 9 |
| 20 | −4 | 28 | 46 | −6 | 6 | 23 | 26 | 9 |
| 21 | −3 | 24 | 49 | −6 | 6 | 22 | 27 | 9 |
| 22 | −2 | 20 | 52 | −6 | 5 | 22 | 27 | 10 |
| 23 | −2 | 18 | 53 | −5 | 5 | 21 | 27 | 11 |
| 24 | −2 | 16 | 54 | −4 | 5 | 20 | 28 | 11 |
| 25 | −2 | 15 | 55 | −4 | 4 | 20 | 28 | 12 |
| 26 | −2 | 14 | 56 | −4 | 4 | 19 | 28 | 13 |
| 27 | −2 | 12 | 57 | −3 | 4 | 19 | 28 | 13 |
| 28 | −2 | 10 | 58 | −2 | 4 | 18 | 29 | 13 |
| 29 | −1 | 7 | 60 | −2 | 3 | 18 | 29 | 14 |
| 30 | 0 | 4 | 62 | −2 | 3 | 17 | 29 | 15 |
| 31 | 0 | 2 | 63 | −1 | 3 | 17 | 29 | 15 |

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If predModeIntra is greater than or equal to 34, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = 0..nTbW + refIdx	(8-119)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbH * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x = −1..( nTbH * intraPredAngle )  >>  5	(8-120)

ref[ ( ( nTbH * intraPredAngle )  >>  5 ) − 1 ] = ref[ ( nTbH * intraPredAngle )  >>  5 ]	(8-121)

ref[ nTbW + 1 + refIdx ] = ref[ nTbW + refIdx ]	(8-122)

- Otherwise,

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = nTbW + 1 + refIdx..refW + refIdx	(8-123)

ref[ −1 ] = ref[ 0 ]		(8-124)

- The additional samples ref[ refW + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 1) are derived as follows:

ref[ refW + refIdx +x ] = p[ −1 + refW ][ −1 − refIdx ]	(8-125)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 + refIdx ) * intraPredAngle )  >>  5 + refIdx	 (8-126)

iFact = ( ( y + 1 + refIdx ) * intraPredAngle ) & 31	(8-127)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(8-128)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(8-129)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-130)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-131)

Four variables P[k], k=0,1,2,3 are identified as P[k] = ref[x + iIdx + k]. If filterFlag = true, then P[1] and P[2] are     filtered as P[k] = (ref[x + iIdx + k – 1] + ref[x + iIdx + k] * 2 + ref[x + iIdx + k + 1] + 2) >> 2, k = 1, 2.

If abs(P[1] – P[2]) < Th, Th = 16 if cIdx = 0, Th = 8, otherwise, then value of the prediction samples predSamples[ x ][ y ] is derived as:

predSamples[ x ][ y ] = ((32 - iFact) * P[1] + iFact * P[2] + 16) >> 5;

Else, the value of the prediction samples predSamples[ x ][ y ] is derived as:

predSamples[ x ][ y ] =  Clip1Y(P[1] + ((((P[2] – P[1]) << 2) * iFact + (P[1] + P[2] – P[0] – P[3])* iFact2 + 64) >> 7)

where iFact2 = iFact if iFact < 16, else iFact2 = 32 - iFact.

Otherwise (predModeIntra is less than 34), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = 0..nTbH + refIdx	(8-132)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbW * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 − refIdx + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 − refIdx ],
	with x = −1..( nTbW * intraPredAngle )  >>  5	(8-133)

ref[ ( ( nTbW * intraPredAngle )  >>  5 ) − 1 ] = ref[ ( nTbW * intraPredAngle )  >>  5 ]	(8-134)

ref[ nTbG + 1 + refIdx ] = ref[ nTbH + refIdx ]	(8-135)

- Otherwise,

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = nTbH + 1 + refIdx..refH + refIdx	(8-136)

ref[ −1 ] = ref[ 0 ]		(8-137)

- The additional samples ref[ refH + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 1) are derived as follows:

ref[ refH + refIdx +x ] = p[ −1 + refH ][ −1 − refIdx ]	(8-138)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 + refIdx ) * intraPredAngle )  >>  5	(8-139)

iFact = ( ( x + 1 + refIdx ) * intraPredAngle ) & 31	(8-140)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(8-141)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ y+iIdx+i ] ) + 32 )  >>  6 )	(8-142)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-143)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-144)

Four variables P[k], k=0,1,2,3 are identified as P[k] = ref[y + iIdx + k]. If filterFlag = true, then P[1] and P[2] are     filtered as P[k] = (ref[y + iIdx + k – 1] + ref[y + iIdx + k] * 2 + ref[y + iIdx + k + 1] + 2) >> 2, k = 1, 2.

If abs(P[1] – P[2]) < Th, Th = 16 if cIdx = 0, Th = 8, otherwise, then value of the prediction samples predSamples[ x ][ y ] is derived as:

predSamples[ x ][ y ] = ((32 - iFact) * P[1] + iFact * P[2] + 16) >> 5;

Else, the value of the prediction samples predSamples[ x ][ y ] is derived as:

predSamples[ x ][ y ] =  Clip1Y(P[1] + ((((P[2] – P[1]) << 2) * iFact + (P[1] + P[2] – P[0] – P[3])* iFact2 + 64) >> 7)

where iFact2 = iFact if iFact < 16, else iFact2 = 32 - iFact.

# Conclusion

This contribution reports the results of using simple linear approximations for interpolation in intra prediction along non-diagonal directions. The approximations are proposed to replace the Gaussian and the cubic filtering for Luma and the conventional linear interpolation for chroma, in VTM 5.0. Experimental results show BD-rate gains of 0.01% for Luma and 0.25% for Chroma over VTM 5.0 in AI configuration when no threshold is used. With threshold, BD-rate gains of 0.09% for Luma and 0.25% for Chroma are reported. Both cases report reduced encoding and decoding times. Considering this, and other advantages, Method 2 is proposed for intra prediction in VTM as a unified method for Luma and Chroma.

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, 13th Meeting, Marrakech, MA, Jan. 2019.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).