| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0250 |
| --- | --- |

| Title: | Non-CE9: DMVR and BDOF on/off control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Vadim Seregin, Muhammed Coban and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, vseregin, mcoban, martak}@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current VTM, DMVR and BDOF are always enabled for a regular merge candidate that satisfies the bi-prediction and CU size constraints. In this contribution, it is proposed to enable zero MVD in the MMVD mode as a CU on/off control, so that an encoder can have an option to choose a regular merge candidate without performing DMVR and BDOF. The Y, U, V BD-rate changes for the proposed method are 0.01%, -0.06%, 0.01% and 0.00%, 0.06%, -0.02% in the random access and low delay B test conditions, respectively. It is also proposed to add a slice header flag for the on/off control of DMVR and BDOF.

# Introduction

DMVR improves bi-directional motion prediction by using bilateral matching. BDOF improves bi-directional prediction by adding an offset for each pixel based on the gradient information. However, in some certain texture area, DMVR and BDOF may not work.  For example, the tool off tests in AHG 13 show that DMVR and BDOF do not have compression efficiency improvement for screen content sequences. In a mixed content video sequence, a picture may be partitioned into slices having camera captured content and computer graphic content. Therefore, a slice level control of DMVR and BDOF is desirable.

In the current VTM, DMVR/BDOF is always applied if DMVR/BDOF is enabled at SPS and some bi-prediction and size constrains are met for a regular merge candidate.

DMVR is applied to a regular merge mode when all the following conditions are true:

- sps_dmvr_enabled_flag is equal to 1

- general_merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_merge_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- BcwIdx[ xCb ][ yCb ] is equal to 0

- Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 128

BDOF is applied to bi-prediction when all the following conditions are true:

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

- BcwIdx[ xCb ][ yCb ] is equal to 0.

- luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

- cbHeight is greater than or equal to 8

- cIdx is equal to 0.

Therefore, an encoder does not have an option to choose a regular merge candidate with bi-prediction but without performing DMVR and BDOF. A CU level on/off control of DMVR and BDOF can be beneficial.

# CU level on/off control of DMVR and BDOF

The MMVD mode adds a signaled MVD to a regular merge candidate, while DMVR is disabled for MMVD mode. The CU level on/off control of DMVR and BDOF for a regular merge candidate can be applied by allowing zero MVD in the MMVD mode. It is proposed to add a zero MMVD distance for the MMVD mode if the following conditions are true:

- sps_dmvr_enabled_flag || sps_bdof_enabled_flag

- mvd_l1_zero_flag is equal to 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 128

If the zero MMVD distance step is chosen, mmvd_direction_idx is not signaled. Parsing of MMVD syntax and semantics is modified as follows (highlighted in green):

The variable mmvdZeroMvdAvail is set equal to 1 if all the following are true:

- sps_dmvr_enabled_flag or sps_bdof_enabled_flag is equal to 1

- mvd_l1_zero_flag is equal to 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 128

Otherwise, mmvdZeroMvdAvail is set equal to 0.

| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| --- | --- |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| if(mmvd_distance_idx[ x0 ][ y0 ] >0 ¦¦ mmvdZeroMvdAvail == 0) { |  |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |

mmvd_distance_idx[ x0 ][ y0 ] specifies the index used to derive MmvdDistance[ x0 ][ y0 ]. If the mmvdZeroMvdAvail is equal to 1,  mmvd_distance_idx[ x0 ][ y0 ] is minused by 1.

MmvdDistance[ x0 ][ y0 ] is specified as in Table 7-12. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-12 – Specification of MmvdDistance[ x0 ][ y0 ] based on mmvd_distance_idx[ x0 ][ y0 ].

| mmvd_distance_idx[ x0 ][ y0 ] | MmvdDistance[ x0 ][ y0 ] |  |
| --- | --- | --- |
|  | slice_fpel_mmvd_enabled_flag = = 0 | slice_fpel_mmvd_enabled_flag = = 1 |
| -1 | 0 | 0 |
| 0 | 1 | 4 |
| 1 | 2 | 8 |
| 2 | 4 | 16 |
| 3 | 8 | 32 |
| 4 | 16 | 64 |
| 5 | 32 | 128 |
| 6 | 64 | 256 |
| 7 | 128 | 512 |

An additional context is added to the second bin of mmvd_distance_idx syntax element.

The derivation of bdofFlag is modified as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbHeight is greater than or equal to 8

      - cIdx is equal to 0

      - cbWidth is less than 8 or cbHeight*cbWidth is less than 128 or MmvdDistance[ x0 ][ y0 ] is not equal to 0.

    - Otherwise, bdofFlag is set equal to FALSE.

# Test results

The test is performed on top of VTM-5.0 under random access and low delay B common test conditions, and the result is shown in Table 3-1. The Y, U, V BD-rate changes for the proposed method are 0.01%, -0.06%, 0.01% in random access, and 0.00%, 0.06%, -0.02% in low delay B. The changes in BD-rate low delay B configuration are due to the added context for mmvd_distance_idx. Runtime is inaccurate due to the use of heterogeneous cluster.

Table 3-1 Test result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.17% | 0.04% | 105% | 105% |
| Class A2 | 0.02% | 0.08% | 0.02% | 101% | 100% |
| Class B | 0.01% | -0.02% | 0.03% | 102% | 103% |
| Class C | 0.00% | -0.14% | -0.05% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.06% | 0.01% | 101% | 101% |
| Class D | 0.07% | -0.02% | 0.27% | 102% | 101% |
| Class F | 0.00% | 0.06% | -0.03% | 103% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.13% | 0.12% | 99% | 101% |
| Class C | -0.01% | -0.03% | -0.03% | 101% | 104% |
| Class E | 0.04% | 0.07% | -0.24% | 100% | 109% |
| Overall | 0.00% | 0.06% | -0.02% | 100% | 104% |
| Class D | -0.02% | 0.18% | 0.84% | 100% | 99% |
| Class F | -0.12% | -0.11% | -0.22% | 103% | 105% |

# Conclusion

In the current VTM, there is no DMVR and BDOF on/off control except for the SPS level flags. In this contribution, a finer level control is proposed by adding a flag in a slice header and CU level on/off control for DMVR and BDOF by adding a zero MVD case to the MMVD mode.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).