| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0253 |
| --- | --- |

| Title: | Non-CE4: Simplification of HMVP in AMVP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the HMVP insertion process in AMVP mode. One loop in the HMVP insertion process is removed, and POC check is replaced by reference index check. The Y, U, V BD-rate changes under common test conditions are 0.00%, 0.01%, -0.01% in RA and 0.01%, 0.06%, -0.21% in LDB.  Similar performance is observed in the low QP test, the Y, U, V BD-rate change are 0.00%, -0.03%, 0.03% in RA and 0.00%, 0.04%, -0.03% in LDB.

# Introduction

In current VTM, the history-based motion vector predictor (HMVP) insertion process is as follows:

When numCurrMvpCand is less than 2 and NumHmvpCand is greater than 0, the following applies for i= 1..Min( 4, NumHmvpCand ) until numCurrMvpCand is equal to 2:

- For each reference picture list LY with Y = X..( 1 − X ), the following applies until numCurrMvpCand is equal to 2:

- When the reference picture corresponding to the reference index of the history-based motion vector predictor candidate HmvpCandList[ i − 1 ] in the reference picture list LY is the same as the reference picture corresponding to reference index refIdxLX in the reference picture list LX, the following applies:

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to the LY motion vector of the candidate HmvpCandList[ i − 1 ], rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded LY motion vector of the candidate HmvpCandList[ i − 1 ] as output is assigned to mvpListLX[ numCurrMvpCand++ ].

There’s an inner loop over two different reference lists to find whether the current HMVP has the same reference picture as the current reference picture. It’s proposed to remove this inner loop and replace POC check with reference index check. The insertion process is modified as follows:

When numCurrMvpCand is less than 2 and NumHmvpCand is greater than 0, the following applies for i= 1..Min( 4, NumHmvpCand ) until numCurrMvpCand is equal to 2:

- For each reference picture list LY with Y = X..( 1 − X ), the following applies until numCurrMvpCand is equal to 2:

- When the reference picture corresponding to the reference index of the history-based motion vector predictor candidate HmvpCandList[ i − 1 ] in the reference picture list LYX is the same as the reference picture corresponding to reference index refIdxLX in the reference picture list LX, the following applies:

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to the LYX motion vector of the candidate HmvpCandList[ i − 1 ], rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded LYX motion vector of the candidate HmvpCandList[ i − 1 ] as output is assigned to mvpListLX[ numCurrMvpCand++ ].

# Test results

The test results under random access and low delay B common test conditions are shown in Table 1. The overall Y, U, V, BD-rate changes are 0.00%, 0.01%, -0.01% in RA and 0.01%, 0.06%, -0.21% in LDB. In the low QP tests, QP set {12, 17, 22, 27}, the BD-rate changes are 0.00%, -0.03%, 0.03% in RA and 0.00%, 0.04%, -0.03% in LDB. Details of the test results are shown in Table 3-1 and Table 3-2, respectively. Runtime is inaccurate due to the use of heterogeneous cluster.

Table 3-1 Test result under common test conditoins

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.05% | 104% | 103% |
| Class A2 | 0.01% | 0.05% | 0.09% | 99% | 99% |
| Class B | 0.02% | 0.05% | -0.04% | 102% | 103% |
| Class C | -0.04% | 0.00% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 102% | 101% |
| Class D | 0.02% | 0.16% | 0.28% | 109% | 117% |
| Class F | 0.00% | 0.04% | 0.03% | 102% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.18% | -0.29% | 100% | 103% |
| Class C | 0.01% | 0.07% | 0.09% | 100% | 93% |
| Class E | 0.02% | -0.15% | -0.47% | 100% | 101% |
| Overall | 0.01% | 0.06% | -0.21% | 100% | 99% |
| Class D | 0.05% | 0.03% | 0.69% | 106% | 104% |
| Class F | 0.01% | 0.15% | 0.26% | 103% | 103% |

Table 3-2 Test result in low QPs

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.04% | 102% | 101% |
| Class A2 | 0.00% | -0.08% | 0.01% | 99% | 100% |
| Class B | 0.00% | -0.01% | -0.03% | 102% | 101% |
| Class C | -0.01% | -0.02% | 0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.03% | 101% | 100% |
| Class D | 0.00% | 0.03% | 0.08% | 105% | 109% |
| Class F | -0.01% | 0.02% | 0.02% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.01% | -0.09% | 100% | 101% |
| Class C | -0.02% | 0.07% | 0.09% | 100% | 98% |
| Class E | 0.04% | 0.05% | -0.09% | 100% | 103% |
| Overall | 0.00% | 0.04% | -0.03% | 100% | 101% |
| Class D | 0.01% | 0.00% | 0.10% | 103% | 101% |
| Class F | 0.03% | -0.07% | 0.17% | 100% | 99% |

# Conclusion

The proposed modification simplified the insertion process of HMVP in AMVP candidate list construction, in both implementation and text without loss in BD-rate performance. It’s suggested to adopt the proposed method in the next version of VTM.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).