| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0257 |
| --- | --- |

| Title: | CE4-related: Simplified local illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang, Hongbin Liu, Li Zhang, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | zhangna.cynthia@bytedance.com lizhang.idm@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, four simplified local illumination compensation methods on top of CE4-3 are proposed. In the proposed methods, the template used to derive LIC parameters only contains four or eight samples. Eight and four samples in a template on top of CE4-3.1a are used in method 1 and 2, respectively. Eight and four samples in a template on top of CE4-3.2a are used in method 3 and 4, respectively. Simulation results are reported as below.

- Method 1: RA: -0.46%/116% /99%, 	LDB: -0.35%/119%/102%.

- Method 2: RA: -0.42%/117%/100%, 	LDB: -0.30%/119%/102%.

- Method 3: RA: -0.37%/118%/98%,	LDB: -0.23%/122%/100%.

- Method 4: RA: -0.34%/118%/98%,	LDB: -0.27%/121%/101%.

# Introduction

In CE4-3.1a, a unidirectional LIC method with 16x16 processing units extended to the affine mode is proposed in JVET-O0066 [1]. To derive LIC linear model parameters, the linear least square method is utilized, which requires the following operations per CU (when the chroma format is YUV4:2:0):

•	Multiplications: 2 * N + 4

•	Additions: 4 *N + 4

•	Shifts: 12

Ns for different luma and chroma block sizes are shown in Table 1 and Table 2, respectively.

Table 1: Ns for different luma block sizes

| cuWidth | 8 | 16 | 4 | 16 | 8 | ≥16 |
| --- | --- | --- | --- | --- | --- | --- |
| cuHeight | 8 | 4 | 16 | 8 | 16 | ≥16 |
| N | 16 | 8 | 8 | 16 | 16 | 16 |

Table 2: Ns for different chroma block sizes

| cuWidthC | 4 | 8 | 2 | 8 | 4 | ≥8 |
| --- | --- | --- | --- | --- | --- | --- |
| cuHeightC | 4 | 2 | 8 | 4 | 8 | ≥8 |
| N | 8 | 4 | 4 | 8 | 8 | 16 |

In CE4-3.2a, it is proposed to replace the linear least square method with the min/max method to estimate linear model parameters of LIC in JVET-O0086 [2].

Prior to applying the min/max method to estimate linear model parameters of LIC, the template samples of reference block and template samples of predicted CU may be processed by an FIR filter. This filter applies only when a CU is coded with the affine mode. Filter cores are specified subject to the size of a block. If a block has more than 32 samples, filter coefficients are [1 0 2 0 1] / 4. If a block has 32 and less samples, filter coefficients are defined as [1 2 1] / 4.

Sampling of the FIR filtering is adapted not to use intra, CIIP or IBC coded template samples. For example, the side position of the FIR coefficient “1” is accordingly changed to be closer to the central coefficient “2” when the side position corresponds to an intra, CIIP or IBC coded template sample.

Table 3 summarizes the complexity of CE4-3.1.a and CE4-3.2.a

Table 3: Complexity of CE4-3.1.a and CE4-3.2.a

|  | Multiplications | Additions | Shifts | Comparisons |
| --- | --- | --- | --- | --- |
| CE4-3.1.a | 36 | 68 | 12 | 0 |
| CE4-3.2.a | 4 | 183 | 121 | 288 |

# Proposed methods

This section describes the proposed simplified LIC methods. The first two sub-sections present the simplified solution on top of CE4-3.1a with eight and four samples in the template. The last two sub-sections present the simplified solution on top of CE4-3.2a with eight and four samples in the template.

## 2.1 Method #1 and Method #2 on top of CE4-3.1a

Eight or four neighboring samples of the current CU and its reference block are utilized to derive the LIC parameters in method #1 and method #2, respectively. Figure 1 and Figure 2 show examples of the sample selection in the two methods, respectively.

Table 4 and Table 5 summarize the complexity comparison between CE4-3.1a and the proposed methods.

Table 4: Complexity comparison between CE4-3.1a and the proposed method #1

|  | Multiplications | Additions | Shifts |
| --- | --- | --- | --- |
| Method #1 | 20 | 36 | 12 |
| Method #1/CE4-3.1.a | 56% | 53% | 100% |

Table 5: Complexity comparison between CE4-3.1a and the proposed method #2

|  | Multiplications | Additions | Shifts |
| --- | --- | --- | --- |
| Method #2 | 12 | 20 | 12 |
| Method #2/CE4-3.1.a | 33% | 29% | 100% |

(a)						(b)

Figure 1. Eight neighboring sample selection if both the left neighboring column and the above neighboring row are available (a) or only the above neighboring row is available (b) for a 16x16 luma block.

(a)						(b)

Figure 2. Four neighboring sample selection if both the left neighboring column and the above neighboring row are available (a) or only the above neighboring row is available (b) for a 16x16 luma block

When current block is a non-square block, the 8 or 4 samples are corresponding to the downsampled locations.

## 2.2 Method #3 and Method #4 on top of CE4-3.2a

The same eight or four samples are selected in method #3 or #4, as in method #1 or #2.

In both Method #3 and Method #4, if a template sample of current CU is intra, CIIP or IBC coded, it was substituted with the value of a corresponding template sample of the reference block.

For a CU is coded with the affine mode, if a block has more than 32 samples, the FIR filter coefficients are [1 0 2 0 1] / 4. If a block has 32 and less samples, the FIR filter coefficients are defined as [1 2 1] / 4. Sampling of the FIR filter adaptation process is skipped.

Table 6 and Table 7 summarize the complexity comparison between CE4-3.2a and the proposed methods.

Table 6: Complexity comparison between CE4-3.2a and the proposed method #3

|  | Multiplications | Additions | Shifts | Comparisons |
| --- | --- | --- | --- | --- |
| Method #3 | 2 | 55 | 41 | 32 |
| Method #3/CE4-3.2a | 50% | 30% | 34% | 11% |

Table 7: Complexity comparison between CE4-3.2a and the proposed method #4

|  | Multiplications | Additions | Shifts | Comparisons |
| --- | --- | --- | --- | --- |
| Method #4 | 2 | 31 | 25 | 16 |
| Method #4/CE4-3.2a | 50% | 17% | 21% | 6% |

β is derived using the averaged values which are calculated inside the sets of the selected neighboring samples of current CU and the reference block.

When current block is a non-square block, the 8 or 4 samples are corresponding to the original locations.

# Experimental Results

The proposed methods have been implemented on top of the CE4-3.1a and CE4-3.2a softwares. Results under RA/LDB configurations are summarized in Table 8 ~ Table 11. The simulations were performed following the the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 8: Coding performance of the proposed method 8.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.66% | -0.60% | -0.63% | 120% | 97% |  |  |  |  |  |
| Class A2 | -0.41% | -0.30% | -0.41% | 112% | 100% |  |  |  |  |  |
| Class B | -0.46% | -0.27% | -0.46% | 115% | 99% | -0.45% | 0.01% | -0.12% | 124% | 103% |
| Class C | -0.34% | -0.42% | -0.46% | 117% | 100% | -0.44% | -0.48% | -0.65% | 126% | 102% |
| Class E |  |  |  |  |  | -0.06% | 0.19% | -0.52% | 104% | 100% |
| Overall | -0.46% | -0.38% | -0.48% | 116% | 99% | -0.35% | -0.11% | -0.40% | 119% | 102% |
| Class D | -0.12% | -0.06% | 0.08% | 113% | 99% | -0.12% | -0.20% | 0.22% | 117% | 101% |
| Class F | -0.64% | -0.35% | -0.39% | 114% | 107% | -1.88% | -1.59% | -2.39% | 119% | 99% |
| Class SCC | -0.82% | -0.78% | -0.87% | 109% | 102% | -1.73% | -1.71% | -1.75% | 111% | 100% |

Table 9: Coding performance of the proposed method 9.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.62% | -0.48% | -0.62% | 123% | 99% |  |  |  |  |  |
| Class A2 | -0.35% | -0.33% | -0.43% | 114% | 102% |  |  |  |  |  |
| Class B | -0.42% | -0.19% | -0.32% | 115% | 99% | -0.42% | 0.13% | 0.02% | 124% | 103% |
| Class C | -0.30% | -0.42% | -0.34% | 118% | 99% | -0.33% | -0.30% | -0.21% | 125% | 103% |
| Class E |  |  |  |  |  | -0.07% | 0.08% | -0.25% | 103% | 99% |
| Overall | -0.42% | -0.34% | -0.41% | 117% | 100% | -0.30% | -0.03% | -0.12% | 119% | 102% |
| Class D | -0.09% | -0.20% | 0.08% | 113% | 99% | -0.10% | -0.33% | 0.60% | 117% | 101% |
| Class F | -0.56% | -0.33% | -0.43% | 114% | 109% | -1.69% | -1.53% | -1.58% | 119% | 99% |
| Class SCC | -0.83% | -0.81% | -0.86% | 109% | 101% | -1.58% | -1.63% | -1.58% | 111% | 100% |

Table 10: Coding performance of the proposed method 10.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.54% | -0.46% | -0.42% | 123% | 98% |  |  |  |  |  |
| Class A2 | -0.30% | -0.38% | -0.44% | 113% | 100% |  |  |  |  |  |
| Class B | -0.38% | -0.19% | -0.37% | 116% | 97% | -0.36% | -0.13% | -0.24% | 127% | 103% |
| Class C | -0.27% | -0.37% | -0.45% | 119% | 98% | -0.30% | -0.37% | -0.61% | 131% | 99% |
| Class E |  |  |  |  |  | 0.07% | -0.22% | -0.03% | 105% | 98% |
| Overall | -0.37% | -0.33% | -0.42% | 118% | 98% | -0.23% | -0.23% | -0.31% | 122% | 100% |
| Class D | -0.10% | 0.03% | 0.16% | 115% | 98% | -0.10% | -0.45% | -0.01% | 119% | 99% |
| Class F | -0.44% | -0.27% | -0.36% | 112% | 96% | -1.43% | -1.52% | -1.56% | 117% | 90% |
| Class SCC | -0.51% | -0.60% | -0.60% | 105% | 94% | -0.88% | -0.79% | -0.84% | 110% | 96% |

Table 11: Coding performance of the proposed method 11.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.49% | -0.48% | 123% | 98% |  |  |  |  |  |
| Class A2 | -0.27% | -0.20% | -0.36% | 115% | 102% |  |  |  |  |  |
| Class B | -0.36% | -0.24% | -0.39% | 116% | 97% | -0.34% | -0.02% | -0.07% | 127% | 105% |
| Class C | -0.26% | -0.23% | -0.38% | 118% | 97% | -0.30% | -0.34% | -0.55% | 128% | 100% |
| Class E |  |  |  |  |  | -0.10% | 0.28% | -0.34% | 104% | 96% |
| Overall | -0.34% | -0.28% | -0.40% | 118% | 98% | -0.27% | -0.06% | -0.29% | 121% | 101% |
| Class D | -0.07% | -0.02% | 0.10% | 115% | 97% | -0.06% | -0.14% | 0.39% | 119% | 99% |
| Class F | -0.42% | -0.24% | -0.35% | 111% | 95% | -1.35% | -1.68% | -1.68% | 118% | 91% |
| Class SCC | -0.49% | -0.52% | -0.53% | 106% | 94% | -0.87% | -0.80% | -0.80% | 108% | 96% |

- Conclusions

In this contribution, four simplified local illumination compensation methods are proposed. It is recommended to adopt one of these simplification methods into VVC.

# References

[1]	V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, H. Chen, X. Ma, H. Yang, “CE4-3.1a and CE4-3.1b: Unidirectional local illumination compensation with affine prediction,” JVET-O0066, Jul. 2019.

[2]	A. Filippov, V. Rufitskiy, E. Alshina, “CE4-3.2a and CE4-3.2b: Unidirectional local illumination compensation with affine prediction and simplified linear model parameter derivation based on min / max method,” JVET-O0086, Jul. 2019.

[3]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).