| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0266 |
| --- | --- |

| Title: | Non-CE6: Simplifications on LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng, Hongbin Liu, Na Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Three modifications on LFNST are proposed. First, it is proposed to disable LFNST when the width or height of a CU is larger than the maximum transform size. Second, it is proposed that the number is counted only for significant coefficients in the LFNST applied 4×4 blocks, and the threshold of the number of significant coefficients to apply LFNST is unified to be 1 for both dual-tree and single-tree coding structure. Third, it is proposed to remove the LFNST for the 2nd 4×4 block for a CU with sizes N×4 or 4×N, N > 8. Therefore, LFNST is only applicable to the top-left 4×4 block for all CUs. The average BD-rate changes are reported as below:

Modification #1: 			AI: 0.00%,	100%, 99%;	RA: 0.00%, 99%, 100%.

Modification #2: 			AI: 0.02%,	100%, 99%;	RA: 0.00%, 100%, 101%.

Modification #3: 			AI: 0.02%,	100%, 100%;	RA: 0.01%, 100%, 100%.

Modification #1+#2+#3:	AI: 0.03%,	100%, 99%;	RA: -0.01%, 100%, 100%.

# Introduction

There are three problems in the current design of LENST.

First, a CU will be split into multiple TUs if the width or height of the CU is larger than the maximum transform size. The decoder need to count the number of significant coefficients in all the TUs to determine whether to parse the LENST information. Therefore, the first TU cannot be processed until the last TU has parsed.

Second, the decoder needs to count significant coefficients in the whole block to determine whether LENST is applicable. When the number of significant coefficients is smaller than a threshold, LENST is not applicable. The threshold is different for dual-tree and single-tree coding structure. Since LENST only affects coefficients in the one or two top-left 4×4 blocks, it is unnecessary to count all the significant coefficients in the whole block.

Third, the top-left two 4×4 blocks need to perform the second transform when the CU dimensions are N×4 or 4×N, N > 8, which increases the decoding complexity.

# Proposed methods

Three modifications are proposed to address the three problems.

Modification #1:

LFNST is disabled without signaling when the width or height of a CU is larger than the maximum transform size. Under the current CTC, LFNST is always disabled for a 128×N or N×128 CU.

Modification #2:

The number is counted only for significant coefficients in the top-left 4×4 block, and the threshold of the number of significant coefficients to apply LFNST is unified to be 1 for both dual-tree and single-tree coding structure.

Modification #3:

The LFNST for the 2nd 4×4 block is removed for a CU with sizes N×4 or 4×N, N > 8. So LENST can only be performed on one top-left 4×4 block for all CU sizes.

# Experimental Results

The proposed methods have been implemented on top of VTM-5.0. Four tests are conducted. Test #1~Test #3 correspond to Modification #1 ~ Modification #3, respectively. Test #4 is a combined test of all the three modifications.

Results under AI/RA configurations in Test #1 ~ Test #4 are summarized in Table 1 ~ Table 4, respectively. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results of Test #1

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 95% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 96% | 0.00% | 0.00% | 0.00% | 99% | 115% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 98% | 0.00% | 0.00% | 0.00% | 99% | 102% |

Table 2: Results of Test #2

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.19% | -0.22% | 98% | 100% | -0.01% | 0.03% | -0.02% | 100% | 101% |
| Class A2 | 0.02% | -0.18% | -0.16% | 102% | 99% | 0.03% | -0.12% | -0.12% | 99% | 100% |
| Class B | 0.02% | -0.28% | -0.15% | 101% | 98% | 0.02% | -0.18% | -0.18% | 100% | 100% |
| Class C | 0.02% | -0.17% | -0.06% | 101% | 100% | -0.03% | -0.14% | -0.06% | 100% | 102% |
| Class E | 0.00% | -0.16% | -0.10% | 99% | 97% |  |  |  |  |  |
| Overall | 0.02% | -0.20% | -0.14% | 100% | 99% | 0.00% | -0.12% | -0.10% | 100% | 101% |
| Class D | -0.01% | -0.01% | -0.13% | 100% | 95% | 0.00% | -0.13% | 0.09% | 100% | 103% |
| Class F | 0.01% | -0.09% | -0.08% | 100% | 100% | -0.01% | -0.02% | -0.03% | 101% | 101% |

Table 3: Results of Test #3

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.02% | 99% | 102% | -0.01% | -0.05% | 0.06% | 100% | 101% |
| Class A2 | 0.01% | -0.03% | 0.01% | 102% | 100% | 0.02% | 0.04% | 0.14% | 100% | 101% |
| Class B | 0.02% | -0.03% | 0.05% | 100% | 100% | 0.02% | -0.06% | -0.26% | 101% | 100% |
| Class C | 0.04% | 0.02% | 0.05% | 100% | 99% | 0.01% | -0.20% | -0.03% | 100% | 101% |
| Class E | 0.02% | 0.06% | -0.03% | 100% | 99% |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.03% | 100% | 100% | 0.01% | -0.08% | -0.06% | 100% | 100% |
| Class D | 0.03% | 0.01% | 0.00% | 100% | 97% | 0.02% | 0.13% | 0.22% | 101% | 107% |
| Class F | -0.01% | 0.03% | 0.13% | 100% | 99% | -0.02% | -0.10% | 0.04% | 100% | 101% |

Table 4: Results of Test #4

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.19% | -0.28% | 101% | 100% | -0.01% | -0.05% | 0.01% | 100% | 100% |
| Class A2 | 0.04% | -0.14% | -0.10% | 101% | 99% | -0.01% | -0.02% | 0.02% | 100% | 100% |
| Class B | 0.03% | -0.19% | -0.10% | 101% | 98% | 0.00% | 0.00% | -0.23% | 100% | 99% |
| Class C | 0.02% | -0.06% | -0.03% | 100% | 102% | -0.04% | -0.30% | -0.21% | 100% | 101% |
| Class E | 0.03% | -0.07% | -0.15% | 99% | 97% |  |  |  |  |  |
| Overall | 0.03% | -0.13% | -0.12% | 100% | 99% | -0.01% | -0.09% | -0.13% | 100% | 100% |
| Class D | 0.02% | 0.05% | -0.10% | 100% | 95% | -0.01% | 0.06% | 0.39% | 100% | 100% |
| Class F | 0.00% | -0.08% | 0.13% | 100% | 98% | -0.01% | 0.01% | 0.15% | 100% | 101% |

# Conclusions

Considering the LFNST can be simplified with almost no coding performance loss, it is suggested that the proposed three aspects be adopted into VVC.

# Suggested changes to working draft

The proposed changes on top of JVET-N1001_v9 are specified as below, with deleted text highlighted in blue and newly added text highlighted in yellow.

#### Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && Max( lfnstWidth, lfnstHeight ) <= MaxTbSizeY && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > 1 ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| … |  |

#### Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) { |  |
| if( n < 16 ) |  |
| numSigCoeff++ |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |
| numZeroOutSigCoeff++ |  |
| } |  |
| … |  |

### Transformation process for scaled transform coefficients

#### General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, numLfnstX, numLfnstY, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-968)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-969)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-970)

nLfnstSize = 1  <<  log2LfnstSize		(8-971)

numLfnstX = ( nTbW > 8  &&  nTbH  = = 4 )  ?  2  :  1	(8-972)

numLfnstY = ( nTbW = = 4  &&  nTbH  > 8 )  ?  2  :  1	(8-973)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-974)

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).