| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0267-v1 |
| --- | --- |

| Title: | AHG15: Cleanups for scaling matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {olena.chubach, jenny.lai, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution is related to scaling matrices support and signalling cleanup. It is proposed to remove signalling the flag for the default scaling matrices on top of VTM5.0 and VVC Draft 5, considering the fact that the default scaling matrices are assigned to flat and the same as the case of not signalling scaling matrices. Also, it is proposed to modify the associated scaling matrices for intra block copy mode (IBC) to inter mode, instead of intra mode. The two proposed cleanups do not change results under common test conditions (CTC).

# Introduction

In the JVET-N0847 [1] adopted at the previous JVET meeting in Geneva, there are three modes of scaling matrices supported: OFF, DEFAULT, and USER_DEFINED. It was suggested that the DEFAULT mode scaling matrices are all assigned to flat, with elements equal to 16 for all transform block (TB) sizes. This assignment is equivalent to having no DEFAULT scaling matrices. DEFAULT mode is the same as OFF mode, which is redundant.

In addition, in the current VVC [2], IBC and intra prediction modes share the same scaling matrices. However, since both IBC and inter prediction modes perform prediction using motion vectors, IBC prediction samples are considered to be analogous to inter prediction samples, not intra prediction samples.

# Proposed cleanups

This proposal contains two cleanups. None of them is affecting CTC [3] results.

## Cleanup 1

It is proposed to eliminate the DEFAULT mode from the VVC and support two modes for scaling matrices only: ON (which is equivalent to USER_DEFINED mode) and OFF.

## Cleanup 2

It is proposed that IBC and inter prediction modes share the same sets of scaling matrices. Thus, for the case of ON (which is equivalent to USER_DEFINED mode), the number of MatrixType and MatrixType_DC are as follows:

- MatrixType: 30 = 2 (2 for inter&IBC/intra) × 3 (Y/Cb/Cr components) × 5 (square TB size: from 4×4 to 64×64 for luma, from 2×2 to 32×32 for chroma)

- MatrixType_DC: 14 = 2 (2 for inter&IBC/intra × 1 for Y component) × 3 (TB size: 16×16, 32×32, 64×64) + 4 (2 for intra&IBC/inter × 2 for Cb/Cr components) × 2 (TB size: 16×16, 32×32)

# Conclusion

This document proposes two cleanups for signalling the scaling matrices in VVC which do not affect CTC results. It is proposed to:

- Support two modes for scaling matrices: ON and OFF

- Update IBC and inter coding modes to share the same sets of scaling matrices

# References

[1] O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), T. Toma, K. Abe (Panasonic), S.-C. Lim, J. Kang (ETRI) “CE7-related: Support of signalling default and user-defined scaling matrices,” Document of Joint Video Experts Team, JVET-N0847.

[2] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).