| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0270-v1 |
| --- | --- |

| Title: | AHG17: Signalling refreshed regions associated with a GRA picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a conditional signalling of the flag of refreshed_region_flag in the slice header to indicate whether the slice belongs to the refreshed region in the current picture and the inference of refreshed_region_flag value for a GRA picture.

# Introduction

The GRA picture related syntax and symantics [1] are as following with characteristics:

  - One flag gra_enabled_flag in the SPS that specifies whether GRA pictures may be present or not

  - One new VCL NAL unit type, GRA_NUT, that indicates a GRA picture

  - One syntax element, recovery_poc_cnt, in the slice header

  - A process to start a CVS with a GRA picture

Table 7-1 – NAL unit type codes and NAL unit type classes

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 1 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 2 3 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| … | … | … |  |
| 26 | CRA_NUT | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 27 | GRA_NUT | Coded slice of a gradual random access picture slice_layer_rbsp( ) | VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified | non-VCL |

      - 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| … |  |
| } |  |

gra_enabled_flag equal to 1 specifies that GRA pictures may be present in CVSs referring to the SPS. gra_enabled_flag equal to 0 specifies that GRA pictures are not present in CVSs referring to the SPS.

      - 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| slice_pic_order_cnt_lsb | u(v) |
| … |  |
| } |  |

recovery_poc_cnt specifies the recovery point of decoded pictures in output order. If there is a picture picA that follows the current GRA picture in decoding order in the CVS and that has PicOrderCntVal equal to the PicOrderCntVal of the current GRA picture plus the value of recovery_poc_cnt, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt is referred to as the recovery point picture. The recovery point picture shall not precede the current GRA picture in decoding order. The value of recovery_poc_cnt shall be in the range of −MaxPicOrderCntLsb / 2 to MaxPicOrderCntLsb / 2 − 1, inclusive.

The variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt	(7-40)

According to the current VVC draft [1], coded video sequence start (CVSS) picture is a coded picture that is an IRAP picture with NoIncorrectPicOutputFlag equal to 1 or a GRA picture with NoIncorrectPicOutputFlag equal to 1. Basically, a CVS may start with a GRA picture.

# Proposed method

This contribution proposes a conditional signalling of the flag of refreshed_region_flag in the slice header to indicate whether the slice belongs to the refreshed region in the current picture and the inference of refreshed_region_flag value for a GRA picture. The related syntax and symantics is highlighed as below.

Proposal 1: proposes to add the flag of refreshed_region_flag with the condition test on gra_enabled_flag.

      - 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| else if( gra_enabled_flag ) |  |
| refreshed_region_flag | u(1) |
| slice_pic_order_cnt_lsb | u(v) |
| … |  |
| } |  |

refreshed_region_flag equal to 1 indicates that the slice belongs to the refreshed region in the current picture. refreshed_region_flag equal to 0 indicates that the slice does not belong to the refreshed region in the current picture. For a GRA picture, refreshed_region_flag is inferred to be 1 for I slice, and 0 for both P slice and B slice. Otherwise, when refreshed_region_flag is not present, refreshed_region_flag is inferred to be equal to 0.

The refreshed region boundaries may be specified by tile boundaries, slice boundaries, sub-picture boundaries, or virtual boundaries in terms of the refreshed region shapes and sizes. When refreshed_region_flag equal to 1 for slices in the current picture, by setting loop_filter_across_bricks_enabled_flag, loop_filter_across_slices_enabled_flag, or pps_loop_filter_across_virtual_boundaries_disabled_flag in the picture parameter sets, the in-loop filtering operations across the refreshed region boundaries shall be disabled to ensure the correctly decoded recovery point picture.

Note 1: refreshed_region_flag may be present for a GRA picture and the following pictures before the recovery point picture. For other pictures, refreshed_region_flag is set equal to 0 or is not present which is inferred to be 1.

Note 2: if the slicedoes not belong to the refreshed region in the current picture, a service application or delivery may decide to/not transport the slice data to the client and a client application may decide to/not feed the decoding engine with the slice.

Proposal 2: proposes to add the flag of pps_gra_enabled_flag in the picture parameter set and the conditional signalling of the flag of refreshed_region_flag is based on pps_gra_enabled_flag.

7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| … |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |
| if( gra_enabled_flag ) |  |
| pps_gra_enabled_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

pps_gra_enabled_flag equal to 1 specifies that refreshed regions associated with a GRA picture may be present in pictures referring to the PPS. pps_gra_enabled_flag equal to 0 specifies that refreshed regions associated with a GRA picture are not present in pictures referring to the PPS.

The corresponding slice header syntax is as follows.

      - 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| else if( pps_gra_enabled_flag ) |  |
| refreshed_region_flag | u(1) |
| slice_pic_order_cnt_lsb | u(v) |
| … |  |
| } |  |

Proposal 3: proposes to move the GRA related test to the front of the slice header. An application may choose to skip the rest of syntax parsing and the slice data decoding if refreshed_region_flag is equal to 0.

The corresponding slice header syntax is as follows.

      - 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| else if( pps_gra_enabled_flag ) |  |
| refreshed_region_flag | u(1) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| else if( pps_gra_enabled_flag ) |  |
| refreshed_region_flag | u(1) |
| slice_pic_order_cnt_lsb | u(v) |
| … |  |
| } |  |

The contribution summary:

Proposal 1: proposes to add the flag of refreshed_region_flag with the condition test on gra_enabled_flag.

Proposal 2: proposes to add the flag of pps_gra_enabled_flag in the picture parameter set and the conditional signalling of the flag of refreshed_region_flag is based on pps_gra_enabled_flag.

Proposal 3: proposes to move the GRA related test to the front of the slice header. An application may choose to skip the rest of syntax parsing and the slice data decoding if refreshed_region_flag is equal to 0.

# References

- “Versatile Video Coding (Draft 5),” JVET-N1001-v7, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).