| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0271-v1 |
| --- | --- |

| Title: | CE2-related: Chroma scaling for CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, three methods are proposed to reduce the luma-chroma dependency of the chroma scaling. In method 1, for each chroma transform block (TB) of intra coding unit (CU), the top-left reconstructed boundary sample of the collocated luma TB is used as the reference luma sample. For each chroma TB of non-intra (inter or intra block copy) CU, the top-left prediction sample of the collocated luma TB is used as the reference luma sample. In method 2, for each chroma TB of intra CU, the top-left reference boundary sample (i.e., reconstructed boundary sample after smoothing) of the collocated luma TB is used as the reference luma sample. For each chroma TB of non-intra CU, the top-left prediction sample of the collocated luma TB is used as the reference luma sample.

As a straightforward method, a combined inter and intra prediction (CIIP) CU is regarded as non-intra when chroma scaling is applied, and the top-left final combined prediction sample of the collocated TB is used as the reference luma sample. However, the luma-chroma dependency of the straightforward method is undesirable. Two different schemes are proposed for CIIP. In scheme 1, CIIP is treated as inter prediction mode for chroma scaling, and the reference luma sample is the top-left inter prediction sample of the collocated luma TB. In scheme 2, CIIP is treated as intra prediction mode for chroma scaling, and the reference luma sample is the top-left reconstructed/reference boundary sample of the collocated luma TB.

BD-rate results of method 1.1 (method 1 with scheme 1) are reported as follows.
VTM5.0-RA: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr)}
VTM5.0-LB: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr)}

BD-rate results of method 1.2 (method 1 with scheme 2) are reported as follows.
VTM5.0-RA: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr)}
VTM5.0-LB: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr)}

BD-rate results of method 2.1 (method 2 with scheme 1) are reported as follows.
VTM5.0-RA: {BD-rates = 0.01% (Y), -0.05% (Cb), -0.03% (Cr)}
VTM5.0-LB: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr)}

BD-rate results of method 2.2 (method 2 with scheme 2) are reported as follows.
VTM5.0-RA: {BD-rates = 0.00% (Y), -0.03% (Cb), -0.07% (Cr)}
VTM5.0-LB: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr)}

In method 3, the reference luma sample is the top-left reconstructed boundary sample of the collocated luma TB regardless of the current CU mode, and the chroma scaling factor of the the first chroma TB in one 64x64-luma/32x32-chroma (64x64-L/32x32-C) unit is shared in the entire 64x64-L/32x32-C unit.

BD-rate results of method 3 are reported as follows.
VTM5.0-RA: {BD-rates = -0.01% (Y), 0.10% (Cb), 0.08% (Cr)}
VTM5.0-LB: {BD-rates = 0.00% (Y), 0.09% (Cb), -0.11% (Cr)}

In method 4, based on the method 3, chroma scaling is also applied to dual tree. In both dual tree and single tree, the reference luma sample is the top-left reconstructed boundary sample of the collocated luma TB regardless of the current CU mode, and the chroma scaling factor of the the first chroma TB in one 64x64-L/32x32-C unit is shared in the entire 64x64-L/32x32-C unit.

BD-rate results of method 4 are reported as follows.
VTM5.0-AI : {BD-rates = 0.03% (Y), -0.37% (Cb), -0.51% (Cr)}
VTM5.0-RA: {BD-rates = 0.02% (Y), -0.39% (Cb), -0.41% (Cr)}
VTM5.0-LB: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr)}

# Introduction

In CE2-1.2 [1], the reference luma sample used for chroma scaling is the top-left prediction sample of the collocated luma TB.

In CE2-1.3.a [2], if the current CU is intra, the reference luma sample used for chroma scaling is the top-left reconstructed sample of the collocated luma TB.

In CE2-1.4.a [3], if the current CU is intra, the reference luma sample used for chroma scaling is the top-left reference boundary sample (i.e., reconstructed boundary sample after smoothing) of the collocated luma TB.

# Proposed methods

To reduce the luma-chroma dependency, it is proposed to apply CE2-1.2 to non-intra (inter or intra block copy) CUs and apply CE2-1.3.a or CE2-1.4.a to intra CUs.

As for CIIP, the CU is straightforwardly regarded as non-intra when chroma scaling is applied, and the top-left final combined prediction sample of the collocated TB is used as the reference luma sample. However, the luma-chroma dependency of the straightforward method is undesirable. Two different schemes are proposed for CIIP. In scheme 1, CIIP is treated as inter prediction mode for chroma scaling. In scheme 2, CIIP is treated as intra prediction mode for chroma scaling.

To further reduce the additional latency between luma reconstruction and chroma reconstruction when applying chroma scaling, it is also proposed to use the top-left reconstructed sample of the collocated luma TB regardless of the current CU mode. In addition, the chroma scaling factor of the first chroma TB in one 64x64-L/32x32-C unit is shared in the entire 64x64-L/32x32-C unit because it is believed that the target size of virtual pipeline data unit (VPDU) [4][5] for hardware decoding is set to 64x64-L/32x32-C. More details are described as follows.

## Method 1 - Using top-left prediction sample for non-intra CU and the top-left reconstructed boundary sample for intra CU

In method 1, for non-intra CU and intra CU, the reference luma sample is the top-left prediction sample and the top-left reconstructed boundary sample of the collocated luma TB, respectively. For CIIP, two different schemes are applied as follows.

### Method 1.1 - Treat CIIP as inter prediction mode for chroma scaling

Method 1.1 is built on top of method 1. In addition, CIIP is treated as inter prediction mode for chroma scaling. For each chroma TB of CIIP CU, the reference luma sample is the top-left inter prediction sample, rather than the final combined prediction sample, of the collocated luma TB. Therefore, the chroma scaling factor can be derived right after obtaining the inter prediction sample.

### Method 1.2 -Treat CIIP as intra prediction mode for chroma scaling

Method 1.2 is built on top of method 1. In addition, CIIP is treated as intra prediction mode for chroma scaling. For each chroma TB of CIIP CU, the reference luma smaple is the top-left reconstructed boundary sample of the collocated luma TB.

## Method 2 - Using top-left prediction sample in non-intra CU and the top-left reference boundary sample in intra CU

In method 2, for non-intra CU and intra CU, the reference luma sample is the top-left prediction sample and the top-left reference boundary sample of the collocated luma TB, respectively. For CIIP, two different schemes are applied as follows.

### Method 2.1 - Treat CIIP as inter prediction mode for chroma scaling

Method 2.1 is built on top of method 2. In addition, for each chroma TB of CIIP CU, the reference luma sample is the top-left inter prediction sample of the collocated luma TB.

### Method 2.2 -Treat CIIP as intra prediction mode for chroma scaling

Method 2.2 is built on top of method 2. In addition, for each chroma TB of CIIP CU, the reference luma smaple is the top-left reference boundary sample of the collocated luma TB.

## Method 3 - Using top-left reconstructed boundary sample of VPDU

In method 3, regardless of the current CU mode, the reference luma sample is the top-left reconstructed boundary sample of the collocated luma TB. Moreover, the chroma scaling factor of the first chroma TB in one VPDU (i.e., 64x64-L/32x32-C unit) is shared in the entire VPDU. In method 3, there is no additional latency between luma reconstruction and chroma reconstruction when applying chroma scaling, since the top-left reconstructed boundary sample of the VPDU is already derived before processing the current VPDU.

## Method 4 - Using top-left reconstructed boundary sample of VPDU in both dual tree and single tree

In method 4, method 3 is applied with chroma scaling enabled for both dual tree and single tree. In both dual tree and single tree, regardless of the current CU mode, the reference luma sample is the top-left reconstructed boundary sample of the collocated luma TB. Moreover, the chroma scaling factor of the first chroma TB in one VPDU (i.e., 64x64-L/32x32-C unit) is shared in the entire VPDU.

# Experiment results

The proposed methods have been implemented on VTM5.0 [6]. The common test conditions [7] are used for evaluation. The results of method 1.1, method 1.2, method 2.1, method 2.2, method 3, and method 4 are summarized in Table 1, Table 2, Table 3, Table 4, Table 5, and Table 6, respectively.

Table 1.  Results of the method 1.1 (to be updated) (runtime data not accurate)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.12% | -0.01% | 145% | 129% |
| Class A2 | 0.84% | 0.66% | 1.09% | 44% | 82% |
| Class B | 0.01% | -0.01% | -0.10% | 142% | 124% |
| Class C | 0.01% | 0.00% | -0.05% | 142% | 128% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | 0.10% | 0.17% | 113% | 116% |
| Class D | -0.03% | 0.08% | 0.26% | 142% | 132% |
| Class F | -0.01% | 0.04% | -0.03% | 142% | 121% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.03% | 0.15% | -0.02% | 144% | 135% |
| Class E | 0.00% | 0.00% | 0.12% | 138% | 127% |
| Overall | 0.34% | 0.90% | 0.39% | 36% | 80% |
| Class D | -0.02% | -0.05% | 0.74% | 142% | 134% |
| Class F | 0.03% | 0.05% | -0.27% | 59% | 94% |

Table 2.  Results of the method 1.2 (to be updated) (runtime not accurate)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.02% | -0.18% | 142% | 125% |
| Class C | -0.01% | 0.01% | -0.12% | 142% | 127% |
| Class E |  |  |  |  |  |
| Overall | 0.30% | -0.25% | 0.03% | 71% | 97% |
| Class D | 0.01% | 0.09% | 0.14% | 141% | 131% |
| Class F | 0.00% | 0.08% | -0.01% | 142% | 124% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | -0.15% | -0.07% | 136% | 128% |
| Class E | 0.09% | 0.17% | -0.15% | 134% | 126% |
| Overall | 0.32% | 0.09% | 0.91% | 35% | 78% |
| Class D | -0.02% | -0.29% | 1.01% | 134% | 130% |
| Class F | -0.03% | 0.13% | -0.30% | 60% | 94% |

Table 3.  Results of the method 2.1 (to be updated)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.19% | 0.03% | 100% | 101% |
| Class A2 | 0.02% | -0.04% | -0.01% | 100% | 99% |
| Class B | 0.02% | 0.01% | -0.10% | 100% | 98% |
| Class C | -0.02% | -0.01% | 0.00% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.05% | -0.03% | 100% | 99% |
| Class D | -0.01% | -0.03% | 0.20% | 100% | 100% |
| Class F | 0.01% | 0.06% | 0.00% | 101% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.03% | -0.12% | 0.02% | 102% | 105% |
| Class E | 0.03% | -0.15% | -0.18% | 101% | 99% |
| Overall | 0.40% | 0.15% | 0.62% | 27% | 64% |
| Class D | -0.04% | -0.32% | 0.55% | 98% | 107% |
| Class F | -0.15% | 0.13% | 0.19% | 45% | 77% |

Table 4.  Results of the method 2.2 (to be updated)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.08% | 0.05% | 100% | 99% |
| Class A2 | 0.02% | -0.03% | -0.03% | 100% | 100% |
| Class B | 0.02% | 0.02% | -0.09% | 100% | 97% |
| Class C | -0.02% | -0.07% | -0.17% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.07% | 100% | 99% |
| Class D | 0.03% | -0.12% | 0.21% | 99% | 100% |
| Class F | 0.01% | 0.07% | 0.03% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.03% | 0.03% | -0.20% | 101% | 102% |
| Class E | -0.02% | 0.22% | -0.31% | 100% | 98% |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.56% | 0.50% | 98% | 102% |
| Class F | 0.03% | 0.11% | 0.25% | 45% | 76% |

Table 5.  Results of the method 3

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.12% | 0.20% | 100% | 99% |
| Class A2 | 0.02% | -0.02% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.04% | -0.02% | 100% | 98% |
| Class C | -0.03% | 0.24% | 0.18% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.10% | 0.08% | 100% | 99% |
| Class D | -0.02% | 0.45% | 0.57% | 99% | 100% |
| Class F | -0.01% | 0.05% | 0.03% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.01% | 0.08% | 100% | 100% |
| Class C | 0.01% | 0.23% | 0.05% | 100% | 101% |
| Class E | -0.02% | 0.07% | -0.64% | 100% | 100% |
| Overall | 0.00% | 0.09% | -0.11% | 100% | 101% |
| Class D | -0.04% | 0.30% | 1.02% | 99% | 102% |
| Class F | 0.04% | -0.01% | 0.02% | 98% | 97% |

Table 6.  Results of the method 4 (to be updated) (runtime not accurate)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.42% | 0.23% | 150% | 128% |
| Class A2 | -0.11% | 0.91% | 1.04% | 151% | 127% |
| Class B | 0.04% | -0.34% | -0.45% | 151% | 125% |
| Class C | 0.14% | -1.16% | -1.38% | 152% | 127% |
| Class E | 0.13% | -1.44% | -1.77% | 150% | 126% |
| Overall | 0.03% | -0.37% | -0.51% | 151% | 126% |
| Class D | 0.07% | -0.50% | -0.73% | 151% | 132% |
| Class F | 0.29% | -1.30% | -1.41% | 147% | 124% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.12% | 0.20% | 142% | 126% |
| Class A2 | 0.02% | -0.02% | 0.00% | 141% | 128% |
| Class B | 0.02% | -0.37% | -0.43% | 143% | 124% |
| Class C | 0.05% | -1.08% | -1.16% | 141% | 125% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.39% | -0.41% | 142% | 126% |
| Class D | 0.02% | -0.26% | -0.41% | 142% | 130% |
| Class F | 0.21% | -1.32% | -1.24% | 142% | 130% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.06% | 0.45% | -0.08% | 139% | 131% |
| Class E | 0.07% | -1.25% | -1.46% | 142% | 129% |
| Overall | 0.34% | 0.45% | -1.49% | 37% | 81% |
| Class D | -0.09% | 0.65% | 0.91% | 143% | 135% |
| Class F | 0.04% | -0.37% | -1.14% | 61% | 96% |

# Conclusions

In this contribution, six tests are proposed to reduce the luma-chroma dependency of chroma scaling. The coding efficiency impact is reported to be minor.

# References

- T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, and T. Chen, “CE2.1.2: Luma-chroma dependency reduction for LMCS,” Document of Joint Video Experts Team, JVET-O0098.

- E. Francois and T. Poirier, “CE2: Report of CE2 experiments from InterDigital,” Document of Joint Video Experts Team, JVET-O0087.

- J. Chen, R.-L. Liao, and Y. Ye, “CE2-1.4: Luma-chroma dependency reduction for chroma scaling,” Document of Joint Video Experts Team, JVET-O0102.

- C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1.2.1: Constraint for binary and ternary partitions,” Document of Joint Video Experts Team, JVET-L0081.

- C.-M. Tsai, Shih-Ta Hsiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1-related: Picture boundary CU split satisfying the VPDU constraint,” Document of Joint Video Experts Team, JVET-M0888.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).