| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0274-v3 |
| --- | --- |

| Title: | CE2-related: Luma-chroma dependency reduction for chroma separate tree by constraining CCLM usage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Cross-component linear model prediction (CCLM) needs to use reconstructed samples of luma coding block (CB) to generate prediction for chroma CB. When chroma separate tree (CST) is enabled, partitions of luma and chroma coding tree units (CTUs) can be different, which causes latency for chroma CB to wait for more luma samples to be reconstructed. In VTM5.0, the worse-case latency is 64x64 luma samples. In this proposal, two methods are proposed to reduce the latency to 32x32 luma samples. Method 1 imposes split constraints on both luma and chroma CB. Method 2 imposes constraints only on the enabling of CCLM mode to allow more split options and is implemented as two versions: encoder-side normative constraints and decoder normative changes. Besides, if intra sub-partitions (ISP) mode is used in the corresponding 64x64 luma CB, CCLM is disallowed in all proposed methods. The results are summarized as follows.

Method 1
AI: BD-rates = 0.18% (Y), 2.43% (Cb), 2.40% (Cr); EncT=93%; DecT=100%
RA: BD-rates = 0.04% (Y), 0.76% (Cb), 0.84% (Cr); EncT=100%; DecT=99%

Method 2, encoder-side normative constraints
AI: BD-rates = 0.10% (Y), 1.21% (Cb), 1.16% (Cr); EncT=98%; DecT=100%
RA: BD-rates = 0.04% (Y), 0.44% (Cb), 0.38% (Cr); EncT=99%; DecT=99%

Method 2, decoder normative changes
AI: BD-rates = 0.07% (Y), 1.03% (Cb), 1.03% (Cr); EncT=98%; DecT=100%
RA: BD-rates = 0.02% (Y), 0.34% (Cb), 0.34% (Cr); EncT=99%; DecT=100%

It is asserted that the proposed methods preserve most coding gain of CST.

# Introduction

In VTM5.0, if CST is enabled, the CB partitions of luma and chroma CTUs can be different, so a luma CB may cross multiple chroma CBs partially or vice versa. This introduces processing latency when generating CCLM prediction for chroma CBs by using luma reconstructed samples. For example, as shown in Figure 1, if the upper 32x16 chroma CB is coded with CCLM mode, to generate the CCLM prediction, it has to wait for the reconstruction of the two 32x64 luma CBs. The worse-case latency is 64x64 luma samples.

Figure 1. An example of luma and chroma CB partitions with potential latency issue.

# Proposed methods

To reduce the luma-chroma latency from 64x64 to 32x32 luma samples for CCLM prediction generation when CST is enabled, method 1 and method 2 are proposed. Besides, on top of the two proposed methods, if intra sub-partitions (ISP) mode is used for the corresponding 64x64 luma CB, then CCLM is disallowed in all proposed methods. In the rest of this contribution, MxN luma samples and PxQ chroma samples are denoted as MxN-L and PxQ-C, respectively.

## Method 1

As shown in Figure 2, split constraints are applied to luma and chrom CBs as follows.

- If luma CB size is 64x64-L, only no split and quadtree (QT) split are allowed.

- If chroma CB size is 32x32-C, only no split and QT split are allowed.

With these constraints, the worse-case latency is 32x32-L and occurs when the following two cases are satisfied: luma case: QT split is applied to a 64x64-L node, and binary tree (BT) or ternary tree (TT) split is applied to the n-th child node of the QT split; chroma case: QT split is applied to a 32x32-C node collocated with the 64x64-L node, and BT or TT split in the opposite direction of the luma BT or TT is applied to the n-th child node of the QT split.

Figure 2. Split constraints of the proposed method 1.

## Method 2

Method 2 imposes constraints only on the enabling of CCLM mode by the following four options.

Option 1:
If luma CB size is 64x64-L, CCLM is allowed for each chroma leaf node.

Option 2:
If a luma 64x64-L parent node is split into four 32x32-L nodes, and a chroma 32x32-C parent node is no further split or split into four 16x16-C nodes, then CCLM is allowed for each of the following leaf nodes.

Option 3:
If a luma 64x64-L parent node is split into four 32x32-L nodes, and a chroma 32x32-C parent node is split into two 32x16-C leaf nodes (i.e., CBs), then CCLM is allowed for each of 32x16-C leaf nodes, as shown in Figure 3.

Option 4:
If a luma 64x64-L parent node is split into four 32x32-L nodes, and a chroma 32x32-C parent node is split into two 32x16-C nodes. In each of the two 32x16-C nodes, if further split is applied to split a 32x16-C node into two 16x16-C child nodes, then CCLM is allowed for each of the following leaf nodes, as shown in Figure 4.

Figure 3. Allowed CCLM cases for option 3

Figure 4. Allowed CCLM cases for option 4

# Experimental results

The proposed methods are implemented on top of VTM5.0 [2], and the simulation is performed under common test conditions (CTC) as described in JVET-N1010 [3]. Table 1, Table 2, and Table 3 summarize the results of method 1, method 2 with encoder-side normative constraints, and method 2 with decoder normative changes. Note that if CST is disabled in VTM5.0, under AI condition the BD-rates are 0.41% (Y), 12.73% (Cb), 13.50% (Cr). The coding efficiency penalty of the proposed luma-chroma latency reduction methods is much lower than that of disabling CST.

Table 1.   Results of the proposed method 1

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.33% | 4.05% | 3.48% | 90% | 99% |
| Class A2 | 0.18% | 2.50% | 1.47% | 91% | 101% |
| Class B | 0.15% | 2.67% | 3.36% | 94% | 100% |
| Class C | 0.09% | 0.46% | 0.64% | 95% | 98% |
| Class E | 0.17% | 2.97% | 3.02% | 94% | 101% |
| Overall | 0.18% | 2.43% | 2.40% | 93% | 100% |
| Class D | 0.03% | 0.52% | 0.27% | 96% | 99% |
| Class F | 0.15% | 1.24% | 1.26% | 96% | 100% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.64% | 0.84% | 100% | 98% |
| Class A2 | 0.07% | 1.06% | 0.67% | 100% | 99% |
| Class B | 0.04% | 1.20% | 1.45% | 100% | 98% |
| Class C | -0.03% | 0.06% | 0.19% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.76% | 0.84% | 100% | 99% |
| Class D | 0.03% | 0.12% | 0.29% | 99% | 100% |
| Class F | 0.06% | 0.74% | 0.78% | 99% | 98% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.44% | 0.18% | 100% | 99% |
| Class C | 0.00% | 0.01% | 0.03% | 99% | 100% |
| Class E | 0.13% | 1.11% | 1.40% | 100% | 98% |
| Overall | 0.03% | 0.47% | 0.44% | 100% | 99% |
| Class D | 0.03% | 0.45% | 0.54% | 98% | 98% |
| Class F | 0.03% | 0.31% | 0.32% | 97% | 97% |

Table 2.   Results of the proposed method 2 with encoder-side normative constraints

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 2.34% | 2.26% | 96% | 100% |
| Class A2 | 0.11% | 1.61% | 0.67% | 97% | 100% |
| Class B | 0.08% | 1.18% | 1.54% | 98% | 102% |
| Class C | 0.04% | 0.32% | 0.38% | 98% | 98% |
| Class E | 0.06% | 0.94% | 0.95% | 98% | 99% |
| Overall | 0.10% | 1.21% | 1.16% | 98% | 100% |
| Class D | 0.02% | 0.41% | 0.18% | 98% | 98% |
| Class F | 0.09% | 0.59% | 0.73% | 99% | 100% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.26% | 0.54% | 100% | 100% |
| Class A2 | 0.06% | 0.87% | 0.20% | 99% | 99% |
| Class B | 0.05% | 0.55% | 0.63% | 99% | 100% |
| Class C | 0.02% | 0.12% | 0.10% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.44% | 0.38% | 99% | 99% |
| Class D | 0.04% | 0.44% | 0.00% | 98% | 100% |
| Class F | 0.06% | 0.37% | 0.37% | 99% | 100% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.40% | 0.35% | 99% | 99% |
| Class C | 0.01% | 0.12% | -0.34% | 99% | 99% |
| Class E | 0.04% | 0.58% | 1.36% | 99% | 98% |
| Overall | 0.01% | 0.35% | 0.37% | 99% | 99% |
| Class D | -0.03% | 0.13% | 0.12% | 96% | 99% |
| Class F | 0.10% | 0.25% | 0.08% | 97% | 97% |

Table 3.   Results of the proposed method 2 with decoder normative changes

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 2.04% | 2.19% | 97% | 99% |
| Class A2 | 0.08% | 1.40% | 0.55% | 98% | 101% |
| Class B | 0.06% | 1.01% | 1.37% | 99% | 100% |
| Class C | 0.03% | 0.26% | 0.27% | 99% | 100% |
| Class E | 0.02% | 0.69% | 0.81% | 99% | 99% |
| Overall | 0.07% | 1.03% | 1.03% | 98% | 100% |
| Class D | 0.01% | 0.34% | 0.13% | 99% | 98% |
| Class F | 0.07% | 0.59% | 0.51% | 100% | 104% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.33% | 0.58% | 99% | 98% |
| Class A2 | 0.03% | 0.56% | 0.28% | 99% | 101% |
| Class B | 0.01% | 0.46% | 0.55% | 99% | 99% |
| Class C | -0.01% | 0.02% | -0.06% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.34% | 0.34% | 99% | 100% |
| Class D | 0.02% | 0.08% | -0.06% | 98% | 100% |
| Class F | 0.03% | 0.26% | 0.33% | 100% | 99% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.58% | 0.18% | 99% | 99% |
| Class C | 0.01% | 0.39% | 0.06% | 100% | 99% |
| Class E | -0.06% | 0.44% | 0.43% | 99% | 98% |
| Overall | -0.01% | 0.48% | 0.20% | 99% | 99% |
| Class D | 0.04% | -0.21% | 0.44% | 97% | 99% |
| Class F | 0.09% | -0.20% | 0.21% | 97% | 98% |

# Conclusion

In this contribution, two methods are proposed to reduce the luma-chroma latency from 64x64 to 32x32 luma samples for CCLM prediction generation when CST is enabled. The results shows that the proposed methods can preserve most of the coding gain of CST.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 5)," Document of Joint Video Experts Team, JVET-N1001.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).